Source:	Qualcomm Inc.

Title:	Qualcomm Listening Lab Report - IVAS Characterization Phase

Document for:	Approval

Agenda item:	7.5

# Summary

The document presents a report on subjective testing performed as a part of the 3GPP-SA4 IVAS Codec Characterization Phase. Qualcomm performed two experiments according to the procedures and test plan specified in permanent documents IVAS-7b (Processing Plan for Characterization Phase) [2] and IVAS-8b (Test Plan for Characterization Phase) [3]. Minor modifications from to the listening test instructions were applied. No deviations from or exceptions to the listening test procedures and specifications described in the test plan have been observed or experienced.

Key Acronyms

CuT	Codec under Test

GAL	Global Analysis Laboratory

LL	Listening Laboratory

MC	Material Collection Entity

# 1. Introduction

Qualcomm performed (2) different experiments (BS1534-10 and BS1534-13) within the 3GPP SA4 IVAS Codec Characterization exercise. The experiment name for each experiment are listed in Table 1.

Table 1: Summary of tests

| Exp. | Name | Participants |
| --- | --- | --- |
| BS.1534-10 | ISM 1-2 | 14 |
| BS.1534-13 | MASA 1TC | 14 |

# 2. Test material for BS.1534

The test material for BS.1534 (MUSHRA) tests was selected and delivered by the MC.

# 3. Listening Environment for BS.1534

The tests were performed in acoustically treated critical listening rooms in three geographical locations part of Qualcomm Inc., located across three countries (USA, India, UK). Each locations used rooms which comformed to ITU-R [4] and IVAS-8b [3].

-having a background noise level of less than NR-25

-listening equipment conforming to the IVAS-8b recommendations (open-back headphones Sennheiser HD600, Sennheiser HD650, or Beyerdynamic DT770)

All used headphones have been verified before and after experiments were performed. A WebMUSHRA client was used to collect the subjective results.

# 4. Test instructions/order for BS.1534

The instructions for subjects tested by Qualcomm for the Characterization Testing are in Annex A. The instructions to subjects were available in writing during the entire test session. The instructions were derived from [4] and modified to provide context to the evaluation.

# 5. Test subjects for BS.1534

# All subjects for BS.1534 testing self-reported having normal-hearing and are expert in subjective audio quality assessment. Each experiment required 14 listeners, participants ages ranged from 24 to 49. The age and gender information for the set of subjects used in each listening test, and over all listening tests by Qualcomm, are provided in Annex B of this report.

# Assessors were excluded from the aggregated responses if he or she rated the hidden reference condition for > 15% of the test items lower than a score of 90 as required by [4]. No assessor ended up being excluded.

6. Raw data delivery

All raw voting data have been delivered to the GAL using the data delivery file provided by the GAL for all tests. This delivery happened in the required time frame.

7. Discussion of any problems encountered during testing and the solution used to address the problem

During the testing, no problems were encountered.

Annex A

Instructions to subjects in English as used by Qualcomm during the subjective listening tests performed during the 3GPP SA4 IVAS characterization phase

A.1. Test instructions in English used in experiments BS1534-10 and BS1534-13.

INSTRUCTIONS FOR LISTENING TESTS BS.1534-10 and BS.1534-13 In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio means that you can locate various sound sources around yourself. For example, a first talker may appear to talk from the left-hand side and a second talker from the right-hand side, a talker can be moving, etc. In each trial, you will hear a reference audio sample followed by various test conditions. The test conditions have the same content as the reference sample but are possibly impaired after being passed through a telecommunication system. Your task is to evaluate your subjective overall audio quality, taking account of potential degradations in the sound quality (e.g., due to additional noise, roughness, clicks or other distortions), and/or degradations in the spatial representation (e.g., sound source location, distance, spatial width, movement, etc.). The test is divided into a familiarization stage and a blind grading stage. The familiarization stage is designed to help you become familiar with the UI and the test conditions, by presenting preliminary items. You will go through four example trials, and each trial contains eight audio items to listen to. Please take your time to learn how the MuSHRA interface works and listen carefully to each condition. This will help you understand how different conditions affect sound quality. During this training phase, the conditions are not randomized and are not blind. No grades given during the training phase will be taken into account in the true tests. After the familiarization trials, the blind grading phase will begin. Please listen to each element before rating and apply your score from a range of 0-100. You may listen to the signals in any order, any number of times. You will use a slider quality scale when assigning your grades. The grading scale is continuous from “excellent” to “bad”. A grade of 0 corresponds to the bottom of the “bad” category, while a grade of 100 corresponds to the top of the “excellent” category. In evaluating the sound excerpts, please note that you should not necessarily give a grade in the “bad” category to the sound excerpt with the lowest quality in the test. However, one or more excerpts must be given a grade of 100 because the unprocessed reference signal is included as one of the excerpts to be graded.

Annex B

Age and gender information for the set of subjects used in each listening test, tested by Qualcomm Inc.

Table B.1 Age and gender information for the set of subjects used in each BS.1534 listening test

| Exp. | #female listeners | #male listeners | Mean Age (years) | Age StdDev (years) |
| --- | --- | --- | --- | --- |
| BS.1534-10 | 1 | 13 | 35.50 | 8.58 |
| BS.1534-13 | 1 | 13 | 36.07 | 8.18 |

Reference Documents

[1]	"Practical procedures for subjective testing," ITU-T Handbook, 2011.

[2]	IVAS-7b (Processing Plan for Characterization Phase)

[3]	IVAS-8b (Test Plan for Characterization Phase)

[4] 	ITU-R BS.1534, “Method for the subjective assessment of intermediate quality level

of audio systems, 2015