Source:	VoiceAge Corporation

Title:	Listening Lab Report - IVAS Characterization Phase

Agenda Item:	7.5

Document for:	Discussion and Agreement

Summary

This report presents the results of subjective testing conducted by VoiceAge Corporation as part of the 3GPP-SA4 IVAS Codec Characterization Phase. The experiments followed the procedures outlined in:

- IVAS-7b: Processing Plan for Characterization Phase

- IVAS-8b: Test Plan for Characterization Phase

No deviations from the specified procedures were observed or experienced.

Key Acronyms

- DCR: Degradation Category Rating

- GAL: Global Analysis Laboratory

- HL: Host Laboratory

- LL: Listening Laboratory

- MC: Material Collection Entity

Introduction

VoiceAge Corporation performed two experiments during the IVAS Codec Characterization Phase. The name and language for each experiment are listed below.

Table 1: Allocation of methodologies and languages

| Exp. | Content | Language |
| --- | --- | --- |
| P800-1 (p01) | Stereo content including speech, music, and mixed media | French |
| P800-10 (p10) | Multi-object audio scenes (3–4 objects) with speech, music, and effects | French |

2. Audio material

VoiceAge Corporation provided original speech, music and mixed content (e.g. advertisement) sampled at 48-kHz with 16-bits resolution to the MC. The MC and the HLs processed the audio samples and delivered them to the VoiceAge LL in time for testing.

2.1. Input audio files

Filenames followed the format: meeeayszz.wav, where:

- m: Listening lab designator (VoiceAge Corporation)

- eee: Experiment designator (e.g., p01)

- a: Audio indicator

- y: Category per experiment

- s: Sample indicator

- zz: Sample number (01–07; 07 is preliminary)

3. Listening Environment

Tests were conducted in acoustically treated rooms at Sherbrooke University, with background noise levels below NR-25. Equipment used:

- Beyerdynamic DT770 Pro headphones

- Surface tablet with in-house software

- Focusrite Scarlett amplifier

- Remote server for synchronization and vote collection

Setup complies with IVAS-8b specifications.

4. Instructions

The instructions for subjects tested by VoiceAge Corporation for the Characterization testing are available in Annex A of this document. The instructions were first presented and discussed verbally and were also available in written form throughout the test. Sample presentation order was provided by the HL on Sept. 16th, using IVAS_char_rands.xlsx.

5. Test Subjects

Only native French speakers participated to the tests. Each experiment required 30 listeners (6 panels of 5 listeners each). Post-screenings confirmed that retained listeners could discriminate across levels of both MNRU and ESDRU., A total of 67 listeners were screened to retain 30 listeners per test. Seven listeners were rejected.

Table 2: Listener Statistics

|  | Male | Female | Total |
| --- | --- | --- | --- |
| Count | 38 | 29 | 67 |
| Avg. Age | 25.8 | 24.0 | 25.0 |
| Std. Dev. | 4.0 | 1.9 | 3.1 |

6. Raw data delivery

Raw voting data were delivered to the GAL on Oct. 29th using the framework provided by the GAL for both P800-1 and P800-10 tests.

7. Discussions

No problems have been encountered during the P.800-1 and P800-10 DCR tests.

References

S4-251612 - IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v1.1.1

S4-251561 - IVAS-7b: Processing plan for characterization phase, v1.1.0

- ANNEX A

INSTRUCTIONS AUX AUDITERUS NAÏFS POUR UN TEST DCR IMMERSIF Durant cette expérience, vous allez évaluer des systèmes qui pourraient être utilisés dans des services de télécommunications futurs utilisant l’audio immersif. L’audio immersif signifie que vous pouvez localiser différentes sources de son autour de vous. Par exemple, un premier locuteur peut parler à votre gauche et un deuxième locuteur à votre droite, un locuteur peut se déplacer, etc. Dans chaque essai, vous entendrez un échantillon d’un signal audio de référence, suivi par un échantillon de test. L’échantillon de test a le même contenu que l’échantillon de référence, mais il a été possiblement altéré après son passage par le système de télécommunications. Votre tâche est d’évaluer le niveau de l’altération du deuxième échantillon par rapport au premier échantillon. L’altération peut être causée aussi bien par un changement de la qualité du son (par exemple à cause d’un bruit additionnel, une rugosité, des clics ou autres distorsions), que par une modification de la représentation spatiale (par exemple changement dans la localisation de la source du son, de sa distance, de son étendue, de son déplacement, etc.) Écoutez attentivement les deux échantillons dans un essai. Veuillez ensuite enregistrer votre opinion globale sur l’altération que vous percevez dans le deuxième échantillon par rapport au premier échantillon, en utilisant l’échelle suivante : 5 - Aucune altération audible 4 - Faible altération parfois audible 3 - Altération audible 2 - Altération clairement audible 1 - Forte altération clairement audible Notez que le niveau d’altération présent dans différents échantillons de test est supposé couvrir toute l’échelle de vote durant l’expérience. Veuillez ne pas discuter vos opinions avec les autres auditeurs participants à l’expérience. Si vous avez des questions, veuillez-vous adresser à l’administrateur du test.