3GPP TSG- SA4 Meeting # 134	S4-251671

17-21 November 2025, Dallas, Texas, USA

3GPP SA WG2 Meeting #170	S2-2507657

Gothenburg, Sweden, 25-29 August 2025

# Title:	Reply LS on LI requirements on IMS Data Channel

# Response to:	LS on LI requirements on IMS Data Channel (S3i250185/ S2-2506116)

# Release:	Release 19

# Work Item:	NG_RTC_Ph2

Source:	SA2

To:	SA3-LI, SA3

Cc:	CT1, SA4

Contact Person:

Name:	Kefeng Zhang

Tel. Number:

E-mail Address:	kefezhan AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks SA3-LI for the LS regarding LI requirements on IMS Data Channel.

SA2 would like to recommend SA4 and SA3-LI to find solutions to meet the LI requirement on IMS Data Channel in the following scenarios:

- Roaming cases with S8HR / N9HR model: SA2 would like to ask SA3 to study how the VPLMN decrypts the Data Channel content when any UE is in roaming state.

- Interoperability case between two CSPs, or in direct communications between two users P2P: When the MF acts as "HTTP Proxy", the P2A, P2A2P Data Channels are always terminated in the MF which can provide a copy of the IMS Data Channel content after decryption for LI. To support the LI requirements for P2P data channel, the serving IMS network can be configured to anchor the P2P Data Channel of the target UE or all UEs in the MF which acts as "DC Application Proxy". The MF can provide a copy of the IMS Data Channel content after decryption.

When the MF acts as a “UDP Proxy”, LI requirements cannot be fulfilled.

- Interoperability between two CSPs, in which one is using IMS Data Channel and the other is using IMS without any IMS Data Channel feature: If the target UE is using the IMS without any Data Channel, and interworking between the DCMTSI UE and MTSI UE (i.e., the LI target) for specific DC application is required (as specified in clause AC.7.9 of TS 23.228), the IMS data channel content from the DCMTSI UE is terminated in the MF or DC AS. The MF or DC AS may support the interworking with the MTSI UE via IMS video flow or other existing mechanism (e.g. SMS, HTTP via Internet DN. It is assumed that LI for such interworking scenarios (IMS video, SMS, Internet) with the MTSI UE is supported by existing specifications. SA2 has not identified any gaps in supporting LI for these scenarios.

2. Actions:

To SA3-LI:

ACTION: 	SA3-LI is kindly requested to take the above information into account.

To SA4 and SA3:

ACTION: 	SA4 and SA3 are kindly requested to take the above information into account and provide any feedback as deem necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, TX, USA