3GPP TSG-SA WG4 Meeting #134	S4-251686

Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Real-Time 3D Face-Mesh Communication using V‑DMC

Agenda item:	6

Document for:	Information

## 1. Introduction

The V‑DMC standard [1] enables efficient compression of dynamic 3D meshes, which is essential for immersive and interactive 3D use cases. Psychovisual perception of rendered human faces can be particularly sensitive to visual artifacts and latency; realistic, dynamic face reproduction is critical for immersive presence in XR, telepresence, and education.

In SA4#131, Nokia demonstrated a prototype of a V-DMC compatible player for the web platform, as described in [2]. In the present contribution, we demonstrate a web‑native reference prototype that validates the utilization of the V‑DMC standard for real‑time 3D face communication, requiring no native applications or plugins and leveraging widely available web APIs and hardware video codecs.

The solution is standards‑based and end‑to‑end: it uses MPEG V‑DMC for representation; WebGL2, WebCodecs, WebAssembly, WebRTC and WebXR for run-time execution. No modifications or add-ons are required on the browser side, simplifying deployment. This approach complements existing 2D video systems by introducing spatial presence while maintaining bandwidth efficiency through V-DMC’s video-based coding.

## 2. System overview

The system enables two browsers to exchange live, V-DMC‑compliant 3D face-mesh streams and to render them interactively in 3D on conventional 2D screens or AR. It operates using the MPEG V‑DMC framework to code and decode meshes generated by this application for the representation of human faces: base mesh coding (using the MPEG EdgeBreaker (MEB) mesh codec), subdivisions, (support for optional) displacements, texture, and atlas metadata.

A web application captures the user’s face and estimates a real‑time 3D mesh. The sender captures facial landmarks from a camera (~468 landmarks) to generate a stabilized base‑mesh.  Successive subdivision steps increase mesh density and result in a smoother geometry. Displacements are added to each subdivided vertex only when a depth map is available.

The V-DMC encoder produces the MEB-encoded base mesh and atlas metadata, while the texture is prepared for hardware video coding. For base-mesh all 468 vertices estimated by MediaPipe are encoded. Face texture is packetized as rectangular atlases and coded by a hardware video codec (e.g., HEVC); atlas metadata describes patch layout and tool usage.

To achieve low latency, streaming takes place over WebRTC using a data channel for V-DMC metadata and base meshes, while hardware-encoded texture and displacement bitstreams are carried in regular media tracks. Texture resolution is 256x256 pixels, displacement stream resolution is 128x128 pixels.

On the receiver side, the base mesh and atlas information are parsed in the CPU, and the texture video (and optionally the displacement video) is/are decoded using a hardware-accelerated decoder in the GPU. Rendering is handled via a WebGL2 pipeline, which performs mesh subdivision for face reconstruction and rendering, optional displacement mapping, and shading. When WebXR is available, an AR visualization mode can be used to place and animate the received face in the physical environment.

Figure 1: High-level processing chain of the V-DMC encoder prototype. Depth map is currently not implemented in our demo.

## 3. Hardware and performance

Using modern mobile phone hardware, the system achieves an interactive frame rate (30 FPS). Hardware-accelerated decoding enables parallel video stream processing, while MEB processing, atlas handling, and subdivision control run on the CPU.

- Base-mesh and Atlas bitstream are processed (encoded/decoded) in CPU using V-DMC TMM v11.0 [3], ported for the Web (JS/WASM).

- Texture and optional displacements are processed utilizing the HW GPU encoder.

Subdivisions and final rendering are done utilizing WebGL2 shaders (using A-Frame [4]).

The total bitrate is 400 kbps: 100 kbps for textures and 300 kbps for basemesh and displacement (with displacement being negligible due to mesh stability). The basemesh uses intra-only encoding.

Demo devices specifications:

| Mobile device |  |
| --- | --- |
| OS | Android™ 15 |
| CPU | Qualcomm SM8750-AC Snapdragon 8 Elite (3 nm) , Octa-core (2x4.47 GHz Oryon V2 Phoenix L + 6x3.53 GHz Oryon V2 Phoenix M) |
| GPU | Adreno 830 (1200 MHz) |
| RAM | 8GB/16GB LPDDR5X |

## 4. Demonstration

In the demonstration, the system runs locally to avoid any networking issues, i.e., the encoded V-DMC bitstream is directly fed to the decoder.

Figures 2 to 4 provide illustrations of the face-mesh encoder/decoding running on a mobile device, on a Desktop browser and on a mobile device in AR mode.

Figure 2: V-DMC based communication on mobile device

Figure 3: V-DMC based communication in a desktop browser

| Figure 4: V-DMC based communication in AR on mobile device |  |
| --- | --- |

## 5. Conclusions

This contribution demonstrates that V‑DMC enables practical, web‑native, real‑time 3D face communication on currently available mobile devices, including AR support. The demonstration highlights the implementation considerations for browsers (data partitioning, HW encode/decode, reconstruction) and the potential suitability of MPEG V-DMC standard for performance-demanding real-time use cases.

## 6. References

[1] ISO/IEC FDIS 23090-29, “Information technology — Coded representation of immersive media, Part 29: Video-based dynamic mesh coding (V-DMC)”.

[2] S4-250051, “V-DMC web player demo”, 3GPP SA4 #131, February 2025, Geneva.

[3] 	“V-DMC TMM 11.0”, ISO/IEC JTC 1/SC 29/WG 7, April 2025. https://mpeg.expert/software/MPEG/3dgh/v-dmc/software/mpeg-vmesh-tm

[4] “Web framework for building virtual reality experiences”. https://github.com/aframevr/aframe/