Source:	Nokia

Title:	Nokia Listening Lab Report - IVAS Characterization Phase

Agenda Item:	7.5 - IVAS_Codec_Ph2

Document for:	Approval

Summary

This document presents a report on subjective listening testing as a part of 3GPP-SA4 IVAS Codec Characterization phase. Nokia performed 6 experiments mainly in Finnish language. All the experiments have been performed according to the test plan [1] and processing plan [2], except listening order. There were 30 naïve Finnish listeners for each P.800 [3] experiment and 14 expert listeners for each BS.1534 [4] experiment.

Key Acronyms

ACR	Absolute Category Rating

DCR	Degradation Category Rating

GAL	 Global Analysis Laboratory

HL	Host Laboratory

MC	Material Collection

LL	Listening Laboratory

XC	Cross-check Laboratory

- Introduction

Nokia Listening Test Laboratory (LL) conducted the following 6 listening test experiments for the IVAS codec Characterization phase:

Table 1 Nokia conducted listening experiments

| Nokia test | Experiment Id | Methodology | Listeners | Language |
| --- | --- | --- | --- | --- |
| A | ACR-1 | P.800 ACR | 30 naïve | Finnish |
| B | P800-14 | P.800 DCR | 30 naïve | Finnish |
| C | P800-20 | P.800 DCR | 30 naïve | Finnish |
| D | BS1534-8 | BS.15334 MUSHRA | 14 experts | - |
| E | BS1534-15 | BS.1534 MUSHRA | 14 experts | - |
| F | BS1534-18 | BS.1534 MUSHRA | 14 experts | - |

The details of experimental designs are specified in the Characterization test plan [1].

- Source material

All the P.800 experiments were performed in Finnish language. The processed listening test material was provided by Host Laboratory (HL). Original Finnish language mono speech samples were provided by Nokia LL to Material collection entity (MC) in 48kHz, 16bit, mono PCM wav format. MC and HL prepared script-generated material for P800-14 (categories 1-4). Pre-produced material for P.800-14 (categories 5,6), P800-20 (all categories) and ACR-1 (all categories) was prepared in Finnish language by Nokia LL in collaboration with MC. The processing of all input material was done by HL and cross-checked by cross-check laboratory (XC) and finally provided to Nokia LL. Nokia performed MD5 comparison to all files provided and confirmed perfect copy.

All MUSHRA experiments contained material in multiple languages and included musical and other critical signals. The processed BS.1534 listening test material was provided by Host Laboratory and again cross-checked by XC. Nokia contributed some critical Finnish language samples for all BS.1534 experiments.

- Listening sessions

3.1	Presentation sequences

It was noted that the presentation order of the samples for P.800 DCR and ACR tests to the listeners was not according to the randomization tables provided by the Global Analysis Lab (GAL) due to listening test software configuration issue (accidental double randomization). The presentation order was that which can be found from the votes data delivered to GAL. Irrespective, all the samples were listened in randomized order, all the panels listened their intended samples, all the samples were listened to equal amount (5 times per sample, 30 times per category/condition, and each condition obtained 180 votes). All the listening panels had the same number of listeners (5).

The listener instructions are presented in Annex 1. These are according to [3] and [5]. All naïve listeners had first the familiarization phase with LTSW-software and did 16 practice expermiments with DCR and 20 with ACR test. After practice session, test material was listened to trough with full (Direct) quality. Then the actual listening test was conducted.

3.2	Listeners

All the listeners were native Finnish with normal hearing. There were 30 different listeners for each of the P.800 experiments. The number of female and male listeners were 22+8, 16+14 and 10+20 for experiments ACR-1, P800-14 and P800-20, respectively. The distribution is overall nominally balanced 48 + 42. The average age of listeners was 26.7; 30.5 and 38.4 years respectively; overall 31.9 years. There were 14 different experienced listeners for each of the BS.1534 experiment. The number of female and male listeners were 1+13, 2+12 and 3+11 for the experiments BS.1534-8, BS.1534-15 and BS.1534-18, respectively. The average age of listeners was 43.4; 40.9 and 39.7 years, respectively; overall 41.3 years.

No special listener elimination was conducted. Delivered results were balanced, and each listening panel had exactly five listeners.

3.3	Listening environment

Listeners were placed in high-quality, acoustically isolated booths. Six identical booths with internal dimensions of 1.4 x 1.1 x 2.1m were used. The background noise-rating curve of each booth fulfils the ISO NR15 requirement. The reverberation times within the booths are <300ms above 315Hz one-third octave bands. No discernible flutters are audible within the booths [6].

3.4	Testing facility

The listening test was done by silent Mac mini M1, (MacOS: Sequiola 15.7.1) computers running Nokia internal LTSW software. Six machines were used to play the samples to the listeners and to collect their answers. Each Mac has a studio quality sound card type, either Arturia Minifuse 1 or Arturia Minifuse 4. In experiments samples were presented to both ears over Sennheiser HD650 headphones. In P.800 tests the signal volume level was set to nominal level. In MUSHRA tests listening level was set to user preferred level during the familiarization phase.

- Results

All the raw voting data has been sent to Global Analysis Lab according to the schedule.

- Conclusion

Nokia performed experiments from A to F in Finnish language for the IVAS Characterization phase, in compliance with the Test plan [1].

References

[1]	S4-251565 IVAS Permanent Document IVAS-8b: “Test Plan for Characterization Phase”.

[2]	S4-251561 IVAS Permanent Document IVAS-7b: ”Processing Plan for Characterization Phase”.

[3]	Recommendation ITU-T P.800 (08/1996): “Methods for subjective determination of transmission quality”.

[4]	Recommendation ITU-R BS.1534 (10/2015): “Method for the subjective assessment of intermedia quality level of audio systems”.

[5]	Supplement ITU-T Suppl29 (01/2023): “ITU-T P.800 - Use cases".

[6]	M. Kylliäinen et al. ”Compact high performance listening spaces”, Euronoise, Naples, 2003.

Annex 1. Instructions to the listeners

ACR test instructions

OHJEITA KUUNTELUUN

Tervetuloa kuuntelukokeeseen. Tässä kuuntelukokeessa arvioit puhenäytteiden äänenlaatua. Kuulemiasi äänenkäsittelymenetelmiä käytetään nykyisissä ja tulevaisuuden tietoliikennejärjestelmissä.

Kokeessa kuuntelet yksittäisiä ääninäytteitä ja arvioit niitä annetulla asteikolla. Tehtäväsi on kuunnella jokainen näyte ja antaa äänenlaadusta arvio kuuntelun jälkeen. Ääninäytteen saa kuunnella vain kerran ennen arviota. Ota huomioon kaikki äänen ominaisuudet, kuten kaistanleveys, puheen vääristyminen tai äänen luonnollisuus. Käytä koko arvosteluasteikkoa testin kuluessa.

Tehtäväsi on kuunnella näyte ja antaa laadusta arvio käyttäen alla olevaa arvosteluasteikkoa. Ota huomioon puhe-, että yleinen äänenlaatu, sekä tilaäänen laatu. Näytteen voi kuunnella vain kerran.

5	Erinomainen
	4	Hyvä
	3	Kohtalainen
	2	Välttävä
	1	Huono

Koe on jaettu kolmeen osioon. Ensimmäisessä osiossa harjoittelet koetehtävän tekemistä. Toisessa osiossa kuuntelet kokeessa käytettävää äänimateriaalia läpi, jossa kuulet alkuperäisen laadun. Kolmannessa osiossa suoritat varsinaisen kuuntelukokeen. Lopuksi voit vielä halutessasi antaa kirjallista palautetta kokeesta.

Älä keskustele ääninäytteiden äänenlaadusta muiden osallistujien kesken.

DCR test instructions

OHJEITA KUUNTELUUN

Tervetuloa kuuntelukokeeseen. Tässä kuuntelukokeessa arvioit ääninäytteiden eroa.

Koe suoritetaan parivertailuna, jossa kuulet aina kaksi ääninäytettä peräkkäin. Ääninäyteparin saa kuunnella vain kerran ennen arviota. Tehtäväsi on arvioida jälkimmäisen näytteen äänenlaatua ensimmäiseen näytteeseen verrattuna. Ota huomioon kaikki äänen ominaisuudet, kuten puheen vääristyminen tai tilaäänen laatu.

Jälkimmäisen näytteen äänenlaadun muutos verrattuna ensimmäiseen on:

5	Heikennys ei ole Kuultavissa

4	Heikennys on Kuultavissa, muttei Häiritsevä

3	Heikennys on Lievästi Häiritsevä

2	Heikennys on Häiritsevä

1	Heikennys on Erittäin Häiritsevä

Anna mielipiteesi mukainen arvosana jälkimmäisen näytteen mahdolliselle äänenlaadun heikentymiselle ylläolevan asteikon mukaisesti.

Koe on jaettu kolmeen osioon. Ensimmäisessä osiossa harjoittelet koetehtävän tekemistä. Toisessa osiossa kuuntelet kokeessa käytettävää äänimateriaalia läpi, jossa kuulet alkuperäisen laadun. Kolmannessa osiossa suoritat varsinaisen kuuntelukokeen. Lopuksi voit vielä halutessasi antaa kirjallista palautetta kokeesta.

Älä keskustele ääninäytteiden äänenlaadusta muiden osallistujien kesken.

Mushra test instructions

OHJEITA KUUNTELUUN

Kokeessa sinulle esitetään useita ääninäytteitä yhtä aikaa joiden välillä voit vapaasti vaihtaa toistoa. Arvioitavien näytteiden lisäksi mukana on Alkuperäinen-näyte, joka vastaa täyttä 100 pisteen laatua. Tehtäväsi on arvioida kaikkien ääninäytteiden laatu annetulla asteikolla niin, että otat huomioon näytteiden laadun suhteessa toisiinsa ja Alkuperäinen-näytteeseen. Voit kuunnella ja vaihdella näytteitä vapaasti niin paljon kuin haluat. Lisäksi voit säätää toiston toistamaan haluattua lyhyempää pätkää koko näytteestä. Kokeessa erot voivat olla pieniä tai suuria, mutta yleisesti ei kannata yrittää liian pitkään löytää eroja näytteistä.

Koe on jaettu kolmeen osioon. Ensimmäisessä osiossa harjoittelet koetehtävän tekemistä. Toisessa osiossa kuuntelet kokeessa käytettävää äänimateriaalia läpi alkuperäisellä laadulla. Kolmannessa osiossa suoritat varsinaisen kuuntelukokeen. Lopuksi voit vielä halutessasi antaa kirjallista palautetta kokeesta.