3GPP TSG SA WG 4 Meeting #134 	S4-251807

17-21 November 2025, Dallas, Texas, USA

Source:	Fraunhofer IIS

Title:	On complexity design constraints for ULBC

Spec:	3GPP TR 26.940

Agenda item:	7.9

Document for:	Discussion

1. Introduction

This document aims to provide clarification and proposals for the complexity design constraint for ULBC. Special focus is given on the ROM size of ULBC and technical benefits are discussed.

2. Diskussion

2.1 Recent proposals

A first proposal regarding memory constraint is for instance given in S4aA250205 where a constraint to the maximum number of parameter of 5-10M is proposed. The reasoning for this number is given by the fact that larger model sizes may lead to more frequent access of DRAM, which can lead to higher power consumption. Consequently, the power consumption is determined by the number of parameters which are inferenced at the same time. As discussed in the following, there are cases, where different sets of parameters are beneficial while their application may not overlap.

2.2. Technology analysis of ML codecs

Considering the ML coding technology DAC as listed in TR 26.940 V0.2.0 (S4-251152) clause 7.1.1, there are two versions given: (original) DAC and the DAC-IBM. Both share the same model architecture; however, the parameters are trained with the focus to operate at different bit rates. DAC focuses on higher rates, while DAC-IBM is trained for very low bitrates. The quality difference of both versions is substantial, as can be seen by the following P.808 excerpt from (S4aA250217) where the two versions of DAC are both operating at 1.5 kbps. For noisy speech and de-noised/clean speech the version trained on the low bitrates (DAC-IBM) outperforms the original version at the same bitrate by far more than 1 MOS. The performance of DCA-IBM at 1.5 kbps is similar to DAC at 6 kbps.

Figure 1 P.808 ACR test results

Consequently, it is evident that including different sets of parameter for different bit rates is beneficial for low rate codecs.

2.3. Impact on design constraints

One the one hand, the design constraints need to limit the overall power consumption when inferencing an ML codec. Therefore, a limitation on the number of parameters per mode needs to be specified. A mode usually determines a specific operation point, e.g. a certain bitrate. One the other hand, several sets of parameters may be required for optimal performance at different modes. To limit the number of alternative parameter sets, a total number of parameters should be specified additionally.

Note that, a mode usually operates over a longer period or the entire session. Even in case of mode switching, the impact on power consumption is limited due to rare switching events in a real-world application. Whether rate adaption is possible at all over NTN NB-IoT, is FFS.

3. Proposal

It is proposed to adapt the following changes to TR 26.940.

* * * First Change * * * *

## 6.2	Design Constraint Parameter

Table 6.2-1 List of ULBC design constraint parameter

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth |  |  |
| Frame length |  |  |
| Complexity and memory demands | The model size per operation mode is less than [5-10] million parameter. The total number of parameters is less than [Z] million. |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec |  |  |
| Discontinuous transmission including voice activity detection and comfort noise |  |  |
| Robustness to non-speech input |  | Editor’s note: May need to be in performance requirement |

* * * Next Change * * * *

## 7.x	Training dependency of ML codecs

### 7.x.1		Bit rate dependency

Considering the ML coding technology DAC as listed in clause 7.1.1, there are two versions given: (original) DAC and the DAC-IBM. Both share the same model graph; however, the parameters are trained with the focus to operate at different bit rates. DAC focuses on higher rates, while DAC-IBM is trained on very low bitrates. The quality difference of both versions is substantial, as can be seen by the following P.808 excerpt from clause 7.3.2.4 where the two versions of DAC are both operating at 1.5 kbps and a 6 kbps version of DAC is added as check point.

Figure 7.x.1-1 P.808 ACR excerpt from clause 7.3.2.4

The following observations are by Figure 7.x.1-1:

- For noisy speech and de-noised/clean speech the version trained specifically on low bitrates (DAC-IBM) outperforms the original version (DAC) by far more than 1 MOS when operating at the same rate.

- The performance of DCA-IBM at 1.5 kbps is similar to DAC at 6 kbps.

- It is evident that including different sets of parameter allowing training for different bit rates is beneficial for low rate codecs.

- If training for specific bitrates is enabled, the benefit of higher bitrates is unclear

* * * End of Change * * * *