3GPP TSG-SA5 Meeting #163	S5-254603

Wuhan, China, 13 - 17 October 2025

Title:	Reply LS on OAM-centric solution for NW-side data collection

Response to:	LS R2-2506543 on Reply LS on OAM-centric solution for NW-side data collection from RAN2

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA5, Meeting #163

To:	RAN2

Cc:	RAN3

Contact person:	Sreekumar Pothera Kalloor

sreekumar.pk@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA5 would like to thank RAN2 for their LS in R2-2506543 on OAM-centric solution for NW-side data collection and considering the input provided by SA5.

SA5 acknowledges the following details and further request for clarification received RAN2:

==========Start R2-2506543==================

RAN2#131 discussed it and concluded the following:

RAN2 measurements 1 For network-side data collection for beam prediction, measurement reports include the following: ⁻ Cell identity: CGI or PCI of the cell to which the measurement results are related to. ⁻ Logged L1 radio measurement results including the beam identifiers associated to CSI-RS resources or SSBs (CSI-RS IDs or SSB IDs) and the corresponding measured L1-RSRPs. - [CB after OI discussion] Information to indicate a gap that is longer than the configured logging periodicity in the logged measurements which can happen due to event-based logging. FFS the details. 2: The required L1 measurements (i.e. L1-RSRP of CSI-RS(s) or SSBs) are defined in TS 38.215 which is referenced in the definition of M1 measurement in TS 37.320. 3: RAN2 needs to ask SA5 how to transfer the measurement report with the above content and whether the M1 measurement defined in clause 5.4.1.1 of TS 37.320 can be used to transfer the measurement reports with the above content or not. 4: The gNB will configure the UE to log the above content via the NW-side data collection configuration, which is different from legacy immediate MDT configuration.

For the 1st agreement above, the last information is updated into: the presence of a gap that is longer than the configured logging periodicity in the logged measurements.

RAN2 notes that the required L1 measurements (i.e. L1-RSRP of CSI-RS(s) or SSBs) are defined in TS 38.215, which is referenced in the definition of M1 measurement in TS 37.320. However, RAN2 is not sure whether this measurement can be used to transfer the measurement reports with the current content mentioned above from gNB to OAM or not. On this aspect, RAN2 expects inputs from SA5.

==========End R2-2506543==================

SA5 would like to confirm that the Rel-20 study item “Data Management Phase 3” (FS_MADCOL_Ph3) intents to bring support for collection of UE generated data not supported by the exiting Trace / MDT mechanisms. We will evaluate the requirements from RAN2 as part of the study in Rel-20.

# 2	Actions

To RAN2

ACTION: SA5 kindly asks RAN2 to consider the above information in to account and contact us for further updates on the work status.

# 3	Dates of next TSG SA WG5 meetings

SA5#164		17 November - 21 November 2025		Dallas, USA

SA5#165		09 February - 13 February 2026		India, IN