3GPP TSG-SA5 Meeting #164 	S5-255063

Dallas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0093 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.550 corrections on GPB schema and descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For streaming perfMetrics, the interpretation of meas_obj_dn in message InfoContent in Annex I “GPB schema for performance data stream units” is unclear. Without any additional prefixes existing, a GPB based multi-vendor interpretation is that the “FDN” is required for every reported metric (instance and class), since the ASN.1 CHOICE of frameContent type is not reflected clearly in a “oneof” statement in the GPB version. - There is a correction to the Annex C “Info Content” using plural “measObjDns” which is not aligned with the GPB or ASN.1 in Annex I.1 and Annex G.2. It is unclear the criteria under which a new infoContent frame must be sent and also the loose association using stream_id between the infoContent frame and subsequent dataContent frames. - The “performance_metrics” (plural) string is only clear if it can be a list of metric names, which delimiter must be used e.g. “metric1,metric2,metric3,etc”. - There is an order between the entries in the performance_metrics string and the order of the DataContent frames, as these can send both standardized and vendor specific metrics. All the standardized should come first in the performance_metrics string followed by any vendor specific metrics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce “one of” use in message FrameContent (to match ASN.1 CHOICE as defined in Annex G) - Remove “plural” from “measObjDns” and correct description to singular as appropriate. - Adjust chapter and table to allow place to describe the outermost container and all contents more clearly. - Add mention of GPB unique delimiter between PDSUs to be aligned with ASN.1 - Clarifications on streamId (and more common text between description in both frames. - Other editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient Use of Network and Storage Resources and risk for non multi-vendor implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C, Annex I.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

Annex C (normative): 
Performance Data Stream content description

Table C-1 lists all the Performance Data Stream content items. It also provides an explanation of the individual items.

Table C-1: Performance Data Stream content description

| Performance Data Stream Content |  | Description |
| --- | --- | --- |
| PDSU | pdsu | Performance Stream Data Unit, each is equivalent to what is sent per RP or smaller numbers of PDSU. Each PDSU has an infoFrameIndicator and Frame Content which transports either a Data Content frame or an Info Content frame. Note: in GPB only, each "PDSU" are preceded with a GPB variable length 32 bit integer that indicates the size of the GPB encoded PDSUs. |
| Data Content | streamId | The streamId of the performance data stream. This shall be the same as the Info Content frame associated with this Data Content frame. |
|  | granularityPeriodEndTime | Time stamp referring to the end of the granularity period. |
|  | measResults | This parameter contains the sequence of result values for the observed metrics (measurement types or KPIs). The "measResults" sequence shall have the same number of elements, which follow the same order as the measurement types or KPIs presented in "performanceMetrics" for the subject stream in the input parameter streamInfoList of the establishStreamingConnection operation (see clause 6.2). |
| Info Content | streamType | This carries the information of the kind of data a stream is carrying and this would have the value “PERFORMANCE”. |
|  | SerializationFormat | This indicates the format of the serioalization, it can contain “ASN.1” or “GPB”. |
|  | streamId | This contains the id of the stream. This shall apply to all subsequent Data Content frames with the same streamId. |
|  | measObjDn | This contains the DN (FDN) of the measured object that is reported as part of this stream. |
|  | performanceMetrics | This contains the performance measurements by name only that are reported by this stream. If more than one metric name, a comma separation/delimiter shall be used, i.e. "metric1, metric2, metric3, …". All standardized metrics for this measObjDn should be listed first, followed by all vendor specific. |

*** START OF NEXT CHANGE ***

# I.1	Performance Data Stream Units (GPB) schema

Normative GPB schema for Performance Data Stream Units (see Annex C).

syntax = "proto3";

import "google/protobuf/timestamp.proto";

message PDSUs {

message PDSUType {

message SubCounterIndexType {

oneof type {

string sum = 1;

int32 bin_index = 2;

int32 qOS_5QI = 3;

int32 qOS_QCI = 4;

int32 cause = 5;

string string_index = 6;

bytes plmn = 7;

SNSSAI snssai = 8;

}

message SNSSAI {

bytes sst = 1;

bytes sd = 2;

}

}

message MeasValue {

oneof meas_value_type {

int64 integer_value = 1;

double real_value = 2;

string string_value = 3;

SubCounterListType sub_counters = 4;

}

}

message InfoContent {

string stream_type = 1;

string serialization_format = 2;

int64 stream_id = 3;

string meas_obj_dn = 4;

string performance_metrics= 5;

}

message DataContent {

int64 stream_id = 1;

google.protobuf.Timestamp granularity_period_end_time = 2;

repeated MeasValue standardized_meas_results = 3;

repeated MeasValue vendor_specific_meas_results = 4; // may be omitted

}

// uses recursion for the value to support multi-dimensional measurements

message SubCounterListType {

SubCounterIndexType sub_counter_index = 1;

optional MeasValue sub_counter_value = 2;

}

message FrameContent {

oneof frame_content {

InfoContent info_content = 1;

DataContent data_content = 2;

}

}

message FrameContent {

InfoContent info_content = 1;

DataContent data_content = 2;

}

bool info_frame_indicator = 1;

FrameContent frame_content = 2;

}

repeated PDSUType pdsu = 1;

}

*** END OF CHANGE ***