3GPP TSG-SA5 Meeting #164	S5-255248

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.567 Remove duplicated attribute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of attribute toleranceLimits is duplicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove one definition for attribute toleranceLimits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant and confusing definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.4.1	Attribute properties

Table 6.4.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL__TARGETED__SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Editor’s Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ScopeDescription | It indicates the description of the scope that is instantiated or being informed about. It is defined according to the ScopeDefinition in TS28.561 | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectParameters | It indicates the list of parameters on the objects in the ScopeDescription which are part of the scope. This applies when the scope is of type measurement scope or control scope. allowedValues: string | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| coordinationCapability | It indicates a capability of a coordination entity to coordinate CCL conflicts | type: CoordinationCapability multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLCoordinationCapabilityID | It indicates an identifier for a specific CCL conflicts coordination capability | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| closedControlLoopRefList | It indicates a list of DN for ClosedControlLoop Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLScopeCoordinationCapability | It indicates a CCL scope assignment and conflict coordination capacity | type: CCLScopeCoordinationCapability multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLTriggerCoordinationCapability | It indicates a specific type of CCL trigger coordination functionality of the ConflictManagementAndCoordinationEntity | type: CCLTriggerCoordinationCapability multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionCoordinationCapability | It indicates a specific type of CCL conflict coordination functionality of the ConflictManagementAndCoordinationEntity | type: CCLActionConflictsHandling multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLMetricValueCoordinationCapability | It indicates a specific type of CCL conflict coordination functionality of the ConflictManagementAndCoordinationEntity | type: CCLMetricValueCoordinationCapability multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| coordinatedCCLsScopes | It indicates the scopes of the CCL that are coordinated by the coordinationEntity It is a pair <string_1, string_2 > where string_1 is the DN of a CCL being coordinated and string_2 the DN of that CCL’s CCLScope. | type: pair <string, string > multiplicity: 2 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operationalState | It indicates the operational state of the ClosedControlLoop instance. It describes whether the resource is installed and partially or fully operable (Enabled) or the resource is not installed or not operable (Disabled). AllowedValues; Enabled/Disabled allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [8] and ITU-T X.731 [9]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Disabled isNullable: False |
| administrativeState | It indicates the administrative state of the ClosedControlLoop instance. It describes the permission to use or the prohibition against using the ClosedControlLoop instance. The administrative state is set by the MnS consumer. AllowedValues; Locked/Unlocked allowedValues: "LOCKED", "UNLOCKED". The meaning of these values is as defined in 3GPP TS 28.625 [8] and ITU-T X.731 [9]. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Locked isNullable: False |
| cCLComponentsInfo | It indicates information on the constituent components of a CCL. allowedValues: N/A | type: CCLComponentInfo multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentId | It indicates the identifier of a CCL component. It is the DN of an object instantiated to act as a component of the CCL | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLSteps | It indicates the CCL steps or functionality that is accomplished by a CCL component. allowedValues: DATA_COLLECTION, ANALYSIS, DECISION, EXECUTION | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| faultManagementAlarmIdList | It describes the list of IDs of alarms to be managed by Fault Management CCL. allowedValues: A list of alarmIds as specified in TS 28.111 [4], clause 7.4.1 | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| faultManagementTimeWindow | It describes the information of a time window (including start and end time) specified by the consumer for fault management to carry out troubleshooting and to clear the alarms. allowedValues: timeWindow as defined in 3GPP TS 28.622 [5], clause 4.4.1 | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| faultManagementBackUpObjectRequirement | It describes whether to back-up the alarmed object is required by the consumer before fault management. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| faultManagementIsolateObjectRequirement | It describes whether to isolate the alarmed object from interaction with other objects is required by the consumer before fault management. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the Fault Management CCL who is the consumer that invokes the clearAlarms operation. allowedValues: clearUserId as defined in 3GPP TS 28.111 [4], clause 7.4.1 | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| faultManagementCCLReport | It describes the Fault Management CCL report. allowedValues: Not Applicable | type: FaultManagementCCLReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| generatedAlarmResultList | It describes the list of generated alarm results allowedValues: A list of GeneratedAlarmResult | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| faultManagementCCLReportTime | It describes the time when the FaultManagementCCLReport is created. allowedValues: DateTime as specified in TS 28.622 [5]. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmId | It identifies an AlarmRecord as specified in TS 28.111 [4] allowedValues: A string as specified in TS 28.111 [4] | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedStatus | It describes whether an alarm is cleared by the Fault Management CCL when the identified root cause is resolved. allowedValues: True, False | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| identifiedRootCauseInformation | It describes root cause information identified by the Fault Management CCL. allowedValues: String | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| enhancedCorrelationInformation | It describes the list of correlated alarm Ids identified by the Fault Management CCL allowedValues: A list of alarmId | type: List multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLActionConflictsHandling | This defines the handling of CCL action conflict between the two existing CCLs. | Type: CCLActionConflictsHandling multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| detectedActionConflict | This indicates the information related with a detected conflict CCL. It is a list of conflicts among a set of action plans that have been evaluated. Each entry is a pair of plans that are conflicting. | Type: ActionConflict multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| conflictResolution | This defines the information related with conflict resolution. | Type: ActionConflictResolution multiplicity: * isOrdered: True isUnique: False defaultValue: None isNullable: False |
| targetCCL | The identification of the CCL that need to be deleted or updated to resolve conflict. This will be decided as per the information ConflictResolution. | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingCCLId | This indicates the CCL identification | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingActions | This provides the set of actions that have been taken by the CCL as part of the Execute step. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLPriority | This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLMetricBreachPercentage | It defines the breach percentage per metric in terms of how bad the metric(s) is breached. For example, if the metric of guaranteed throughput is 200mbps and the actual throughput is coming to be 100mbps then the breach percentage would be 50%. The CCL that have higher percentage of breach will be prioritized | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLComponentList | It indicates the list of components acting as steps of the CCL, each either a MnF or a MnS producer whose services can be part of the CCL. The cCLComponent may have a role among MONITOR; ANALYSIS; DECISION; EXECUTION. Or OTHER. OTHER. Is used for example in the cases where a components fulfils more than 1 role or where the role can be simply described by the four options. The cCLComponents are sequenced, i.e., cCLComponents is an ordered list. For example, if there are 2 steps that contribute to the analysis role, it is necessary to show how those steps are sequenced. The order in which they are listed indicates the order in which their services should be chained to complete the CCL | type: CCLComponent multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cCLType | It indicates a type or Category of CCL that is to be instantiated or dynamically composition. It indicates the kind of capability that will be accomplished by the CCL instance, e.g. ENERGYOPTIMIZATION, SLICEASSURANCE, etc. The specific details, characteristics and behavior of a CCL for a given CCL type are then written into the CCL purpose. Note: The allowed values are FFS | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLComponentRole | It indicates a role accomplished by CCL component. AllowedValues: MONITOR; ANALYSIS; DECISION; EXECUTION, OTHER. Is used for example in the cases where a components fulfils more than 1 role or where the role can be simply described by the four options | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLComponentIdentification | It indicates the entity accomplishing the component. It may be the DN of an MOI or the combination of URI and DN that can be used to fulfil that role. | Type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLInstanceIdentifier | This defines the specific CCL instance | Type: Dn multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| satisfactionScore | The numerical value from 1 to 10 (1 being the worst), providing the consumer satisfaction with the CCL. | Type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| metricBreachInformation | This defines the goalrequirement breach information related with the CCL. | Type: MetricBreachInformation multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| breachedMetricIdentification | This defines the goalrequirement which got breached | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| breachedTime | This defines the time of the goalrequirement breach | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mitigationAction | This defines the configuration actions that was performed by the CCL execution to mitigate the breach. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLInstantiationTrigger | This defines dynamic closed control loop invocation criteria that can be configured by the consumer. The producer will instantiate an CCL based on the criteria defined. | Type: TriggerConditionDescriptor multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLCompositionTrigger | This defines dynamic closed control loop composition criteria that can be configured by the consumer. The producer will compose an CCL based on the criteria defined. | Type: TriggerConditionDescriptor multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| closedControlLoopRef | This refers to the CCL that is composed or instantiated using triggers. | Type: Dn multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLActionTrigger | This defines the criteria/conditions under which the CCL is allowed to take actions. | Type: CCLTrigger multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| desiredBehavior | This will define the corresponding behavior of the CCL. The behaviours can be represented by an ENUM to include: - DECISION_ACTIVATION: The CCL executes the recommendations that it derives on to the network. - NOTIFY_RCOMMENDATION: The CCL starts processing input to derive recommendations but without the corresponding actions executed on the network. Instead, the recommendation is notified to the consumer who then considers whether it should be applied or not. - DO_NOTHING: do not do anything. | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| scopeOutcomes | It indicates the set of outcomes to be coordinated for a given scope as part of scope coordination. It is a pair <A,B> where A is the metric and B the desired outcome on that metric. | Type: pair<string, Real> multiplicity: 1...* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictID | It identifies a conflict event | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingCCLs | It identifies the set of CCLs that are conflicting | type: DN multiplicity: 2 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conflictScope | It indicates the scope for which two or more CCLs are conflicting. | Type: ScopeDefinition multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ConflictType | It indicates the type of conflict that has been observed, i.e., either a potential conflict or an actual conflict. allowedValues: POTENTIAL_CONFLICT; ACTUAL_CONFLICT | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coordinatedScopeTypes | It indicates the types of scopes under consideration for coordination by a scope coordination functionality. allowedValues: CCLMEASUREMENTSCOPE, CCLTARGETSCOPE, CCLCONTROLSCOPE, CCLIMPACTSCOPE, CCLMONITOREDSCOPE | Type: ENUM multiplicity: 1 ..5 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| fullCoordinatedScopeSpace | It indicates the full scope which is to be considered by the CoordinationEntity when selecting sub-allocations to different CCL instances. | Type: Scope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| toBeCoordinatedCCLScopes | It indicates the list of scopes which the coordinatinEntity is responsible for coordinating to ensure they have no conflicts. A CCL that requires its scope to be evaluated for conflicts can add its scope set into the list of scopes sets | Type: CCLScope multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| detectedScopeConflicts | It indicates the list of scope conflicts that are detected by the coordinationEntity. Each entry is of type: scope conflict | Type: ScopeConflict multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| detectedTriggerConflicts | It indicates the list of trigger conflicts that are detected by the coordinationEntity. Each entry is of type: TriggerConflict | Type: TriggerConflict multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| precedentEntities | It indicates the set of instances of CCLs or other functionality that should be executed before the CCL | Type: DN multiplicity: 1 ..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cCLhierarchyList | It indicates the ordered list of CCL instances defining the order in which CCLs should be executed. It is an ordered list where the first entry is the one to be executed first. | Type: DN multiplicity: 1 ..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| toBeCoordinatedPrecedentCCLs | It indicates the set of instances of CCLs or other functionality that need to be coordinated | Type: DN multiplicity: 1 ..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| detectedScopeConflicts | It indicates the list of scope conflicts that are detected by the coordinationEntity. Each entry is of type: ScopeConflict. | Type: ScopeConflict multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| actionPlanID | It identifies an actionPlan generated by a CCL | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCLID | It identifies the DN of a CCL that has generated an actionPlan | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| actions | It indicates the CM changes proposed a CCL | type: PlannedConfigurationDescriptor multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| toBeCoordinatedActionPlans | It indicates the list of action plans which the coordinatinEntity is responsible for coordinating to ensure they have no conflicts. A CCL that requires its action plan to be evaluated for conflicts can notify its plan to the coordinationEntity to then be added to an appropriate list of toBeCoordinatedActionPlans. Each list includes plans with related (or same) scope in managed objects and time. | Type: ActionPlan multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| cCLParameterValuesUsefulness | It indicates the relative goodness of different values of the parameter to the CCL. It a list of pairs <A, B> where A is a value of CCL control parameter and B is an integer indicating the usefulness of value A. B is in the scale [0:100], where “0” indicates that the value is useless while “100” indicates that the functionality of the CCL completely depends on that value. allowedValues: [0, 100] | Type: pair<string,integer> multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cCLinterestInConflictParameter | It indicates CCL’s relative interest in the parameter. It is a measure of how useful different parameters are to the objectives of the CCL, regardless of how useful specific values of those parameters contribute to fulfilling those objectives. allowedValues: [0, 100] | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictMonitoringContext | It indicates the scope that one CCL B notify to another CCL A to monitor and ensure to maintain the performance within some stated limits. It is written by the CCL B into coordinatinEntity as the pair pair<actionID, Scope> where actionID is the identifier of a previous action that has been taken by a CCL and Scope is the scope which that CCL wants other CCLs to maintain within certain limits | Type: pair<actionID, Scope> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| toleranceLimits | It indicates the limits within which the compromise on the parameters and metrics can still be acceptable. It is an integer indicting the acceptable percentage change in the values on parameters in a specific action plan. allowedValues: [0, 100] | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ComputedCompromizePlans | It indicates the compromise action plans that are recommended by the coordinationEntity for each CCL. It is list with each entry a pair <CCL_ID, compPlan> where CCL_ID is the identifier of a CCL for which a compromise plan has been computed, and compPlan is the proposed compromise plan | Type: ActionPlan multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| desiredMetrics | It indicates the set of metrics that the CCL intends to optimize. These need to be coordinated among several CCLs, e.g. so that 2 CCLs don’t aim to optimize the same metric | Type: string multiplicity: 1...* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| proposedReviseddActionPlan | It indicates a compromise action plan proposed by the coordination entity for the case where the action plan executed by a CCL resulted in metric value conflict | Type: ActionPlan multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| actionPlanFailedCriteria | It indicates criteria which an action plan for which an action plan failed and caused metric value conflicts. | Type: string multiplicity: 1...* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| TrustedCCLs | It indicates the list of CCL that have performed consistently well and have achieved full trust that not further check of their actions is necessary. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observedMetricValueConflicts | It indicates the list of observed metric value conflicts | Type: MetricValueConflict multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| correlatedOscillationMetrics | It indicates the metrics noted to be experiencing correlated oscillations | Type: string multiplicity: 1..* isOrdered: False isUnique: TruedefaultValue: None isNullable: False |
| conflictingMetrics | It indicates the list of metrics that are in conflict | Type: string multiplicity: 1...* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| detectedMetricValueConflicts | It indicates the list of MetricValueConflicts that are detected by the coordinationEntity. Each entry is of type: MetricConflict | Type: MetricValueConflict multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| flipflopMetrics | It indicates the list of metrics that are observed by a CCL as flip flopping. It is a pair <objDN, ffmetric> where objDN is DN of the managed object whose metric is flipflopping and ffmetric is identifier of the flip flopping metric. | Type: pair <DN, string> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * End of Changes * * * *