3GPP TSG-SA5 Meeting #164	S5-255316

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0093 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.274 Correction of Charging Characteristics use in SMSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The information that the charging characteristics can indicate should be aligned with SMF and AMF. Wherever to use an NRF lookup is guided by the information received, e.g. if there is only a CHF Set Id then the SMSF must do a NRF lookup. Since charging characteristics may be provisioned in the SMSF they are applicable for both the SMSF and the UDM. The PCF may update the Charging information, CHF Group ID, as well as the Default charging method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separation of Charging information and CHF Group ID.´ Removing the CHF selection method from the table B.1.1. Make the charging characteristics usage more generic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is inconsistent which may lead to different interpretation and interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.3 and B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 5.4.1.3	CHF selection

The CHF selection by the SMSF is done during the initial procedures, this selection may be based on the following options and with priority order depending on Operator's policies:

-	NRF based discovery upon SMS activation from AMF to SMSF.

-	UDM provided charging characteristics.

-	SMSF locally provisioned charging characteristics.

The charging characteristics may be used to indicate charging information (i.e. CHF address and optionally the associated CHF instance ID and CHF set ID) and CHF Group ID, for charging characteristics see annex B and NRF based discovery see TS 32.290 [57] clause 6.1.

In the case the charging characteristics is not provided, then the CHF selection by SMSF is performed via NRF based discovery:

-	For UE determined by SMSF as served by the same PLMN as the SMSF (i.e. non-roaming scenario), the CHF of the PLMN can be discovered based on the UE identifier.

-	For UE determined by SMSF in V-PLMN as an in-bound roamer (i.e. roaming scenario), the CHF of the V-PLMN can be discovered based on the PLMN Id of the UE H-PLMN.

* * * * Next Change * * * *

# B.1	General

A subscriber may have Charging Characteristics assigned to his subscription. Default Charging Characteristics may also be pre-provisioned on the SMSF.

The Charging Characteristics from UDM, if any, shall override the SMSF pre-provisioned Charging Characteristics.

The Charging Characteristics parameter consists of a string of 16 bits designated as behaviours, freely defined by Operators. These behaviours may be things like:

-	Charging information: CHF address and optionally the associated CHF instance ID and CHF set ID to be used by the SMSF

-	CHF Group ID: the CHF Group ID to be used by the SMSF

-	Charging scenario: indicates the charging scenario to be used i.e., ECUR, IEC, or PEC

One usage may be as a behaviour index associated to SMS Submit, SMS to deliver, SMS Submit Answer and SMS Deliver Answer in converged charging as described in the example in Table B.1.1:

Table B.1.1: Example of Charging Characteristics behaviours for SMSF

|  |  |  | SMS Submit | SMS to deliver | SMS Submit Answer | SMS Deliver Answer |
| --- | --- | --- | --- | --- | --- | --- |
| Behaviour Index | Charging information | CHF Group ID | Charging scenario | Charging scenario | Charging scenario | Charging scenario |
| 0 | CHF Set ID 1 | - | ECUR | ECUR- | - |  |
| 1 | Primary & secondary CHF addr. 1 & 2 | - | - | - | PEC | PEC |
| 2 | - | CHF Group ID 1 | IEC | IEC | PEC | PEC |
| 3 | Primary & secondary CHF addr. 3 & 4 | - | ECUR | ECUR- | - |  |
| … | … |  | … | … | …… | … |

* * * * End of Changes * * * *