3GPP TSG-SA5 Meeting #164	S5-255408

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0759 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Update measurement definitions to include measurement sub-counter dimensions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce definition for filter and add reference to subcounter definitions. Describe the relation between filter and subcounter dimension. Add example diagrams for filters for measurements involving multiple subcounter dimensions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition for filter added. Describe the relation between filter and measurement subcounter dimensions and add examples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Filter not defined. Releation between filter and Measurement subcounter dimensions information missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.2.0, 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Conditional handover: A handover which is executed by the UE.

Dual Active Protocol Stack: A procedure for handovers where the UE connects to the target before it releases the connection to the source.

IP latency:  the time it takes to transfer a first/initial packet in a data burst from one point to another.

Key Performance Indicator (KPI): This term is defined in TS 32.401 [2].

Legacy handover: A handover that is executed by the source gNB, in contrast to Conditional Handover.

Mapped 5QI:   5QI that is used for a DRB within the gNB when a single 5QI is assigned to the DRB.

NOTE1:	In this case the mapped 5QI is used for separating certain measurements per QoS class.

NOTE 2:	Individual QoS flows into a common 5QI is specified in TS 38.473 [6].

Packet delay: the time it takes to transfer any packet from one point to another.

Packet drop rate: share of packets that were not sent to the target due to high traffic load or traffic management and should be seen as a part of the packet loss rate.

Packet loss rate: share of packets that could not be received by the target, including packets dropped, packets lost in transmission and packets received in wrong format.

Performance indicators: The performance data aggregated over a group of NFs which is derived from the performance measurements collected at the NFs that belong to the group, according to the aggregation method identified in the Performance Indicator definition.

Filter: Possibility/Feature to select a subset of the specified performance metrics, measurement or subcounter based on certain criteria.

Subcounter: This term is defined in TS 32.404 [3].

Next change

## 4.2	Filters and filter naming

### 4.2.0	General

In case a performance measurement is required for one or more specific criteria, it is convenient to use Filters to define the performance measurement of interest. "Measurement SubCounter Dimension" defined under sub-clause "e" of measurement definition template provides information on the dimensions which can be used for filtering.

### 4.2.1	Filters

Performance measurements may be subdivided by using applicable filters to form new performance measurements. Any applicable Filter(s) is(are) specified in each performance measurement definition. Performance measurements may be identified by a consumer with/without any applicable Filters, if it is allowed by the performance measurements definition.

For a performance measurement that indicates a single Filter, the separator '_' (or vendor-specific separator other than '.' or '_')  shall be used to append the Filter to a given performance measurement name. The resulting performance measurement name shall be in the following form:

<familyName>.<measurementName>_<Filter>

For a performance measurement that indicates multiple Filters, the separator ‘_’ (or vendor-specific separator other than '.' or '_')  shall be used between Filters. The order of the Filters is not important. The resulting performance measurement name shall be in the following form:

<familyName>.<measurementName>_<Filter1>_..._<FilterN>

It is possible to to append the Filter to a Subcounter. However, it is not allowed to have a Subcounter appended to a Filter.

The following examples and diagrams show the filtering in case of measurements with subcounters in different dimensions.

Example 1: In the scenario depicted by Figure 4.2.1-1, the measurement "Max number of Active UEs in the DL per cell" (specified in clause 5.1.1.23.2 of this document) is composed of subcounters which report the maximum number of active UEs per network slice identified by S-NSSAI. Each of these network slices may have multiple 5QI values with subcounter at 5QI level.

In order to retrieve the measurement value for a particular network slice for a specific 5QI, filter can be defined in the following manner.

DRB.MaxActiveUeDl_S-NSSAI4_5QI0

Figure 4.2.1-1 Measurement with subcounters in two different dimensions.

Example 2: In this example depicted in Figure 4.2.1-2, the measurement "DL Total PRB Usage"(specified in clause 5.1.1.2.1 of this document)  is composed of measurements which are reported by different network slices represented by S-NSSAI, multiple 5QI and different UE Types.

Filter can be applied for this measurement in order to retrieve the measurement for a particular network slice and a specific 5QI and a set of different UE Types.

RRU.PrbDl_S-NSSAI1_5QI0_UEType3-5

Figure 4.2.1-2 Measurement with sub-counters in three different dimensions.

### 4.2.2	Filter naming

The Filter shall be in the form: NameValue(s)

The Filter Name shall be an alphanumeric string specified in the performance measurement definition. The Filter Name shall always end with an alphabet character. The Filter Value shall be a numeric string.

For the following Filters:

-	QOS, where QOS represents the mapped 5QI or QCI value.

-	SNSSAI, where SNSSAI represents the S-NSSAI.

-	PLMN, where PLMN represents the PLMN ID.

It is possible to specify a Filter in the form of Filter Name with single Filter Value, or with multiple Filter Values. When multiple Filter Values are provided for a given Filter Name, Filter Values shall be separated by '|'.  Filter Value ranges shall be defined in the form of 'x-y'. Combination of using '|' and ranges for the Filter Values of a given Filter Name is possible. The order of the Filter Values is not important. Any other vendor-specific multiple value form is allowed for the combination. It is possible to have multiple Filter Values enclosed in square brackets '[...]' to explicitly indicate that the measurement result shall be aggregated across the specified values.

The followings are examples of possible resulting performance measurement type with:

-	no Filters: DRB.PdcpF1DelayDl

-	single Filter with single value: DRB.PdcpF1DelayDl_QOS2

-	single Filter with multiple values: DRB.PdcpF1DelayDl_QOS1|3-9

-	single Filter with aggregated values: DRB.PdcpF1DelayDl_QOS[2-5]

End of change