3GPP TSG SA5 Meeting #165	S5-260066

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0760 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of measurements for Inter-gNB LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rakuten Mobile, Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | RAN WGs have added inter-gNB LTM as a new feature in TS 38.300. This requires SA5 to add the required performance measurements for inter-gNB LTM. In addition, existing measurements need to be updated to account for inter-gNB LTM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurements and changes to support inter-gNB LTM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Observability of inter-gNB LTM is not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.11, 5.1.1.6.12, 5.1.1.6.13, A.141 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.1.1.6.11.2	Number of successful TA acquisitions for the candidate target LTM cell

a)	This measurement provides the number of successful TA acquisitions as part of the Early TA acquisition procedure for LTM (L1/L2 Triggered Mobility) for the scenario when possible Cell Switch Command has not been yet sent to UE for the Candidate Cell ID.

b)	CC.

c)	This measurement is pegged:

-	in case of inter gNB-DU LTM or inter-gNB LTM by reception of the CU-DU TA INFORMATION TRANSFER message at source gNB-DU from gNB-CU CP with included TA Value within the TA Information List IE (clause 9.2.1.24, 3GPP TS 38.473 [6]) and possible Cell Switch Command has not been yet sent to UE for the Candidate Cell ID.  In case the TA Information List IE contains the TA value for more than one Candidate Cell ID (candidate target cell) the pegging is done for each of the cells. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

-	in case of intra gNB-DU LTM internally at source gNB-DU after successful TA acquisition per each Candidate Cell ID (candidate target cell) and possible Cell Switch Command has not been yet sent to UE for the Candidate Cell. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

NOTE:	In case of intra gNB-DU LTM the source gNB-DU is also candidate target gNB-DU.

d)	Each measurement is an integer value.

e)	The measurement name has the form MM.TAAckSuccLTM.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.11.3	Number of successful but late TA acquisitions for the candidate target LTM cell

a)	This measurement provides the number of successful but late TA acquisitions as part of the Early TA acquisition procedure for LTM (L1/L2 Triggered Mobility).

b)	CC.

c)	This measurement is pegged:

-	in case of inter gNB-DU LTM or inter-gNB LTM by reception of the CU-DU TA INFORMATION TRANSFER message at source gNB-DU from gNB-CU CP with included TA Value within the TA Information List IE (clause 9.2.1.24, TS 38.473 [6]) but Cell Switch Command has been already sent to UE for the Candidate Cell ID. In case the TA Information List IE contains the TA value for more than one Candidate Cell ID (candidate target cell) the pegging is done for each of the cells. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

-	in case of intra gNB-DU LTM internally at source gNB-DU after successful TA acquisition per each Candidate Cell ID (candidate target cell) but Cell Switch Command has been already sent to UE for the Candidate Cell. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

NOTE:	In case of intra gNB-DU LTM the source gNB-DU is also candidate target gNB-DU.

d)	Each measurement is an integer value.

e)	The measurement name has the form MM.TAAckSuccLateLTM.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.1	Distribution of time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (successful scenario)

a)	This measurement provides distribution of the time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (LTM) in successful scenario. The successful scenario reflects here to a scenario when early TA acquisition is successfully completed before the LTM is triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38.401 [66]) with TA value include for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38.401 [66]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38.401), and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the clause 9.2.6 of TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38.401 [66]) with TA value include for the candidate target cell of the gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38.401 [66]) which was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter-gNB LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 14 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) with TA value included for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 10  in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB (step 11 in Figure 9.2.3.5.2-2 of TS 38.300 [49]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [66]) which is triggered on successful transmission of  RA Preamble by the source gNB to UE for Early TA acquisition procedure as defined in the clause 9.2.6 of TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckLTMSuccDist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.2	Distribution of time interval between initiation and successful completion of Early TA acquisition (successful scenario)

a)	This measurement provides distribution of the time interval between initiation and successful completion of Early TA acquisition related to LTM in successful scenario. The successful scenario reflects here to a scenario when early TA acquisition is successfully completed before the LTM is consequently triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38.401 [66]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38.401 [66]) which was consequently followed with sending the Cell switch command to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38.401 [66]) with TA value include, and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38.401 [66]) was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU which was consequently followed with sending the Cell switch command to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38.401 [66]) with TA value include, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter-gNB LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 11 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which was consequently followed with sending the Cell switch command to UE from the source cell of source gNB-DU serving the UE in this point of time (step 14 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) with TA value include, and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) which is triggered on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckSuccDist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.3	Distribution of time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (unsuccessful scenario 1)

a)	This measurement provides distribution of the time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (LTM) in unsuccessful scenario 1. The unsuccessful scenario 1 reflects here to a scenario when early TA acquisition is successfully completed but the LTM was already triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38.401 [66]) with TA value not included for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38.401 [66]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38.401 [66]) but after sending the Cell switch command to UE (after the step 18 in Figure 8.2.1.5-1 of TS 38.401 [66]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38.401 [66]) with TA value include for the candidate target cell of the gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38.401 [66]) which was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU but after sending the Cell switch command to UE (after the step 13 in Figure 8.2.1.4-1 of TS 38.401 [66]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter gNB LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 14 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) with TA value not included for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU (step 11 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) but after sending the Cell switch command to UE (after the step 14 in Figure 9.2.3.5.2-2 of TS 38.300 [49]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which is triggered on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckLTMUnSucc1Dist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.4	Distribution of time interval between initiation and successful completion of Early TA acquisition (unsuccessful scenario 1)

a)	This measurement provides distribution of the time interval between initiation and successful completion of Early TA acquisition related to LTM in unsuccessful scenario 1. The unsuccessful scenario 1 reflects here to a scenario when early TA acquisition is successfully completed but the LTM was already triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38. 401 [66]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38.401 [66]) but after sending the Cell switch command to UE (after the step 18 in Figure 8.2.1.5-1 of TS 38. 401 [66]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38. 401 [66]) was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU but after sending the Cell switch command to UE (after the step 13 in Figure 8.2.1.4-1 of TS 38. 401 [66]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38.401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter-gNB LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU (step 11 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) but after sending the Cell switch command to UE (after the step 14 in Figure 9.2.3.5.2-2  of TS 38.300 [49]), and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) which is triggered on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckUnSucc1Dist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.5	Distribution of time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (unsuccessful scenario 2)

a)	This measurement provides distribution of the time interval between initiation of Early TA acquisition and initiation of L1/L2 Triggered Mobility (LTM) in unsuccessful scenario 2. The unsuccessful scenario 2 reflects here to a scenario when early TA acquisition is successfully completed before the LTM is triggered however source cell side evaluates the TA value as invalid at the point in time when LTM is triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38. 401 [66] ) with TA value not included for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38. 401 [66]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38. 401 [66]) before sending the Cell switch command to UE (after the step 18 in Figure 8.2.1.5-1 of TS 38. 401 [66]) but source of source gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38. 401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38. 401 [66]) with TA value include for the candidate target cell of the gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38. 401 [49]) which was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU before sending the Cell switch command to UE (after the step 13 in Figure 8.2.1.4-1 of TS 38. 401 [66]) but the source cell of the gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38. 401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter-gNB LTM:

Each sample is obtained as difference between the point in time when Cell switch command is send to UE from the source cell of source gNB-DU serving the UE in this point of time (step 14 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) with TA value not included for the candidate target cell of the candidate target gNB-DU for which UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU (step 11 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) before sending the Cell switch command to UE (after the step 14 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) but source of source gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [49]) which is triggered on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckLTMUnSucc2Dist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

5.1.1.6.12.6	Distribution of time interval between initiation of Early TA acquisition and successful completion of Early TA acquisition (unsuccessful scenario 2)

a)	This measurement provides distribution of the time interval between initiation of Early TA acquisition and completion of the TA acquisition in unsuccessful scenario 2. The unsuccessful scenario 3 reflects here to a scenario when early TA acquisition is successfully completed before the LTM is triggered however source cell side evaluates the TA value as invalid at the point in time when LTM is triggered. The measurement is provided per source and target candidate cell pair and optionally may be also provided per 5QI.

b)	CC.

c)	Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 13  in Figure 8.2.1.5-1 of TS 38. 401 [66]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU ((step 15 in Figure 8.2.1.5-1 of TS 38. 401 [66]) which was consequently followed with sending the Cell switch command to UE from the source cell of source gNB-DU serving the UE in this point of time (step 18 in Figure 8.2.1.5-1 of TS 38. 401 [66] ) with TA value include, but source of source gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 13 in Figure 8.2.1.5-1 of TS 38. 401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Intra gNB-DU LTM:

Each sample is obtained as difference between the point in time when TA acquisition procedure (step 11  in Figure 8.2.1.4-1 of TS 38. 401 [66]) was successfully completed via reception of the TA value in the source cell from candidate target cell internally within the gNB-DU which was consequently followed with sending the Cell switch command to UE from the source cell of gNB-DU serving the UE in this point of time (step 13 in Figure 8.2.1.4-1 of TS 38. 401 [66]) with TA value include, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38. 401 [66]) but the source cell of the gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 11 in Figure 8.2.1.4-1 of TS 38. 401 [66]) which is in details triggered  on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in gNB-DU per source and target candidate cell pair.

Inter gNB-DU LTM:

Each sample is obtained as difference between the point in time when early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) for the candidate target cell of target gNB-DU was successfully completed via reception of the TA value in the source cell of source gNB-DU within the CU-DU TA INFORMATION TRANSFER message from gNB-CU (step 11 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) which was consequently followed with sending the Cell switch command to UE from the source cell of source gNB-DU serving the UE in this point of time (step 14 in Figure 9.2.3.5.2-2  of TS 38.300 [49]) with TA value include, but source of source gNB-DU evaluated the TA value as invalid, and point in time when UE was previously instructed to start early TA acquisition procedure (step 10 in Figure 9.2.3.5.2-2 of TS 38.300 [66]) which is triggered on successful transmission of  RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6, 3GPP TS 38.300 [49].

The source gNB-DU increments the corresponding bin with the delay range where the measured time interval falls into by 1 for the counters. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

d)	Each measurement is an integer.

e)	The measurement name has the form MM.TAAckLTMUnSucc3Dist.Bin.5QI, where Bin indicates a delay range which is vendor specific.

f)	NRCellDU;

NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

==============Next change==============

##### 5.1.1.6.13	Inter-gNB LTM Cell Switches

5.1.1.6.13.1	Number of configured LTM Cell Switch candidates

a)	This measurement provides the number of outgoing inter-gNB LTM Cell Switch candidates requested by the source NRCellCU.

b)	CC.

c)	On transmission of RRCReconfiguration message (TS 38.331 [20] clause 5.3.5), where the message denotes a LTM Cell Switch configuration, to the UE configuring an inter-gNB LTM Cell Switch from the source NRCellCU to the target NRCellCU. The counter on NRCellCU is incremented by the number of candidates configured in the LTM-Candidate IE. The counter on NRCellRelation is incremented by 1 for each relation that is present in the LTM-Candidate IE.

d)	A single integer value.

e)	MM.ConfigInterReqLTMCellSwitch.

f)	NRCellCU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

5.1.1.6.13.2	Number of UEs configured with LTM Cell Switch

a)	This inter-gNB handover measurement provides the number of UEs that has been configured with LTM Cell Switch by the source cell.

b)	CC.

c)	On transmission of RRCReconfiguration message (TS 38.331 [20] clause 5.3.5), where the message denotes a LTM Cell Switch configuration, to the UE configuring an inter-gNB LTM Cell Switch from the source NRCellCU to the target NRCellCU, the counter on NRCellCU is incremented by 1, and the counter on NRCellRelation is incremented by 1.

d)	A single integer value.

e)	MM.ConfigInterReqLTMCellSwitchUes.

f)	NRCellCU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

5.1.1.6.13.3	Number of successful LTM Cell Switches

a)	This measurement provides the number of successful inter-gNB LTM Cell Switch executions received by the source NRCellCU.

b)	CC.

c)	On reception of RRCReconfigurationComplete message (see TS 38.331 [20] clause 5.3.5) from the UE to the target NRCellCU indicating a successful inter-gNB LTM Cell Switch, the counter is incremented by 1.

d)	A single integer value.

e)	MM.LTMCellSwitchExeInterSucc.

f)	NRCellCU;

NRCellRelation.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

.

==============Next change==============

# A.141	Monitoring of LTM cell switches

In addition to monitoring of handovers, see clause A.17, there is a need to monitor the performance of LTM cell switches, see TS 38.300 [49] clause 9.2.3.5. Like handovers, LTM cell switches are used for mobility of UEs. Failures of LTM cell switches can cause service discontinuation, therefore the performance of LTM cell switches has direct impact on user experience.

The LTM cell switch procedure includes the configuration, and the execution of LTM cell switches.

Non-conditional LTM cell switches are defined for intra-gNB mobility (TS 38.401 [66] cl. 8.2.1.4), inter-gNB mobility (TS 38.401 [66] cl. 8.2.1.5) and inter-gNB mobility (TS 38.300 [49] 9.2.3.5.2). Furthermore, conditional intra-gNB LTM mobility (TS 30.401 [66]) is defined. LTM cell switches occur Intra-frequency and Inter-frequency.

As LTM cell switches can be performed between beams, it is important to have information about the beams used in source and target cells, in order to be able to optimise the performance of the beams used.

For LTM cell switch failures, measurements of the failure use cases are required for troubleshooting.

The LTM cell switch parameters setting could be specific for each NCR, and the LTM cell switch performance could vary significantly for different NCRs. Therefore, the performance needs to be measured per NCR to support LTM cell switch parameters optimization when necessary.

==============End of change==============