3GPP TSG-SA5 Meeting #165	S5-260074

Goa, INDIA 9 - 13 February 2026

Source:	Huawei

Title:	Pseudo-CR on TR 28.881 Update Use case #13 Investigation on the intent lifecycle management state transition

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

In Potential solution #2, the state “EXPLORING” is introduced to represent the feasibility check fulfilment negotiation, however, the exploration is missing. Based on Intent negotiation functionalities described in clause 4.6 in TS 28.312, Intent negotiation functionalities includes the intent negotiation functionalities in the pre-evaluation phase (including feasibility check and exploration) and intent negotiation functionalities in the fulfilment phase. To align with the concept in clause 4.6 in TS 28.312, it propose to change the state “EXPLORING” to “Negotiation” which represents the process of feasibility check, exploration and fulfilment negotiation.

Proposed Changes

* * * First Change * * *

4.13	Use case #13:	Investigation on the intent lifecycle management state transition

4.13.1	Description

3GPP TS 28.312 [1] studies the state transitions during fulfilment phase. However, the intent Lifecycle management states that describes the pre-evaluation phase (i.e. intent feasibility check and intent exploration) and negotiation in intent fulfilment phase is missing. The main difference between the pre-evaluation phase, fulfilment phase and negotiation in intent fulfilment phase is on the intentMgmtPurpose. Thus, investigation on the intent lifecycle management state transition should be made, including the intent lifecycle management state transition diagram and the state transition events based on the intentMgmtPurpose.

In clause 4.7 of 3GPP TS 28.312 [1], the state transitions have been described in figure 4.7-1. However, how intent negotiation, feasibility check, and validity are mapped to this figure and represented in intent state diagram is missing. Additionally, it is not clear that the state diagram and state transition events are combined in one figure. Thus, further enhancements should be made in intent handling state and lifecycle management from several aspects, including but not limited to the following:

-	Addressing the issue of the unclear combination of the state diagram and state transition events in one figure by explicitly defining new states and state transitions.

-	Clarifying the issues with the state transition events.

4.13.2	Potential requirements

4.13.3	Potential solutions

4.13.3.1	Potential solution #1

It proposes to add intent lifecycle management state transition diagram and intent lifecycle management state transition table to indicate the state transitions. Figure 4.13.2.1 shows the intent lifecycle management state diagram, where the number in the figure identify the changes to the intent lifecycle management state. The explanations for the changes are described in table 4.13.2.1.

Figure 4.13.2.1: intent lifecycle management state transition diagram

The transition numbers in the first column represent the intent lifecycle management state changes in figure 4.13.2.1. The interactions specified under the column "Intent lifecycle management state transition events" of table 4.13.2.1 shall be present for the transition.

NOTE 1:	The transition numbers do not indicate any strict order and not correspond to procedure steps.

NOTE 2:	Figure 4.13.2.1 may not be complete, and other states and transitions may be added.

Table 4.13.2.1: The intent lifecycle management state transition diagram table

| Transition number | Intent lifecycle management state transition events | Intent lifecycle management state |
| --- | --- | --- |
| 1 | The MnS producer creates the intent instances for intent exploration based on the received intent creation request, with the intentMgmtPurpose specified as "EXPLORATION" and the value of specific intent targets or contexts that need to be explored initially set to 'NULL'. | EXPLORATION |
| 2 | The MnS producer creates the intent instances for intent feasibility check based on the received intent creation request, with the intentMgmtPurpose specified as "FEASIBILITYCHECK" or "FEASIBILITYCHECK_WITH_EXPLORATION". | FEASIBILITYCHECK |
| 3 | The MnS producer triggers the exploration process when intentMgmtPurpose is specified as "FEASIBILITYCHECK_WITH_EXPLORATION" and the feasibilityCheckResult is infeasible. | EXPLORATION |
| 4 | The MnS producer creates or modify the intent instances during fulfilment phase based on the received request, with the intentMgmtPurpose specified as "FULFILMENT_WITH_NEGOTIATION" or "FULFILMENT_WITHOUT_NEGOTIATION". | FULFILMENT |
| 5 | The MnS producer triggers the negotiation process. The process only can be triggered when intentMgmtPurpose is specified as "FULFILMENT_WITH_NEGOTIATION". | NEGOTIATION |
| 6 | The MnS producer finishes the negotiation process. | FULFILMENT |

NOTE 3:	The table 4.13.2.1 also provides information regarding the impact of the potential solution provided in clause 4.11.3.1 for enhancing intent feasibility check capability. The potential solution provided in clause 4.11.3.1 does not require a new intent lifecycle management state in potential solutions studied in clause 4.13.3.

4.13.3.2	Potential solution #2

It proposes to add a new intent handling state diagram based on figure 4.13.3.2-1. New FULFILLING and NEGOTIATING states are defined to represent the full lifecycle of the intent and Figure 4.13.3.2-1 shows the intent handling state diagram, where the numbers identify the state changes. The explanations for the state changes are described in table 4.13.3.2-1.

Figure 4.13.3.2-1: Intent handling state diagram

Table 4.13.3.2-1: The intent handling state transition table

| Transition number | The state transition events | State |
| --- | --- | --- |
| 1 | Upon receiving an intent creation request, the MnS Producer creates the corresponding intent instance. The MnS Producer verifies the intent syntax, assess its compliance with the MnS Producer's supported capabilities, and validates its overall correctness. If the intent creation request is accepted, the intent state transitions to RECEIVED. If the request is rejected, the intent state transitions to TERMINATED. | RECEIVED |
| 2 | The MnS Producer may perform a feasibility check of the intent instance by transitioning it to the NEGOTIATING state. In this state, the MnS Producer and MnS Consumer may exchange negotiation or modification requests, or terminate the intent. The transition to NEGOTIATING may occur after the intent has been received, before the fulfilment phase to assess its feasibility or explore the best values, during the fulfilment phase to initiate the negotiation or upon a specific request from the MnS Consumer. The request can be issued by the MnS Consumer when the intent state is FULFILLING, DEGRADED or FULFILLED. | NEGOTIATING |
| 3 | If the intent is correct and is accepted, it may directly transition to FULFILLING state, or it may do so after the NEGOTIATING state, after the MnS Producer configures the intentAdminState attribute with the value "DEACTIVATED" or upon a specific request from the MnS Consumer. In the FULFILLING state, the intent handling function initiates the necessary actions to meet the intent requirements. | FULFILLING |
| 4 | If the MnS Producer determines that it is unable to take any further actions to fulfil the intent, the intent state transitions to DEGRADED. | DEGRADED |
| 5 | The MnS producer evaluates that the intent has been fulfilled. | FULFILLED |
| 6 | The MnS Producer determines that the intent, which was previously not fulfilled, has recovered and is now considered FULFILLED. | FULFILLED |
| 7 | The MnS Producer determines that the intent, which was previously considered FULFILLED, is no longer meeting the stated requirements after a period of observation. | DEGRADED |
| 8 | Upon request by the MnS Consumer, the MnS Producer configures the intentAdminState attribute with the value "DEACTIVATED" and the intent transitions to SUSPENDED. | SUSPENDED |
| 9 | The MnS Producer modifies the intent instance based on the received intent modification request. Correspondingly, the intent state shall transition from NEGOTIATING, SUSPENDED, FULFILLING, or DEGRADED to RECEIVED. | RECEIVED |
| 10 | The MnS producer deletes the intent instances upon receiving the intent deletion request. | TERMINATED |

4.13.4	Evaluation of potential solutions

Two solutions have been proposed concerning the lifecycle management state transitions. The evaluation considers how well each solution aligns with the intent states already defined in 3GPP TS 28.312 [1], the clarity and completeness of the state transitions, and the implementation complexity. 3GPP TS 28.312 [1] already defines lifecycle states ACKNOWLEDGED, FULFILLED, DEGRADED, SUSPENDED and TERMINATED.

Potential solution #1 introduces an intent lifecycle management state diagram defining discrete states for EXPLORATION, FEASIBILITYCHECK, FULFILMENT_WITH_NEGOTIATION and FULFILMENT_WITHOUT_NEGOTIATION. This explicit split between exploration, feasibility check and fulfilment clarifies the existence of different management purposes for the intent instance, but the proposal does not clearly explain how transitions occur when the intent is modified (the management purpose of an existing intent instance is changed by the MnS consumer). The value for adding extra states for each of the management purposes is also not clear.

Potential solution #2 proposes an intent handling state diagram with states RECEIVED, NEGOTIATING, FULFILLING, DEGRADED, FULFILLED, SUSPENDED and TERMINATED. These states map naturally to those in 3GPP TS 28.312 [1]: RECEIVED corresponds to ACKNOWLEDGED (initial state after creation), while FULFILLED, DEGRADED, SUSPENDED and TERMINATED mirror the normative states. The NEGOTIATING state groups together feasibility checking, exploration and fulfilment negotiation. This reflects the 3GPP TS 28.312 [1] description that the MnS producer performs an automatic feasibility check upon receiving or modifying an intent and reports the result to the consumer, who may modify, suspend or delete the intent. By combining negotiation functionalities on pre‑evaluation and fulfilment phases and introducing a separate FULFILLING state for execution, solution #2 offers a simple and implementable state machine.

In summary, solution #1 provides detailed pre‑fulfilment phases but diverges from existing 3GPP TS 28.312 [1] states and introduces extra complexity. Solution #2 maintains fidelity with existing specification states, unifies feasibility checking and negotiation, and introduces only two transient states (NEGOTIATING and FULFILLING) between intent creation and completion. This simplicity makes the state machine easier to understand and implement while still supporting negotiation and fulfilment handling.

* * * Next Change * * *

5.13	Use case #13: Improve intent life cycle documentation

The evaluation shows that potential solution #2 (clause 4.13.3.2) aligns most closely with the intent state definitions and behaviours described in 3GPP TS 28.312 [1]. States like FULFILLED, DEGRADED and SUSPENDED are already defined. By introducing only two new transient states - NEGOTIATING for feasibility check, exploration and fulfilment negotiation, and FULFILLING for the fulfilment phase - solution #2 provides a clear separation between non-fulfilment and fulfilment activities without creating additional observable states. The NEGOTIATING state captures the automatic feasibility check and any negotiation that occurs before the intent progresses to fulfilment or is deleted. The FULFILLING state explicitly represents ongoing execution, allowing the system to report progress and to transition to DEGRADED, FULFILLED, SUSPENDED or TERMINATED as appropriate.

The detailed potential solution #2 in clause 4.13.3.2 should be used as baseline for normative work. The new description and state diagram should replace the content in clause 4.7 of 3GPP TS 28.312 [1].

* * * End of Changes * * * *