3GPP TSG-SA5 Meeting #165	S5-260135

Goa, India 09 – 15 February  2026

Source:	Nokia

Title:	Pseudo-CR on CCL traceability

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TS28.889

Version:	0.2.0

Work Item:	Closed Control Loop Management phase 2

Comments

CCLs may be triggered automatically by different entities. This pCR is to add a use case for tracing the tasks triggered on a CCL.

Proposed Changes

* * * First Change * * * *

## 4.X	Closed Control Loop traceability

### 4.X.1	Description

CCLs may be instantiated or execution configured by several MnS consumers, some by other network management entities in an automated manner. E.g.,, during fulfilment of an intent, the intent handling function may need to instantiate control loops to fulfil tasks derived from the received intent.  On the other hand, a CCL A may escalate a task to another CCL B, even if CCL A does not necessarily instantiate CCL B. For such CCLs that are instantiated or triggered automatically, the operator/administrator needs visibility of the relationships between the triggering entities (e.g., intents) and the CCLs which have been created or triggered by the system. The CCL should identify the management entity that instantiated the CCL and the task which the CCL is required to accomplish for that entity.

The figure 4.X.1-1 illustrates the information and its handling. CCL B indicates the intent A that instantiated the CCL and the triggered task A1 for intent A. CCL K indicates the CCL B that triggered (escalated) task B2 on the CCL K.

Figure 4.X.1-1: Traceability information of CCL instantiated or triggered automatically by other management functionalities.

### 4.X.2	Potential requirements

REQ-CCL_Trace-1: The 3GPPP management system should provide information on the CCL to identify the management entity that instantiated CCL and the task which the CCL is required to accomplish for that entity.

### 4.X.3	Potential solutions

To address REQ- CCL _Trace-1, this solution proposes add two attributes in the CCL model definitions, i.e.:

- the identity of the entity (e.g., each intent handling function) which instantiated the CCL or triggered the specific task on the CCL

- the task for the closed loop, which is the targets with priorities for specific values to be achieved.

### 4.X.4			Evaluation of solutions

Only one potential solution is provided in clause 4.X.3 which enables the MnS consumer to identify the entities that have instantiated or triggered a closed control loop and to identify the tasks that have been instantiated on the CCL. It is a feasible solution to address requirements REQ-CCL_Trace-1.  It is recommended to proceed to normative phase.

* * * End of Changes * * * *