3GPP TSG-SA5 Meeting #165	S5-260154

Goa, India, 09 - 13 February 2026

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 32.801-01 Add New Management Scenario on Autonomous Agent Reshapes 6G Network Operation and Management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add New Management Scenario on Autonomous Agent Reshapes 6G Network Operation and Management.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1 New 6G management scenarios

Editor's note 1: This clause will contain identified 6G management scenarios and the corresponding requirements.

6.1.X Autonomous Agent

6.1.X.1 Management Scenario #1: Autonomous Agent Reshapes 6G Network Operation and Management

6.1.x.1.1 Description

6G networks introduce new architectural and service capabilities that significantly broaden the scope of network management. From an OAM perspective, future management scenarios can be grouped into two categories: traditional foundational OAM and new service‑driven OAM. Traditional foundational OAM includes functions such as performance monitoring, fault management, and configuration management, which in 6G must operate at much larger scale and complexity. New service‑driven OAM arises from emerging 6G services such as ISAC and AI‑native capabilities, which introduce more diverse managed entities and more complex operating environments. Together, these two categories greatly increase the heterogeneity and operational complexity of 6G networks and place significantly higher demands on the management system. As these traditional and new service‑driven OAM scenarios expand, the 6G management system is expected to face several challenges:

- Management scale: The growth in the number of network elements, distributed functions, and connected devices in 6G will exceed the capacity of manual procedures and fixed‑logic automation, making it difficult for operators to maintain full visibility and control.

- Real-time requirements: Millisecond‑level scheduling, service adaptation, and autonomous fault recovery require localized and time‑critical decision-making that cannot depend on human intervention.

- Limitations of human‑driven operations: Conventional operational workflows depend on domain experts to interpret network conditions and determine corrective actions. This reliance leads to slow response, high operational effort, and poor scalability, making human‑driven operations insufficient and unsustainable for 6G.

Driven by these challenges, the evolution toward higher levels of autonomous network operation becomes unavoidable. Although 5G and 5G‑Advanced introduced management data analytics (MDA) and intent‑driven management, these capabilities remain fragmented and limited in their ability to support adaptive decision‑making. MDA functions are designed to analyse management data and generate predictions, but they do not produce autonomous decisions, nor can they address complex, rare, or previously unseen scenarios where prediction alone is insufficient. Intent Handling focuses on receiving consumer requirements and interpreting them into intents through predefined intent templates. The subsequent execution of these intents relies on predetermined rule sets for selecting and performing management tasks, which cannot adapt or evolve their behaviour in response to dynamic or unfamiliar network conditions. As a result, these mechanisms cannot meet the autonomy and adaptability requirements of future 6G network management.To address these limitations, 6G network management introduces Autonomous Agents, which is not a static workflow-based function but a continuously operating intelligent entity capable of perception, reasoning & planning, action execution, and iterative learning. It can dynamically adapt to environmental changes, generalize to unfamiliar situations, and continuously refine its behavior based on feedback. A typical Autonomous Agent follows a five-step loop:

- Goal acquisition: Receives and interprets the consumer’s requirements

- Perception: Collects real‑time management data, discovers available management capabilities, and interprets historical and contextual information.

- Deep reasoning and planning: Decomposes objectives, evaluates alternative strategies, and formulates an optimal plan.

- Action execution: Utilizes available management capability to perform actions.

- Learning and optimization: Observes execution outcomes, updates internal knowledge, and improves future strategies.

Through this adaptive process, Autonomous Agents introduce native autonomous decision‑making into the 6G management system. By continuously perceiving, reasoning, acting, and learning, they can sustain service performance and resource efficiency under highly dynamic, heterogeneous, and previously unseen conditions, and supports a shift from reactive to proactive network operation. With these capabilities, Autonomous Agents become a core enabler for scalable and sustainable 6G OAM and mark a key step toward realizing AI‑native, self‑evolving network intelligence.

6.1.x.1.2 Potential Requirements

REQ-Agent-1: The 3GPP management system shall support autonomous agent to enable autonomous network management and operation.

REQ-Agent-2: The autonomous agent within the 3GPP management system shall support the capabilities of 	goal acquisition, perception, reasoning & planning, action execution, and iterative learning.

* * * End of Changes * * * *