3GPP TSG-SA5 Meeting #165	S5-260156

Goa, India, 09 - 13 February 2026

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 32.801-01 Add New Management Scenario on Data Management Framework Empowers Intelligent Evolution of Network Management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add New Management Scenario on Data Management Framework Empowers Intelligent Evolution of Network Management.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1 New 6G management scenarios

Editor's note 1: This clause will contain new 6G management scenarios and the corresponding requirements.

6.1.y Data Management Framework

6.1.y.1 Management Scenario #1: Data Management Framework Empowers Intelligent Evolution of Network Management

6.1.y.1.1 Description

With the evolution of 6G mobile networks toward the deep integration of communications, sensing, and intelligence, operators are required to manage a highly distributed and complex network. This environment spans cloud‑native core network functions, disaggregated RAN components, multiple layers of edge computing, and a wide range of network slices. These domains continually generate large volumes of heterogeneous management data with differing semantics, granularity, and temporal characteristics.

As operators progress toward higher levels of autonomy in network operation and maintenance, management data becomes increasingly central to supporting intelligent network management capabilities. AI‑assisted network optimization, proactive analysis, and real‑time synchronization with network digital twins rely on timely, accurate, and semantically consistent data. However, existing data collection, storage, and exposure mechanisms are fragmented and insufficient to support the needs of AI‑driven and autonomous network management. Operators may encounter several limitations, including:

- Fragmented and inconsistent management data. Management data is collected from RAN, Core, edge, and user equipment using different data formats, schemas, and semantics. As a result, operators struggle to construct an accurate and coherent end‑to‑end view of network status, which increases operational complexity. Operators therefore require a unified data management approach that harmonizes data models and semantics across management data.

- Inability to meet diverse latency, granularity, and consumption requirements. Real‑time optimization may require millisecond‑level streaming, while long‑term analytics and model training rely on large volumes of historical data. Existing data control and reporting mechanisms are not always able to meet such diverse and evolving demands.

- Lack of unified data quality assurance. Management data used to support intelligent decision‑making in the management system lacks standardized quality evaluation, preventing operators from ensuring that automated and intelligent functions operate on reliable, consistent, and trustworthy data.

To address these limitations, a Data Management Framework (DMF) is needed as a key enabler for the intelligent evolution of network management in 6G. The DMF provides standardized mechanisms across the lifecycle of management data, including collection, processing, storage, and exposure. It ensures that producers supply harmonized, well‑described, and quality‑assured data, while consumers—including Intelligence and Automation Functions such as network digital twins—obtain validated and contextually enriched data that satisfies their functional requirements.

By ensuring consistent, timely, and high‑quality management data availability, the DMF enables the transition from reactive monitoring to proactive optimization and supports the realization of advanced levels of autonomy in 6G network management.

6.1.y.1.2 Potential Requirements

REQ-DMF-1: The 3GPP management system shall support the data management framework to empower intelligent and autonomous network operation and maintenance.

REQ-DMF-2: The 3GPP management system shall support the data management framework that accommodates diverse latency, granularity, and consumption demands associated with management data.

REQ-DMF-3: The 3GPP management system shall support data quality assurance for management data.

* * * End of Changes * * * *