3GPP TSG SA5 Meeting #165	S5-260234

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 V-CHF store charging information when V-CHF detected failure of charging session with H-CHF

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue2.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.a	Solution #1.a: V-CHF store charging information when V-CHF detected failure of charging session with H-CHF

The solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF. This solution only applicable for offline charging.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement.

In the case of V-CHF decides to continue the charging session between the V-CHF and CTF in offline charging scenario, the V-CHF stores Charging Data Request(s) or charging information from the NF consumer (CTF).

* * * End of Changes * * * *