3GPP TSG SA5 Meeting #165	S5-260238

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Define the trigger conditions for V-CHF

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.d	Solution #1.d: Define the trigger conditions for V-CHF

The solution addresses key issue #1 covering requirements REQ-3GPPCH-LBIC-3.

The Charging Data Requests are issued by the V-CHF towards the H-CHF when certain trigger conditions are met. The applicable triggers in the V-CHF for inter-CHF communication are outlined as follows:

- Receive Charging Data Request [Initial] from original NF consumer;

- Receive Charging Data Request [Update] from original NF consumer;

- Receive Charging Data Request [Termination] from original NF consumer.

The triggers for V‑CHF may not be included in the message of Charging Data Requests sent to the H‑CHF.

The triggers from original NF consumer (e.g., V-SMF) are included in the information elements ‘Related Triggers’ of the Charging Data Requests sent to the H‑CHF. The Charging Data Request message contents are shown below as an example:

NOTE:	Only relevant information elements are shown in table 5.1.4.d.1.

Table 5.1.4.d.1: Charging Data Request message contents in V-CHF

| Information Element | Converged Charging Category | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | OC | / |
| Triggers | OC | OC | This field identifies the event(s) triggering the request. |
| Related Triggers | OC | OC | For inter-CHF communication, this field identifies the triggers from original NF consumer. |
| Multiple Unit Usage | OC | OC | / |
| Rating Group | M | M | / |
| Requested Unit | OC | - | / |
| Used Unit Container | OC | OC | / |
| Related Triggers | OC | Oc | This field holds reason for charging information reporting or closing for the used unit container. For inter-CHF communication, this field holds reason for charging information reporting or closing for the used unit container from original NF consumer.. |
| Related Trigger Timestamp | OC | OC | This field holds the timestamp of the trigger. For inter-CHF communication, this field holds the timestamp of the trigger from original NF consumer. |

* * * End of Changes * * * *