3GPP TSG SA5 Meeting #165	S5-260297

Goa, India, 9-13 February 2026

Source:	Samsung

Title:	pCR on Use case of Semantic Network Management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	V0.0.0

Work Item:	FS_6G_OAM

Comments

This pCR provides the UC for semantic network management

Proposed Changes

* * * First Change * * * *

## 6.1 New 6G management scenarios

### 6.1.x Semantic Network Management

#### 6.1.x.1 Management Scenario #<1>: Knowledge Representation and Management

##### 6.1.x.1.1 Description

The existing network management techniques are syntactic in nature where management functions collets management data (performance, fault, trace, MDT) and take management decision based on the data. Knowledge is the structured data that provide analytical insights to be used for efficient network management. It is desirable for the operators to not just base the network management on data but also on the knowledge that can be extracted from the data.

Performance and fault prediction Knowledge: Knowledge can be as simple as prediction of the value for a particular performance measurement and KPI at a future point of time. Network can be better managed based on the prediction knowledge. Similarly, the fault prediction knowledge can predict a network failure/fault with a particular severity of a future point of time. Resilient networks can be enabled utilizing fault prediction Knowledge. MDAS which can provide analytical reports containing similar information (i.e performance degradation prediction). However, they put additional burden of implementation in the OAM domain. The semantic knowledge can be created, maintained at the node level only enabling AI native networks.

Management Function conflict Knowledge: Multiple management and automation functionality can sometime produce conflicting configuration changes. The collected/produced data can be used to extract MnF conflict Knowledge that can be used to better manage multiple MnFs running in the network.

The following figure shows the multiple deployment of the Semantic Network Management system.

- Management Data Generator: This entity refers to any 5GS managed function as defined in 3GPP TS 28.541. This entity generates management data as specified in 3GPP TS 28.552.

- SNM MnS Producer: This entity is responsible to generate Knowledge based on the management data received from the management data generator. This entity is also responsible to maintain the Knowledge Base containing the historical knowledge.

- SNM MnS Consumer: This entity is responsible for taking network management decision based on the available knowledge. The role of this entity can be played by existing management functions e.g CCL. A CCL can utilize available Knowledge to take configuration decision as per the execution step of CCL stages.

| SNM Deployment - A | SNM Deployment - B |
| --- | --- |

##### 6.1.y.1.2 Potential Requirements

REQ-SNM-KR-1: The 3GPP management system shall support the capability for an authorized consumer to request for the available Knowledge.

REQ-SNM-KR-1: The 3GPP management system shall support the capability to provide available Knowledge to the authorized consumers.

#### 6.1.y.1 Management Scenario #<1>: Knowledge Graph Representation and Management

##### 6.1.y.1.1 Description

With evolving NEs, the management data has become huge and insignificant to convey meaningful information over time. In a network traffic monitoring system of a telecom service provider, multiple nodes transfer requested management data periodically creating huge amount of data being dumped in the management system to process and reason with. For a network having multiple NEs, this creates a big traffic load in network, between NE and EMS/NMS and between EMS/NMS and OSS. The time taken by EMS/NMS/OSS in processing and storing this large-scale data is also of big concern for many network equipment and service providers. Analytics solutions (e.g NWDAF, MDAS) which sits in the top of the hierarchy deal with this humongous data and typically finds it difficult to analyse and get insights in real time due to lots of redundant and insignificant data which need to be cleaned. The AI-ML algorithms struggle to remove the noise and to create an accurate model with this level of data. All this delivered data is provided in isolation without providing details on the dependencies and correlation with other management data. The management data can very well have dependencies with other management data.

For example, whenever AMF NG Handover Attempt failure KPI increases, all the UEs served by AMF will re-register with the target AMF increasing the UE Initial Registration Attempt KPI. AMF Selection Relocation attempts KPI from gNB increases indicating shift from source AMF to target AMF. In a typical monitoring system all the above performance data will be sent to the management system for resolution. However, in an intelligent Knowledge based deployment only the AMF NG Handover Attempt failure can be highlighted for resolution. Once resolved all other performance degradation should be resolved automatically

Figure x: PM Dependency depiction

Another example of dependencies among management data is depicted below: When the Physical Resource Block (PRB) utilization becomes very high in a cell, the failure of handovers to this target cell from a source cell increases as indicated by cause ‘No radio resources available in target cell’. This leads to increase in number of abnormal releases of active DRBs and active QoS flows in source cell. This results in poor end user experiences like enhanced call drops in source cell. Furthermore, very high PRB utilization in target cell could lead to higher energy consumption in that cell resulting in degradation of energy efficiency in cell. A high PRB utilization would also lead to decrease in user throughput especially at the cell edge, resulting in a poor QoE for users. A high PRB utilization in target cell would also imply an increase in number of MLB handovers from this (target) cell to another neighbor cell which is necessary to reduce its load.

Figure y: PM Dependency depiction

It is beneficial for the management system to maintain the data dependency Knowledge then can be either queried by the authorized entities or utilized within the management system to better manage the network

##### 6.1.y.1.2 Potential Requirements

REQ-SNM-KR-1: The 3GPP management system shall support the capability for an authorized consumer to request for the available Knowledge Graphs.

REQ-SNM-KG-1: The 3GPP management system shall support the capability to provide available Knowledge Graphs to the authorized consumers.

* * * Next Change * * * *

* * * End of Changes * * * *