3GPP TSG SA5 Meeting #165	S5-260417

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0258 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel20 CR TS 28.531 Creation of a 3GPP NF on a generic orchestration and management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NF_Deployment_LCM-OAM |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Currently clause 5.1.17 of TS 28.531 defines the creation of a 3GPP NF when the orchestration and management system is ETSI-NFV MANO. This CR adds another clause to enable the creation of a 3GPP NF on a generic orchestration and management system not limited to only ETSI-NFV MANO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add definitions of NF Deployment and NF Deployment instance - Add a new clause to enable the creation of a 3GPP NF on a generic orchestration and management entity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification leads the industry to perceive the 3GPP management system as being able to interact exclusively with ETSI NFV-MANO for the creation of a 3GPP NF, and not with other O&M systems commonly used in telco environments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 28.530 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 28.530 [4].

IAB-node: as defined in TS 38.401 [18].

NF Deployment: a software deployment used to realize a part of, or the full function of one or more 3GPP NFs. The NF Deployment is not restricted to any specific virtualization technology (e.g. VM-based or container based).

NF Deployment instance: is a running instance of an NF Deployment.

NF Deployment package: collection of artefacts required to deploy, configure, and lifecycle manage an NF Deployment.

NF Deployment descriptor: defines the behavior (e.g., desired configuration, operational parameters and lifecycle state) of an NF Deployment.

==============Next change==============

### 5.1.17	Creation of a 3GPP NF

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of an instance of 3GPP NF. |  |
| Actors and Roles | An authorized consumer of the NF creation provisioning management service. |  |
| Telecom resources | VNF package(s) of the virtualized part of 3GPP NF; ETSI NFV MANO system; NF provisioning management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The VNF package(s) of the virtualized part of 3GPP NF have been on-boarded to ETSI NFV MANO system. |  |
| Begins when | The authorized consumer needs to create a new instance of 3GPP NF. |  |
| Step 1 (M) | The authorized consumer requests the NF provisioning management service producer to create a new instance of 3GPP NF. |  |
| Step 2 (M) | The NF provisioning management service producer checks the subject 3GPP NF contains virtualized part and/or non-virtualized part. If it contains virtualized part, then the NF instantiation service producer performs the step 3 and 4 to instantiate the virtualized part of the subject 3GPP NF. How to instantiate the non-virtualized part of the subject 3GPP NF is out of scope of present specification. |  |
| Step 3 (M) | The NF provisioning management service producer interacts, or requests another NF provisioning management service producer to interact, with ETSI NFV MANO system to instantiate the VNF(s) that are realizing the virtualized part of subject 3GPP NF. |  |
| Step 4 (M) | If all of the contained parts (i.e., virtualized part and non-virtualized part if any) of the 3GPP NF have been successfully instantiated, the NF provisioning management service producer informs the consumer(s) (who have subscribed to the notifications for NF creation) that the instance of 3GPP NF has been created, and creates the MOI(s) for the subject 3GPP NF. |  |
| Step 5 (M) | Created MOI(s) may be maintained by a Management Function which has the NF provisioning management service or the 3GPP NF. When the MOI(s) is maintained by the 3GPP NF, the NF provisioning management service producer sends a request of creating the MOI(s) to the corresponding NF provisioning management service producers in the created NF. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The instance of 3GPP NF has been created, and the MOI(s) of the 3GPP NF have been created. |  |
| Traceability | REQ-PRO_NF-FUN-1, REQ-PRO_NF-FUN-2, REQ-PRO_NF-FUN-7 |  |

### 5.1.17a Creation of a 3GPP NF on a generic orchestration and management system

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of an instance of 3GPP NF. |  |
| Actors and Roles | An authorized consumer of the NF creation provisioning management service. |  |
| Telecom resources | NF Deployment package(s) of the virtualized part of 3GPP NF; Generic orchestration and management system; NF provisioning management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The NF Deployment package(s) of the virtualized part of 3GPP NF have been on-boarded to the generic orchestration and management system. |  |
| Begins when | The authorized consumer needs to create a new instance of 3GPP NF. |  |
| Step 1 (M) | The authorized consumer requests the NF provisioning management service producer to create a new instance of 3GPP NF. |  |
| Step 2 (M) | The NF provisioning management service producer checks the subject 3GPP NF contains virtualized part and/or non-virtualized part. If it contains virtualized part, then the NF instantiation service producer performs the step 3 and 4 to instantiate the virtualized part of the subject 3GPP NF. How to instantiate the non-virtualized part of the subject 3GPP NF is out of scope of present specification. |  |
| Step 3 (M) | The NF provisioning management service producer interacts or requests another NF provisioning management service producer to interact, with the generic orchestration and management system over the deployment management reference point to instantiate the NF Deployment(s) that are realizing the virtualized part of subject 3GPP NF. |  |
| Step 4 (M) | If all of the contained parts (i.e., virtualized part and non-virtualized part if any) of the 3GPP NF have been successfully instantiated, the NF provisioning management service producer informs the consumer(s) (who have subscribed to the notifications for NF creation) that the instance of 3GPP NF has been created, and creates the MOI(s) for the subject 3GPP NF. |  |
| Step 5 (M) | Created MOI(s) may be maintained by a Management Function which has the NF provisioning management service or the 3GPP NF. When the MOI(s) is maintained by the 3GPP NF, the NF provisioning management service producer sends a request of creating the MOI(s) to the corresponding NF provisioning management service producers in the created NF. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The instance of 3GPP NF has been created, and the MOI(s) of the 3GPP NF have been created. |  |
| Traceability | REQ-PRO_NF-FUN-1, REQ-PRO_NF-FUN-2, REQ-PRO_NF-FUN-7 |  |

==============End of change==============