3GPP TSG-SA5 Meeting #165	S5-260457

Goa, India, 9 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.314 | CR | 0007 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | - x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.314 Add use case and requirements for WAB-node connecting to management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2026-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A mobile NR node can implement different network features, including IAB and WAB. When IAB, it becomes an IAB-node. When WAB, it becomes a WAB-node. The management of a mobile NR node includes the node connectivity to management system (specified in TS 28.314/28.315) and node configuration (specified in TS 28.531). In TS 28.314, the mobile NR node connecting to management system only covers IAB feature; WAB feature is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add WAB-node and mobile NR node definitions. Add use case description and requirements for WAB-node connecting to management system, so both IAB and WAB features are covered. Update the location information for IAB-node, adding the geographical coordination as potential option, to comply with the solution in TS 28.541. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case on node-OAM connectivity for WAB feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 6.1.2.1, 6.1.2.2, 6.1.2.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.315 CR XXXX |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Plug and Connect: The procedure by which a NE gets basic connectivity information after it is powered up and gets connected to its management system.

Software and Configuration Server (SCS): A server that provides software and configuration functions for each connected network element.

IAB-node: as defined in TS 38.401 [5].

WAB-node: as defined in TS 38.401 [5].

Mobile NR node: it represents either an IAB-node or a WAB-node.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Certification Authority

CMP	Certificate Management Protocol

DHCP	Dynamic Host Configuration Protocol

DNS	Domain Name System

FQDN	Fully Qualified Domain Name

NAT	Network Address Translation

NE	Network Element

PnC	Plug and Connect

RA	Registration Authority

SCS	Software and Configuration Server

SeGW	Security Gateway

VLAN	Virtual LAN

IAB	Integrated Access and Backhaul

WAB		Wireless Access Backhaul

Next Change

### 6.1.2	Use case mobile NR node connects to management system

#### 6.1.2.1	Overview

IAB-node is a mobile NR node implementing IAB feature. IAB architecture is specified in 3GPP TS 38.401[5]. The IAB-node connects to an upstream IAB-node or an IAB-donor-DU via a subset of the UE functionalities of the NR Uu interface (named IAB-MT function of IAB-node). IAB-node differs from static NE in term of its wireless connection with IAB-donor-node and its mobility behaviours. IAB-node can move to different geographic areas or traverse to different PLMNs that require to connect to different management system based on its location.

WAB-node is a mobile NR node implementing WAB feature. WAB architecture is specified in clause 6.1.7 of 3GPP TS 38.401[18]. A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [3] and serves UEs by means of a terrestrial NR Uu radio link. The WAB-MT provides PDU sessions for backhauling all WAB-gNB traffic, including Xn, NG and OAM traffic. The WAB-node differs from static NE in terms of its wireless connection with two 5GC instances (i.e., an 5GC instance, which serves WAB-gNB, and another 5GC instance, which serves WAB-MT via a gNB) and its mobility behaviours. WAB-node can move to different geographic areas or traverse to different PLMNs that require to connect to different management system based on its location.

Mobile NR node connects to management system at power up and during mobility by following PnC steps as described in clause 6.1.1. At step of IP autoconfiguration, mobile NR node differs from static NE in term of obtaining IP configuration for OAM connectivity. When mobile NR node powers up, mobile NR node may get the initial IP configuration via PDU session that is set up between IAB-MT and 5GC (when IAB-node) or between WAB-MT and 5GC (if WAB-node). Using this connectivity,  mobile NR node can obtain the initial IP configuration provided by IP autoconfiguration service (e.g. DHCP server), and additionally may perform DNS query depends on the operator's deployment scenario. The management system that mobile NR node connects to may change due to mobile NR node mobility behaviours. To support mobile NR node connecting to a management system based on its location, mobile NR node should obtain IP configuration for OAM connectivity that is associated to its location via notification operations as described in clause 6.1.2.2. With the IP configuration obtained, mobile NR node performs the subsequent PnC procedures, satisfies security aspects including CA/RA enrolment and connectivity to SeGW before connecting to SCS.

#### 6.1.2.2	Obtain IP configuration for OAM connectivity via notifications

3GPP management system should support a mobile NR node to use notification mechanism to obtain IP configuration for OAM connectivity. When notification mechanism is used, 3GPP management system shall support the mapping of IP configuration for OAM connectivity with mobile NR node location, see clause 6.1.2.3.

When mobile NR node powers up and after connecting to 3GPP management system for the first time, the MnS producer for mobile NR node shall provide location information with mobile NR node indication via notifications to 3GPP management system as specified in 3GPP TS 28.532 [6] (e.g. notifyMOIChanges) to indicate its current location. Upon the reception of notifications, 3GPP management system shall provision the IP configuration for OAM connectivity (e.g. via createMOI operation) as specified 3GPP TS 28.532 [6] based on its current location.

When mobile NR node moves to a new location which may need connects to new management system, the provisioning MnS producer of mobile NR node shall provide new location information via "notifyMOIChanges" or "notifyMOIAttributeValueChanges" or other notifications as specified in 3GPP TS 28.532 [6] to 3GPP management system. Upon the reception of notifications of new location, 3GPP management system shall provision the new IP configuration for OAM connectivity via "modifyMOIAttributes" or other operations as specified in 3GPP TS 28.532 [6] to the MnS producer for mobile NR njode based on its new location.

#### 6.1.2.3	IP configuration for OAM connectivity with location information

The IP configuration for OAM connectivity should include one or more the following depending on the deployment scenarios:

-	Basic IP configuration information (e.g. IP address, netmask, default gateway, domain name, IP address(es) of DNS servers, time servers)

-	FQDN or IP address of the initial OAM SeGW

-	FQDN or IP address of the CA/RA servers. If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s).

-	FQDN or IP address of the initial SCS. If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s)

For IAB-node, the location information should include one or more of the following depending on IAB-node physical location or the mobility scenarios as specified in 3GPP TS 38.401 [5] clause 8.12 and clause 8.23:

-	The geographical coordinates of the IAB-node.

-	TAC or TAI pertaining to the cells where IAB-MT is connected.

-	Serving cell ID that IAB-MT is connected to.

-	The gNBId of the IAB-donor-CU and PLMN info that target IAB-DU connects to.

-	The gNBId of the IAB-donor-CU and PLMN info that serves the IAB-MT.

For WAB-node, the location information should include one or more of the following depending on WAB-node physical location or the mobility scenarios as specified in 3GPP TS 38.401 [18] clause 12.7:

-	The geographical coordinates for the WAB-node.

-	TAC or TAI pertaining to the cells where WAB-MT is connected.

- 	Serving cell ID that WAB-MT is connected to.

- 	The gNBId and PLMN information that serves the WAB-MT.

Next Change

### 6.2.2	Specification Requirements for mobile NR node connecting to management system

REQ-MOBILE_NR_NODE_CON-1 1: 3GPP management system should have the capability to support IP configuration for connecting a mobile NR node to the management system based on the node’slocation.

REQ-MOBILE_NR_NODE_CON-2: 3GPP management system should have the capability to receive notification about the location of mobile NR node.

REQ-MOBILE_NR_NODE_CON-33GPP management system should have the capability to enable mobile NR node to fulfill the security aspects of PnC procedures to connect to management system at power up and during mobility.

End of Changes