3GPP TSG-SA5 Meeting #165	S5-260470

GOA, India, 9th Feb 2026 - 13th Feb 2026

Source:	Ericsson Hungary

Title:	pCR TR 28.884 SW Management solution

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	V0.2.0

Work Item:	FS_SBMA_Ph4

Comments

In v0.2.0 terminology, some use-case examples and requirements were agreed for SW management. Here we provide a proposal for a potential solution.

Proposed Changes

* * * First Change * * * *

### 5.4.1	Description

3GPP TS 32.531 [11], TS 32.532 [12], and TS 32.533 [13] define the concepts, requirements, Information Service, and CORBA solution set for the software management of NEs for 4G, encompassing both automated and non-automated software management approaches.

Software management for 5G enhances 5G network operational efficiency. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [2] does not include support for software management. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs and NFs.

Editor's note:	To enable software management within SBMA, the associated potential requirements and potential solutions are FFS, building upon the legacy of software management of NEs for LTE while adapting to the principles of SBMA.

Based on 4G definitions in [11], when describing SW management in this document we use the following terminology:

Download – move the SW from a remote server to the producer.

Verify – verify the downloaded SW's integrity and that it is fit for its purpose.

Activation: Activation of software makes it ready to be used and the software starts providing service

Installation: Installation of software puts it into a form suitable for activation or use.  (Activation may include steps like unpacking, unzipping, checks, but does not include starting to provide service.)

Fallback: The process of activating a previous SW version.  (Fallback may involve moving to a previous configuration content of NRM).

Cancellation: Is the process where a consumer stops an ongoing operation (e.g. download, activation). If the process is stopped by the producer, it is considered a failure of the process not a cancellation.

Deactivate: Remove a SW from providing service.

Delete: delete SW from the producer

Use cases for PNF SW management

The following are examples of possible/typical use-cases. (They are presented here as informative examples)

- UC1: Download software on MnS producer: In preparation for a software upgrade, relevant software artefacts are downloaded by suitable means to the MnS producer. After download, other preparatory steps (a.k.a. software installation) may be performed (such as unzipping, integrity validation) to make sure the software artefacts are suitable and ready for subsequent activation. This download/installation activity is typically not traffic-impacting and may happen sometime before the software is activated. The activity might be triggered by an MnS consumer, or might be triggered by some other event (perhaps the installation of new hardware on the PNF, requiring a software upgrade). The download/installation is typically long-running, and its progress can be externally observed. At the end of this use case, the software is ready to be activated. In exceptional error situations (such as unsuitable software being installed; checksum errors), activation of this software will not be possible.

- UC2: Activate software: Previously downloaded/installed software is activated. The activation of the software brings the software into service, effectively replacing the previously running software. As part of the software activation, the PNF’s configuration may be automatically updated. Software activation may or may not be traffic-impacting, depending on the particulars of the PNF implementation. The activation activity is usually triggered by an MnS consumer; but may be triggered by an event or a scheduler. The software activation is typically long-running, and its progress can be externally observed. At the end of this use case, the activated software has been brought into service. In exceptional error situations, the software activation procedure is aborted, the previous software brought (back) into service, and any automated updates to the PNFs configuration undone.

- UC3: Cancellation: The software installation and activation are typically long-running operations. In some scenarios (e.g., ongoing emergency, slow/faulty O&M network, a mistake having been made), the ongoing operation is cancelled. A cancellation is typically triggered by an MnS consumer. After the cancellation, the MnS producer will be in the same state as it was before the operation started. It is possible to re-start the same operation.

- UC4: Fallback to previous software: After the PNF’s software has been successfully upgraded, after observing the system's behaviour for some time, a decision may be made that the upgraded software is unsatisfactory (perhaps it is functionally or performance-wise inferior to the previously installed software). In this case, a software fallback may be triggered. The software fallback will re-activate previously activated software and effectively repeat the steps of the software activation (see UC2) for the previously activated software.

- UC5: Remove inactive software (house cleaning): Software residing on the producer may become unneeded. Unneeded and in-active software is removed from the producer. This operation is typically triggered by the MnS consumer. At the end of this use case the in-active software has been removed from the PNF and therefore is not available anymore for fallback or activation. One typical  case when software becomes unneeded is when a PNF’s software is successfully activated, and found to be operating satisfactorily (perhaps after having been observed for a while), other software still present on the PNF, but not in service, may be removed in order to free resources on the PNF.

Implementation considerations for further study

The producer should check the SW intended to be activated. Whether these checks are executed immediately after download, in a separate step, or at the beginning of activation is implementation dependent.

The producer may take additional preparatory steps before activation (e.g. unpacking the SW). Whether these are executed immediately after download, in a separate installation step or at the beginning of activation is implementation dependent.

Open issues for further study:

-Whether fallback shall be initiated by the consumer or by the producer or potentially both

- Whether activation shall be triggered explicitly by the consumer or automatically in the producer (e.g. by a scheduler) or both

Two modes shall be considered:

- a single step activation that includes download, any checks and activation. Activation may be immediate or  delayed, triggered by a scheduler or some other condition.

- a multi-step activation, where after download, activation is executed as a separate step within the producer. The separate activation step shall be initiated by a consumer operation.

* * * Next Change * * * *

### 5.4.3	Potential solutions

### 5.4.3.1	Potential solution for PNF

##### 5.4.3.1.1		Introduction

It is the goal to define a flexible solution, that can accommodate different vendor practices.

It shall be possible to handle upgrades that consist of a single overall SW package but also upgrades that use a set of separate SW packages in a single activation. It shall be possible to handle systems where the full functionality of the system has to be upgraded at the same time (using one or more SW packages) but also systems that allow upgrading different parts of the system separately at different times.

It is assumed that the consumer knows a priori which SW packages can be installed/activated on which ManagedElements, and whether there are dependencies between SW packages.

A solution is proposed that follows SBMA principles. IOCs are defined in the NRM to facilitate SWM. The solution takes ideas and some solution elements from TS 32.532.

The SWManagement IOC is a container for the other IOCs. It has not attributes.

A SW information IOC UpgradeItem is defined to allow the consumer to list SW packages available on the PNF including the state of that SW item.

A job IOC SWManagementJob is defined for executing the procedures and to store the results of the procedure.

SWMCapabilities IOC is defined that allows the consumer to discover the exact SWM capabilities of the producer. As different implementation may support different steps in SWM, the consumer shall be able to discover which of the  standard  steps does an individual implementation support.

Based on the requirements and use-cases above the following SWM steps are specified:

- Download

- Verify

- Install

- Activate –  may imply a full or partial restart

- Deactivate – step is only applicable if partial upgrade is supported, as it is not allowed to remove all SW from the activated state

- Delete

Fallback should be implemented as a normal activate of a previously used UpgradeItem, thus it is not a separate step.

Any step can be canceled. Cancelling a step will take the job back to the state where the step started.

Only steps exposed by the implementation may be explicitly ordered to be executed. Not all implementations expose each of the above steps. Steps may not be exposed because

- It is not implemented. E.g. deactivate is only implemented if partial upgrades are supported

- The functionality is implemented but is executed as part of another step. E.g. “verify” is not exposed as a separate step, but when install is executed, the SW is automatically verified first.

In a minimal system only the activate step is mandatory to be exposed. In this hypothetical minimum case activate would include download, verify and install while also deleting old SW. It is assumed that most systems will expose multiple steps. Vendors may implement additional steps.

Steps can only be executed in a specific order, see figure below. If a step is not implemented, it should be considered automatically executed, immediately when the preceding state is reached.

Each exposed step has to be explicitly initiated by the consumer.

Health checks or verifications can happen at any time. They are implementation specific. They can be defined as a separate implementation specific step and/or they can be implemented as a part of, or consequence of any of the exposed steps.

##### 5.4.3.1.2		UpgradeItem

Represent a SW package that can be upgraded. Includes SW in any state: remote, downloaded, activated etc. Each MOI can be subject to a SWM step.

A specific implementation may have a single or multiple UpgradeItems to cover a full upgrade of a system. Multiple MOIs may represent the multiple versions of the same upgrade item. If multiple separate upgrade items are needed to cover a full upgrade of a system the activation may be implemented as a single step handling all needed upgrade items together or it may allow upgrading different parts in separate activate steps.

How many and which upgrade items are needed for the upgrading a ManagedElement is out of scope. It shall be known by the consumer by means outside this specification.

The UpgradeItem is created and deleted by the consumer, its initial state is "remote". Indicating that it is not yet available on the producer.

Attributes:

- Id

- Name

- number

- version

- userLabel

- description

- filename URI

- createdAt

- downloadedAt

- state: remote, downloaded, verified, installed, activated

- installationTime

- activationTime

- additionalInformation,

##### 5.4.3.1.3		SWManagementJob

It represents a SW upgrade job to be handled. The different steps of the process can be invoked separately or at the same time, but only in the allowed order. A job may handle one or multiple UpgradeItems.

The install, activate, deactivate steps are each atomic,

Attributes:

Steps to execute (only steps in capabilities are accepted)

List of UpgradeItems

Parameter per individual steps:

VendorParameters

State of the step: not-requested,  requested, executing, succeeded, failed, partly-failed

Detailed progressinfo (ProcessMonitor) or just an ENUM + list of strings

OverallResult: ENUM

Detailed result per UpgradeItem: Basic ENUM + list of strings

##### 5.4.3.1.4		SWMCapabilities

The IOC shall indicate which of the 3GPP specified steps (see clause 5.4.3.1.1) the producer exposes (and thus can be invoked explicitly) and whether any additional vendor specific steps are supported.

The IOC provides a read-only ordered set of steps. Each step can be invoked by setting the appropriate data in the SWManagementJob.  The SWMCapabilities IOC is created by the producer and is strictly read-only.

Attributes

Exposed steps

Name (extensible enumeration)

additionalProperties: nameValuePair – this allows both for future standard and for vendor extensions

### 5.4.4	Evaluation of potential solutions

### One potential solution has been identified for SW management for PNFs. The solution is feasible.

### No solution has been identified for virtualized systems.

* * * Next Change * * * *

## 6.x	Use case#4 on software management for 5G

The requirements for the SW management for PNFs are specified in clause 5.4.2. A potential solution for PNFs is described in clause 5.4.3.1.

The requirement and the solution for PNFs are recommended for normative work.

The requirements and solution for virtualized systems is missing, so at the moment no normative work is recommended for virtualized systems.

* * * Next Change * * * *

### Annex X. PlantUml

@startuml SW Management fragment

skinparam monochrome true

skinparam ClassStereotypeFontStyle normal

class ManagedElement <<InformationObjectClass>>

class SWManagement <<InformationObjectClass>>

class UpgradeItem <<InformationObjectClass>>

class SWManagementJob <<InformationObjectClass>>

class SWMCapabilities <<InformationObjectClass>>

hide empty members

hide circle

ManagedElement “1” *-d- “1” SWManagement : <<names>>

SWManagement “1” *-d- “*” UpgradeItem : <<names>>

SWManagement “1” *-d- “*” SWManagementJob : <<names>>

SWManagementJob “1” --l--> “*” UpgradeItem

SWManagement “1” *-d- “*” SWMCapabilities : <<names>>

@enduml

@startuml

title UpgradeItem state diagram

[*] -d-> Remote : CreateMOI

Remote -d----> [*] : DeleteMOI

Remote -r---> Downloaded : Download

Downloaded --right--> Verified : Verify

Verified -d-> Installed : Install

Installed -d-> Activated : Activate

Activated --> Installed : Replace UpgradeItem \nanother with item\t\t\t

Activated --> Installed : Deactivate

Activated --> Installed : Fallback

Downloaded -l----> Remote : Delete

Verified --> Remote : Delete

Installed --> Remote : Delete

@enduml

* * * End of Changes * * * *