3GPP TSG-SA5 Meeting #162	S5-254036d3

Goteborg, Sweden, 25 - 29 August 2025

Source:	Ericsson

Title:	Pseudo-CR on TR 28.881 Add issue, requirements and potential solution for intent traceability

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

An intent consumer (owner) submits an intent to a single intent management function.  In some cases, in order to fulfil the intent an intent handling function may need to further submit intent(s) to other intent handling functions for fulfilment.  Such handling can occur multiple times across intent handling functions across multiple management and/or domain layers.

For example, TS 28.312 clause 4.2.3 shows an intent MnS producer in communication services layer may submit an intent towards an Intent MnS producer in the communication services provider (CSP) layer.  To fulfil the original intent the CSP producer creates additional intents which are then submitted to intent handling functions in the network operator (NOP) and network equipment provider (NEP) domains.

There is currently no specified method to identify that such handling has occurred and is an impediment to traceability of the intent fulfilment.  This submission proposes a method to enhance the IDMS intent model to contain such information.

This contribution proposes toadd a new issue for intent traceability.

This pCR is related to WT-7.

# 4	Detailed proposal

1st Change

- 4.X Issue#X: Intent traceability

- 4.X.1 Description

An intent consumer (owner) submits an intent to a single intent handling function. In some cases, in order to fulfil the intent an intent handling function may need to submit additional intent(s) to other intent handling functions.  Such handling can occur multiple times across intent handling functions across multiple management and/or domain layers.

There is however no identified method, standardized or otherwise, which allows for traceability as an intent is propagated to various intent handling functions.

Since intents can be propagated to multiple intent handling functions, likely with different implementations, it is insufficient to rely on external mechanisms such as logging or local network management audit tools to trace the intent.  The information identifying each intent handling function which has handled the intent must be propagated along with the intent itself and accessible/meaningful within the content of each intent handling function.

The following figure provides an overview of such information and its handling:

Figure 5.X.1-1: Intent traceability information handling

- 4.X.2 Potential requirements

REQ-Intent_Trace-1: The intent driven MnS producer should provide information in the intent to identify that an intent has been handled by a particular intent handling function.

REQ-Intent_Trace-2: The intent driven MnS producer should provide information (as defined in REQ-Intent_Trace_1) to identify any subsequent intents created by it as part of fulfilment.

REQ-Intent_Trace-3: The intent driven MnS producer should propagate information (as defined in REQ-Intent_Trace_1) identifying the intent handling functions to subsequent intent(s) to allow traceability of the intent across multiple intent handling functions.

End of changes