3GPP TSG-SA5 Meeting #163	S5-254655

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	Rel-20 pCR TR 28.881 Add Use case on intent expectation satisfaction information

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

Existing expectationTarget which is based on average values may not sufficiently capture the distribution of actual experience, as averages may mask variations across the expectationObject. By providing the intent expectation satisfaction information of the whole intentExpectation in the intent report, the IDMS consumer can better capture the proportion of relevant entities (e.g., UEs, cells, packets, etc) meeting the target value, enabling network operator a better understanding on the result of intent fulfilment or intent fulfilment feasibility check.

This contribution related to WT-1.1 in SP-250496.

Proposed Changes

* * * First Change * * * *

4.X Use case #X:  intent expectation satisfied information

4.X.1	Description

In 3GPP TS 28.312 [x], the existing intentReport allows the IDMS consumer to obtain the values of KPI as indicated by the corresponding expectation targets. However, this information may not accurately reflect the actual situation. For example, considering an intent that includes a radio network expectation with the target aveDLRANUEthptTarget, the existing intent report provides the average value of the downlink RAN UE throughput and tells whether the intent expectation is fulfilled or not based on the average value. However, in the case where the average value is impacted by some extreme values, the fulfilled intent report will miss the information that the majority of UE experience doesn’t fulfil the UE throughput specified in the target. Such information matters because it helps the network operator to have a better understanding on the intent expectation satisfaction. The report can include the information besides the average information,

which can be entity based or time based, and is derived through statistical measurement or calculation, such as distributions, range or std deviation.

4.X.2	Potential requirements

REQ-Intent_IESI-1: The intent driven MnS producer should have a capability providing information besides average value in the intent report.

4.X.3	Potential solutions

* * * End of Changes * * * *