3GPP TSG-SA5 Meeting #163	S5-254707d2

Wuhan, China, 13 - 17 October 2025											Revision of S5-254557

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0053 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | - x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.540 Add requirements for WAB-gNB management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MWAB IOC solution has attributes (e.g. “currentLocation”, “allowedTime”, “allowedArea”) which do not have a counterpart requirement. For the sake of alignment between stages 1 and 2, it is proposed to add missing requirements. Additionally, the MWAB feature developed in SA2 and RAN3 pressumed the OAM involvement for a number of Rel-19 functionalities, which are documented in TS 23.501 and TS 38.401 v19.0.0, respectively. This involvement means the definition of new requirements on 3GPP management system. These requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing requirements for WAB-gNB management in TS 28.540: - One requirement for supporting the definition of conditions triggering activation/deactivation of MWAB operation (TS 23.501, 5.49.3.3). Currently, there is a solution (allowedArea, allowedTime attributes) in TS 28.541 fulfilling this, but there is no OAM requirement associated in TS 28.540. - One requirement for supporting receiving location information from WAB node. Currently, there is a solution (currentLocation attributes) in TS 28.541 fulfilling this, but there is no OAM requirement associated. - Three requirements for supporting MWAB functionalities developed in RAN3 (TS 38.401) as documented in the following clauses from R3-256014: X.1, X.3, X.4, X.7.1, X.8. Align REQ-VMR-CON-001 with REQ_IAB_NRM_2, as both represent the same feature. The IP configuration on MWAB-gNB (IAB-node) is to enable connectivity with OAM for management purposes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 1 and 2. Lack of OAM support for some MWAB functionalities specified in SA2 and RAN3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.11	Requirements for management of WAB-gNB

REQ-VMR-CON-001: The 3GPP management system should support IP address configuration for WAB-gNB OAM connectivity.

REQ-VMR-CON-002: The 3GPP management system should support TAC/RANAC (re)configuration for WAB-gNB management.

REQ-VMR-CON-003: The 3GPP management system should support activating/deactivating the WAB-gNB operation.

REQ-VMR-CON-00a: The 3GPP management system should support configuring policies for service authorization of the WAB-gNB.

REQ_VMR_CON-00b: The 3GPP management system should support receiving location information from WAB-gNB.

REQ-VMR-CON-00c The 3GPP management system should ensure continuity of the OAM connectivity for the WAB-gNB, when the WAB-node location is updated.

End of Changes