3GPP TSG-SA5 Meeting #163	S5-254299

Wuhan, China, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 1060 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add informatin elements to support charing aspects of multi-modality XRM features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_PH2-CH |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A Multi-modal Service ID is introduced as an additional optional identifier enable application coordination and QoS management&monitoring of related service data flows that belong to the same multi-modal application. This may require the charging function to associate multiple service data flows across different rating groups and service identifiers to a single multi-modal session, to support unified monitoring and potential session-level billing in the future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CDR parameters of multi-modality service ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-modality service ID will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.2.24, 5.1.5.0, 5.1.5.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

5.1.2.2.24	List of Service Data

This list includes one or more service data containers. Depending on the reporting level of PCC/ADC rules, one service data container either includes charging data for one rating group or for one rating group and service id combination. Each service data container may include the following fields:

-	AF-Record-Information.

-	Charging Rule Base Name.

-	ADC Rule Base Name.

-	Data Volume Downlink.

-	Data Volume Uplink.

-	Event Based Charging Information.

-	Local Sequence Number.

-	PS Furnish Charging Information.

-	EPC Qos Information.

-	Rating Group.

-	Report Time.

-	Result Code.

-	Service Condition Change.

-	Service Identifier.

-	Multi-modal Service Identifier.

-	Service Specific Info.

-	Serving Node Address.

-	Time of First Usage.

-	Time of Last Usage.

-	Time Quota Mechanism.

-	Time Usage.

-	user location information.

-	3GPP2 User Location Information.

-	UWAN User Location Information.

-	TWAN User Location Information.

-	Sponsor Identity.

-	Application Service Provider Identity.

-	Presence Reporting Area Status.

-	List of Presence Reporting Area Information.

-	User CSG Information.

-	RAT Type.

-	Serving PLMN Rate Control.

-	APN Rate Control.

-	Related Change of Service Condition.

-	Traffic Steering Policy Identifier Downlink.

-	Traffic Steering Policy Identifier Uplink.

-	VoLTE Information.

-	Rating Group is the identifier of rating group. This field is mandatory. The parameter corresponds to the Charging Key as specified in TS 23.203 [203].

-	Charging Rule Base Name is the reference to group of PCC rules predefined at the PCEF. This field is included if any of the PCC rules, which usage is reported within this service data container, was activated by using the Charging Rule Base Name as specified in TS 29.212 [220]. In case multiple Charging Rule Base Names activate PCC rules, which usage is reported within this service data container, the P-GW/IPE-CDR shall include only one occurrence to the service data container.

-	ADC Rule Base Name is the reference to group of ADC rules predefined at the TDF. This field is included if any of the ADC rules, which usage is reported within this service data container, was activated by using the ADC Rule Base Name as specified in TS 29.212 [220]. In case multiple ADC Rule Base Names activate ADC rules, which usage is reported within this service data container, the TDF shall include only one occurrence to the service data container.

-	Result Code contains the result code after the interconnection with the OCS. This field may be added to the service data container if online and offline charging are both used for same rating group. The result code in service data container is the value of the Result-Code AVP received within last CCA message in corresponding MSCC AVP to this service data container.

-	Local Sequence Number is a service data container sequence number. It starts from 1 and is increased by 1 for each service date container generated within the lifetime of this IP-CAN bearer/TDF session.

-	Time of First Usage is the time stamp for the first IP packet to be transmitted and mapped to the current service data container. For envelope reporting controlled by the Time Quota Mechanism, this indicates the time stamp for the first IP packet to be transmitted that causes an envelope to be opened – see TS 32.299 [50].

-	Time of Last Usage is the time stamp for the last IP packet to be transmitted and mapped to the current service data container. For envelope reporting, controlled by the Time Quota Mechanism, this indicates the time stamp for an envelope to be closed – see TS 32.299 [50] for conditions for envelope closure.

-	Time Usage contains the effective used time within the service data container recording interval.

-	Service Condition Change defines the reason for closing the service data container (see TS 32.251 [11]), such as tariff time change, IP-CAN bearer modification (e.g. QoS change, S-GW change, user location change, user CSG information change), access change of service data flow, indirect service condition change, service usage thresholds, service idled out, termination or failure handling procedure. When one of the "CGI/SAI, ECGI or TAI or RAI Change" are reported as user location change, the dedicated value in service Condition Change is set instead of the generic "user location change" value. This field is specified as bitmask for support of multiple change trigger (e.g. S-GW and QoS change). This field is derived from Change-Condition AVP at Service-Data-Container AVP level defined in TS 32.299 [50] when received on Rf. Each value is mapped to the corresponding value in "ServiceConditionChange" field. When simultaneous change triggers are met, multiple Change-Condition values are set in field bitmask. When no Change-Condition AVP is provided, the "recordClosure" value is set for the service data container. For envelope reporting, the Service Condition Change value shall always take the value "envelopeClosure". The mechanism for creating the envelope is identified within the Time Quota Mechanism field.

-	EPC Qos Information in service specific service data containers contains the QoS applied for the service.   This is included in the first service data container. In following container EPC QoS information is present if previous change condition is "QoS change". The P-GW/TDF/IPE-CDR shall include only one EPC QoS Information occurrence to one service data container.

-	Serving Node Address contains the valid serving node (e.g. SGSN/S-GW) control plane IP address during the service data container recording interval.

-	Data Volume Uplink and Downlink, includes the number of octets transmitted during the service data container recording interval in the uplink and/or downlink direction, respectively. The amount of data counted in P-GW shall be as specified in clauses 5.2.1.10.1, 5.3.1.1 and 5.3.1.6.1 of TS 32.251 [11]. The amount of data counted in TDF shall be based on full payload of the data transferred. The amount of data counted in IP-Edge shall be based on full payload of the data transferred.

-	Report Time is a time stamp, which defines the moment when the service data container is closed.

-	Service Identifier is an identifier for a service. The service identifier may designate an end user service, a part of an end user service or an arbitrarily formed group thereof. This field is only included if reporting is per combination of the rating group and service id.

-	Multi-modal Service Identifier indicates the multi-modal service with which the service data flow is associated. This field is only included if the "MultiModaIId" feature is supported as defined in TS 29.512 [251].

-	PS Furnish Charging Information includes charging information per rating group in case it is sent by OCS.

-	User location information contains the user location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE was located during the service data container recording interval. This is included in the service data container only if previous container's change condition is "user location change" or one of the "CGI/SAI, ECGI or TAI or RAI Change". Note the user location information in PGW-CDR/TDF-CDR main level contains the location where the UE was when PGW-CDR/TDF-CDR was opened, and the "Last user location information" in PGW-CDR main level contains the location where the UE was when PGW-CDR is closed.

-	Presence Reporting Area Status contains the status of the UE presence (i.e. indication on whether the UE is inside or outside) in the Presence Reporting Area(s) identified by "Presence Reporting Area identifier(s)" contained in PGW-CDR main level. This is included in the service data container when the initial status is reported after opening of PGW-CDR, or if previous container’s change condition is "Presence in Presence Reporting Area Change". This field is not applicable in multiple PRA(s).

-	List of Presence Reporting Area Information contains the list of Presence Reporting Area Information. Each Presence Reporting Area Information includes the "Presence Reporting Area identifier", the status of the UE presence (i.e. indication on whether the UE is inside or outside the Presence Reporting Area or Presence Reporting Area(s) inactive), and the Node (PCRF, OCS) which subscribed-to the PRA.

-	3GPP2 User Location Information contains the 3GPP2 user location (i.e. 3GPP2-BSID: Cell-Id, SID, NID) where the UE was located during the service data container recording interval. This is included in the service data container only if previous container's change condition is "user location change". Note the "3GPP2 user location information" in PGW-CDR/TDF-CDR main level contains the location where the UE was when PGW-CDR/TDF-CDR was opened.

-	UWAN User Location Information contains the Untrusted Wireless Access Network (UWAN) user location during the service data container recording interval. This is included in the service data container only if previous container’s change condition is "user location change". Note the user location information in PGW-CDR main level contains the location where the UE was when PGW-CDR was opened.

-	TWAN User Location Information contains the user location in a Trusted WLAN Access Network during the service data container recording interval. This is included in the service data container only if previous container’s change condition is "user location change". Note the user location information in PGW-CDR/TDF-CDR main level contains the location where the UE was when PGW-CDR/TDF-CDR was opened.

-	User CSG information contains the status of the user accessing a CSG cell (CSG ID within the PLMN, Access mode and indication on CSG membership) during the service data container recording interval. This is included in the service data container only if previous container’s change condition is "user CSG information change". Note the "user CSG information" in PGW-CDR main level contains the "user CSG information" when PGW-CDR was opened.

-	AF-Record-Information includes the "AF Charging Identifier" (ICID for IMS) and associated flow identifiers generated by the AF and received by the P-GW over Gx interfaces as defined in TS 29.212 [220]. In case usage of PCC rules, which usage is reported within the container, has different AF-Record-Information then the P-GW shall include all occurrences to the service data container.

-	Event Based Charging Information includes the number of events and associated timeStamps (each event is timestamped) during the service data container recording interval.

-	Time Quota Mechanism contains two further subfields and is included if envelope reporting is required:

-	Time Quota Type identifies the mechanism by which time based usage should be reported – as defined in TS 32.299 [50].

-	Base Time Interval identifies the length of the base time interval, for controlling the reporting of time based usage, in seconds.

-	Service Specific Info holds service specific data for a pre-defined PCC or a predefined ADC rule that is used for enhanced packet filtering.

-	Sponsor Identity identifies the sponsor willing to pay for the operator's charge for connectivity.

-	Application Service Provider Identity identifies application service provider that is delivering a service to an end user.

-	RAT Type contains the RAT type for the IP-CAN bearer that is first reported for the Rating Group or Rating Group / Service Identifier in the container. If traffic from multiple bearers is included in the report for the container, only one field is included.

NOTE: 	Sponsor Identity and Application Service Provider Identity are not used together with Service Identification reporting. Furthermore, neither the Sponsor Identity nor Application Service Provider Identity is applicable to the TDF.

-	Related Change of Service Condition indicates the reason a related container was closed when the current container is indirectly closed and the supplemental information for the event. This information is applicable when charging per IP-CAN session is active for a multi-access PDN connection.

-	Serving PLMN Rate Control This field contains the Serving PLMN Rate Control applied by MME during the transfer of the data volume captured by the container (applicable only to the PGW-CDR). This is included in the service data container only if previous container's change condition is "Serving PLMN Rate Control change ". Note the Serving PLMN Rate Control field in PGW-CDR main level contains the Serving PLMN Rate Control when PGW-CDR was opened.

-	APN Rate Control This field contains the APN Rate Control applied by PGW during the transfer of the data volume captured by the container (applicable only to the PGW-CDR). This is included in the service data container only if previous container's change condition is "APN Rate Control change ". Note the APN Rate Control field in PGW-CDR main level contains the APN Rate Control when PGW-CDR was opened.

-	Traffic Steering Policy Identifier Downlink This field contains the Traffic Steering Policy Identifier Downlink applied by PGW/ TDF during the service data container recording interval.

-	Traffic Steering Policy Identifier Uplink This field contains the Traffic Steering Policy Identifier Uplink applied by PGW/TDF during the service data container recording interval.

-	VoLTE Information This field contains the caller and callee Information of VoLTE. Caller Information of VoLTE is described in clause 5.1.3.1.24. Callee Information contains:

-	Called Party Address (described in clause 5.1.3.1.9).

-	Requested Party Address (described in clause 5.1.3.1.43).

-  	List of Called Asserted Identity (described in clause 5.1.3.1.23).

Second Change

5.1.5.0	CHF record (CHF-CDR)

If enabled, CHF records shall be produced for chargeable events, with or without quota management. A CHF-CDR shall be opened when the CHF receives Charging Data Request [Initial].

As an alternative to the default CHF behaviour, the "Individual Partial record" mechanism can be used based on Operator's policy configured in the CHF. In this case a new CDR shall be opened for each Charging Data Request [Initial, Update, Termination], charging information shall be added and the CDR shall then be closed. The Sequence Number will be incremented for each Charging Data Request [Initial, Update, Termination] received by the CHF.

The generic fields in the record are specified in table 5.1.5.0.1. The NF specific parts will be concatenated to this e.g. the PDU Session Information, PDU Container Information and Roaming QBC Information are concatenated for the SMF.

Table 5.1.5.0.1: CHF record (CHF-CDR)

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record, clause 5.1.5.1.10. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, clause 5.1.5.1.11. |
| Charging Session Identifier | OC | This field holds the Session Identifier described in TS 32.290 [57]. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI), clause 5.1.5.1.13. |
| Tenant Identifier | OM | This field holds the tenant identifier |
| MnS Consumer Identifier | OM | This fields holds the identifier of the MnS Consumer. |
| NF Consumer Information | M | This field holds the information of the NF consumer of the charging service, clause 5.1.5.1.6. |
| NF Functionality | M | This field holds the type of functionality the NF provides. |
| NF Name | OC | This field holds the name of the NF used. |
| NF Address | OC | This field holds the IP Address of the NF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the NF. |
| Invocation Timestamp | OM | This field holds the timestamp of the charging service invocation, clause 5.1.5.1.19. |
| Charging Identifier | OM | Charging identifier for correlation between different records. Only applicable if not available in the service specific information. |
| Triggers | OC | This field holds the triggers that are common to all Multiple Unit Usage. Can be the same as in Used Unit Container. |
| SMF Triggers | OC | This field holds the 5G data connectivity specific triggers described in TS 32.255 [15]. |
| List of Multiple Unit Usage | OC | This field holds the parameters for the unit reporting. It may have multiple occurrences, clause 5.1.5.1.3. |
| Rating Group | M | This filed holds the rating group, clause 5.1.5.1.7. |
| Used Unit Container | OC | This field holds the used units and information connected to the reported units, clause 5.1.5.1.14. |
| Service Identifier | OC | This field holds the Service Identifier. |
| Multi-modal Service Identifier | OC | This field holds the identifier of the multi-modal service. |
| Quota management Indicator | OC | This field holds an indicator on whether the reported used units are with or without quota management control. If the field is not present, it indicates the used unit is without quota management applied. |
| Local Sequence Number | OM | This field holds the container sequence number. |
| Time | OC | This field holds the amount of used time. |
| Uplink Volume | OC | This field holds the amount of used volume in uplink direction. |
| Downlink Volume | OC | This field holds the amount of used volume in downlink direction. |
| Total Volume | OC | This field holds the amount of used volume in both uplink and downlink directions. |
| Service Specific Units | OC | This field holds the amount of used service specific units. |
| Event Time Stamp | OC | This field holds the timestamps of the event reported in the Service Specific Units, if the reported units are event based. |
| Rating Indicator | OC | This field indicates if the units have been rated or not. |
| Triggers | OC | This field holds the triggers that caused the Used Unit Container to be reported, independently on if they are PDU Session or RG level triggers. |
| SMF Triggers | OC | This field holds the 5G data connectivity specific triggers described in TS 32.255 [15]. |
| IMS Triggers | OC | This field holds the IMS specific triggers described in TS 32.260 [20]. |
| MB-SMF Triggers | OC | This field holds the 5G Multicast-broadcast Services related triggers described in TS 32.279 [39]. |  |
| Trigger Time Stamp | OC | This field holds the timestamp of the trigger. |
| PDU Container Information | OC | This field holds the 5G data connectivity specific information described in TS 32.255 [15]. |
| NSPA Container Information | OC | This field holds the network slice performance and analytics container specific information described in TS 28.201 [151]. |
| PC5 Container Information | OC | This field holds the PC5 container information |
| MBS Container Information | OC | This field holds the MBS container information |  |
| Allocated Unit | OC | This field holds the Allocated Unit. |  |
| Quota management Indicator | OC | This field holds an indicator on whether the reported allocated unit are with or without quota management control. If the field is not present, it indicates the allocated unit without quota management applied. |  |
| Triggers | OC | This field holds the triggers that caused the Allocated unit Container to be reported. |  |
| NSACF Triggers | OC | This field holds the Network slice admission control Charging specific triggers described in TS 28.203 [72]. |  |
| Trigger Timestamp | OC | This field holds the timestamp of the trigger. |  |
| Local Sequence Number | OM | This field holds the container sequence number. |  |
| NSAC Container Information | OC | This field holds the Network Slice Admission Control specific units in use described in TS 28.203 [72]. |  |
| UPF ID | OC | This field holds the UPF identifier used to identify the UPF when reporting the usage for the UPF. |
| Record Opening Time | OC | This field contains the time stamp when the record is opened, clause 5.1.5.1.8. |
| Duration | M | This field holds the duration of this record, clause 5.1.5.1.3. |
| Record Sequence Number | C | Partial record sequence number, clause 5.1.5.1.9. |
| Cause for Record Closing | M | The reason for the release of the record, clause 5.1.5.1.2. |
| Local Record Sequence Number | OM | This field holds consecutive record number, described in clause 5.1.5.1.5. The number is allocated sequentially including all CDR types. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record, clause 5.1.5.1.12. |
| Service Specification Information | OC | Identifies service specific document that applies to the request, clause 5.1.5.1.16. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information described in TS 32.255 [15] |
| Roaming QBC Information | OM | This field holds the roaming 5G data connectivity specific information described in TS 32.255 [15] |
| SMS Charging Information | OC | This field holds the SMS specific information described in TS 32.274 [34]. |
| Registration Charging Information | OM | This field holds the 5G registration specific information described in TS 32.256 [16]. |
| N2 connection charging Information | OM | This field holds the N2 connection specific information described in TS 32.256 [16]. |
| Location reporting charging Information | OM | This field holds the Location reporting specific information described in TS 32.256 [16]. |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in TS 32.254 [14]. |
| NSPA Charging Information | OM | This field holds the performance and analytics specific information described in TS 28.201 [151]. |
| NSM charging Information | OM | This field holds the Network Slice Management (NSM) specific information described in TS 28.202 [71]. |
| IMS Charging Information | OM | This field holds the IMS specific information described in TS 32.260 [20]. |
| ProSe charging Information | OM | This field holds the ProSe specific information described in TS 32.277 [37]. |
| Edge Enabling Infrastructure Resource Usage Charging Information | OM | This field holds the Edge Enabling Infrastructure Resource Usage Charging Information described in TS 32.257 [17]. |
| EAS Deployment Charging Information | OM | This field holds the EAS Deployment Charging Information described in TS 32.257 [17]. |
| Direct Edge Enabling Service Charging Information | OM | This field holds the Direct Edge Enabling Service Charging Information described in TS 32.257 [17]. |
| Exposed Edge Enabling Service Charging Information | OM | This field holds the Exposed Edge Enabling Service Charging Information described in TS 32.257 [17]. |
| EAS ID | OC | This field holds the EAS ID described in TS 32.257 [17]. |
| EDN ID | OC | This field holds the DN of EdgeDataNetwork MOI described in TS 32.257 [17]. |
| EAS Provider Identifier | OC | This field holds the identifier of the ASP that provides the EAS described in TS 32.257 [17]. |
| NSACF Charging Information | OC | This field holds the Network slice admission control Charging Information described in TS 28.203 [72]. |  |
| TSN Charging Information | OM | This field holds the time sensitive networking charging information described in TS 32.282 [43]. |  |
| MBS Session charging Information | OC | This field holds the MBS Session specific information described in TS 32.279 [39]. |  |
| NSSAA Charging Information | OC | This field holds the Network slice-specific authentication and authorization Charging Information described in TS 28.204 [73]. |  |
| Ranging and Sidelink Positioning Charging Information | OC | This field holds the Ranging and Sidelink Positioning Charging Information described in TS 32.271 [31]. |  |
| LCS Information | OC | This field holds the 5GS LCS Charging Information described in TS 32.271 [31]. |  |

Third Change

5.1.5.1.14	Used Unit Container

This list applicable in CHF-CDR includes one or more containers.

Each container includes the following fields:

-	Service Identifier may designate an end user service, a part of an end user service or an arbitrarily formed group thereof.

-	Multi-modal Service Identifier indicates the multi-modal service with which the service data flow is associated.

-	Quota management Indicator holds an indicator on whether the reported used units are with or without quota management control. If the field is not present, it indicates the used unit is without quota management applied.

-	Local Sequence Number is the sequence number for the used unit containers, i.e. the order in which charging information was reported or used unit container was closed.

-	Time includes the duration of a time based service.

-	Uplink Volume  includes the number of octets transmitted during the use of the packet data services in the uplink direction. The counting of uplink data volumes is optional.

-	Downlink Volume  includes the number of octets transmitted during the use of the packet data services in the downlink direction.

-	Total Volume includes the total number of octets transmitted in both uplink and downlink direction.

-	Service Specific Units includes the number of units, specific for the service, used during the service.

-	Event Time Stamp defines the moment when the event was reported in the Service Specific Units when event based charging applies.

-	Rating Indicator indicates if the units have been rated or not.

-	Triggers includes the reason for charging information reporting or closing for the used unit container, the 5G data connectivity specific triggers are described in TS 32.255 [15].

-	Trigger Time Stamp is the date and time of the charging information reporting or closing for the used unit container.

-	PDU Container Information is the 5G data connectivity specific information described in TS 32.255 [15].

End of Changes