3GPP TSG-SA5 Meeting #163	S5-254441

Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR | 0004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.567 Correction on Stage-3 Fault Management CCL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA5#162, S5-253486 pCR was approved for Stage-2 corrections for fault management attributes to follow camel case, however these were not reflected in Stage-3. Also faultManagementAlarmIdList is specified as a List in Stage 2 but as a string in Stage 3. This CR aims to make necessary changes in Stage-3 to align with Stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Apply necessary Stage-3 changes in Rel19 TS28567_CclNrm.yaml file to align with Stage 2 definitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 implementation of Fault Management CCL is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TS28567_CclNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1939 at commit e40aa4542e97fbf884a13cada7ceea5f41d1e01f |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1939 at commit e40aa4542e97fbf884a13cada7ceea5f41d1e01f

*** START OF CHANGE 1 ***

*** OpenAPI/TS28567_CclNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: CCL NRM

version: 19.0.0

description: >-

OAS 3.0.1 definition of the CCL NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.567; management aspect of closed control loops

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.567/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-CCLNrm:

type: object

properties:

ClosedControlLoop:

$ref: '#/components/schemas/ClosedControlLoop-Multiple'

#-------Definition of generic IOCs ----------#

ClosedControlLoop-Single:

description: >-

This IOC represents the properties of an CCL management information between MnS consumer and MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

cCLComponentsInfo:

type: array

items:

$ref: '#/components/schemas/CCLComponentsInfo'

description: >-

It indicates information on the constituent components of a CCL.

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

cCLPriority:

type: integer

minimum: 1

maximum: 10

description: This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority.

cCLComponentList:

type: array

items:

$ref: '#/components/schemas/CCLComponent'

description: >-

It indicates the list of components ating as steps of the CCL.

cCLType:

type: string

enum:

- ENERGYOPTIMIZATION

- SLICEASSURANCE

cCLActionTrigger:

type: string

desiredBehavior:

type: string

enum:

- DECISION_ACTIVATION

- NOTIFY_RCOMMENDATION

- DO_NOTHING

cCLPurposeRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

CCLScope-Single:

description: It indicates a scope of a CCL.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

cCLPurposeRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

CCLReport-Single:

description: This class represents the reported outcomes on a CCL instance.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

scopeType:

type: string

enum:

- CCL_MEASUREMENT_SCOPE

- CCL_TARGET_SCOPE

- CCL_CONTROL_SCOPE

- CCL_IMPACT_SCOPE

ConflictManagementAndCoordinationEntity-Single:

description: This defines the conflict management functionality.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

coordinationCapability:

type: string

description: >-

It indicates a capability of a coordination entity to coordinate CCL conflicts.

coordinatedCCLsScopes:

type: string

cCLActionConflictsHandling:

$ref: '#/components/schemas/CCLActionConflictsHandling'

FaultManagement:

description: This IOC represents the Fault Management CCL purpose.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

faultManagementAlarmIdList:

type: array

items:

type: string

faultManagementBackUpObjectRequirement:

type: boolean

faultManagementIsolateObjectRequirement:

type: boolean

faultManagementTimeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

clearUserId:

type: string

#-------Definition of the generic dataType --------------#

CCLComponentsInfo:

type: object

properties:

cCLComponentId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

cCLSteps:

type: string

enum:

- DATA_COLLECTION

- ANALYSIS

- DECISION

- EXECUTION

CCLComponent:

type: object

properties:

cCLComponentIdentification:

type: string

cCLComponentRole:

type: string

enum:

- MONITOR

- ANALYSIS

- DECISION

- EXECUTION

- OTHER

FaultManagementCCLReport:

type: object

properties:

generatedAlarmResultList:

type: array

items:

type: string

faultManagementCCLReportTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

GeneratedAlarmResult:

type: object

properties:

alarmId:

type: string

alarmClearedStatus:

type: boolean

identifiedRootCauseInformation:

type: string

enhancedCorrelationInformation:

type: array

items:

type: string

CCLScopeCoordinationCapability:

type: object

properties:

cCLCoordinationCapabilityID:

type: string

CCLActionConflictsHandling:

type: object

properties:

conflictInformation:

$ref: '#/components/schemas/ConflictInformation'

conflictResolution:

$ref: '#/components/schemas/ConflictResolution'

targetCCL:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

ConflictInformation:

type: object

properties:

conflictingCCLId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

conflictingActions:

type: string

ConflictResolution:

type: object

properties:

conflictingCCLId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

cCLGoalBreachPercentage:

type: integer

#-------Definition of the generic dataType --------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

ClosedControlLoop-Multiple:

type: array

items:

$ref: '#/components/schemas/ClosedControlLoop-Single'

CCLScope-Multiple:

type: array

items:

$ref: '#/components/schemas/CCLScope-Single'

CCLReport-Multiple:

type: array

items:

$ref: '#/components/schemas/CCLReport-Single'

ConflictManagementAndCoordinationEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/ConflictManagementAndCoordinationEntity-Single'

resources-CCLNrm:

oneOf:

- $ref: '#/components/schemas/ClosedControlLoop-Single'

- $ref: '#/components/schemas/CCLScope-Single'

- $ref: '#/components/schemas/CCLReport-Single'

- $ref: '#/components/schemas/ConflictManagementAndCoordinationEntity-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of changes