3GPP TSG-SA5 Meeting #162	S5-254566

Wuhan, China, 13 - 17 October 2025

Source:	Nokia, Samsung

Title:	Pseudo-CR on Rel-20 pCR TR 28.888 Add use case and requirements for authorization of the external MnS consumers at the CCF

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.0.0

Work Item:	FS_EnExpo

Comments

This pCR proposes to add a use case “Authorization of the external MnS consumer to access the management service API” and the corresponding requirements to TR 28.888.

Proposed Changes

* * * First Change * * * *

# 5 Use cases

## 5.X Management service exposure alignment with other related exposure industry solutions

### 5.X.Y	Authorization of the external MnS consumer to access service APIs at the CCF

#### 5.X.Y.1 Description

This usecase describes how the external MnS consumer can be configured at the CCF with the authorization information to enable the consumption of the service APIs at the AEF of MSED.  For the external MnS consumer, the authorization information provided by the CCF contains the following:

- The identity of the external MnS consumer as assigned by the CCF

- The access permissions granted to the external MnS consumers. The permissions will determine, for example, what service API(s), corresponding resources (MnS Component B and C) and operations (MnS Component A) that the external MnS consumer will have access to within the management system.

- The expiry date of the granted authorization.

#### 5.X.Y.2	Potential requirements

PREQ-FS_EnExpo-AuthCCF-01: The exposure of management services using CAPIF shall provide the capability enabling the CCF to authorize the external MnS consumer to access the management service(s).

Note: The authorization for external MnS consumer shall indicate the resources represented by MnS Component B and C that the MnS consumer can access and the operations represented by MnS Component A that the MnS consumer can perform on these resources.

PREQ-FS_EnExpo-AuthCCF-02: The exposure of management services using CAPIF shall provide the capability enabling the AMF of MSED to indicate to the CCF that the 3GPP management system shall be involved in the authorization of the external MnS consumer.

* * * End of Changes * * * *