3GPP TSG-SA WG6 Meeting #69	S6-254152

Wuhan, China, 13th – 17th October 2025	(revision of S6-253xxx)

Source:	Huawei, Hisilicon

Title:	New solution of AIoT data processing and event notification

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This solution aims to address the issues identified in Key Issue 2 and key Issue 4.

2. Reason for Change

This AIoT application desires the applcation events rather than each AIoT device ID. So the enabler layer could make use of the reported AIoT device ID and other provisioned service information to create the application interested event notification.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS / TR <TS/TR number and version>.

* * * First Change * * * *

<Proposed change in revision marks>

7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #X |  | Y |  | Y |
| Sol #2 |  |  |  |  |

* * * Next Change * * * *

7.x	Solution #X: AIoT Data Processing management

7.x.1	Solution description

This solution address key issue#2, and 4 to introduce a solution for AIoT data processing.

The AIoT tasks from the application layer are first transferred into inventory intentions, then integrated into AIoT inventory tasks. Correspondingly, the inventory results from AIoT services need to be first integrated into AIoT events, then transferred into applictaion events by scenario-based data processing process. These applictaion events can be reported as the result of applictaion AIoTtasks to the application layer for consumption.

The figure 7.x-1 shows the high-level procedure of AIoT data processing management and event notification.

Pre-conditions:

- The AIoT task execution service is invoked by the AIoT service provider, and the AIoT enabler has initiated the AIoT service request to the 3GPP network.

Figure  7.X-1 AIoT Data Processing management

Step1.	 After the 3GPP AIoT services are called, the AIoT data are collected from 5GC to AIoT enabler server.

Step2.	The AIoT enabler integrates the discrete AIoT raw data during a limited time to meaningful service event per provisioing phase. The AIoT service event contains the information of “what AIoT device ID(s)”,  “in which area,” and “what happened”. For “what happened”, there are  four types of AIoT events are summarized:.

Type1: AIoT device appearance, the AIoT device recognized at the reader or region for the first time;

Type2: AIoT device disappearance, the AIoT device can’t be recognized during the limited time;

Type3: AIoT device stay, the AIoT device is continuously recognized at the same reader or region;

Type4: AIoT device moving, the AIoT device is recognized from area A to Area B.

Step3. Application events generation (Contextual processing).

Application events follow "which application object, in which logical location, experienced what meaningful application event, and associated with which application attributes". Based on AIoT events, together with AIoT service provision information, semantic translation and scenario-based processing is conducted to generate application events that can be understood and utilized by the application layer.

- Application semantic translation:

Based on AIoT service provisioning information, establish preliminary associations between application objects and application regions.

| AIoT Event information | Application Event information |
| --- | --- |
| AIoT device ID | Application object |
| AF ID(s) Target Area Information | Application area |

(2)	Contextual processing

The application’s AIoT task information includes the application’s object ID(s) or type, application service area and AIoT service scenario type. Based on the requiement of application, the AIoT events can be filterred by application ID(s)/type or application area or both, to ensure that the application layer receives interested information .

With the AIoT service type information, especially the interested AIoT service events information, more contextual process can be performed to generate the application event.

-	Inbound or Outbound: for all the AIoT devices appearance events during the task execution period, compare the objects ID(s) reported from AIoT networkand the objects ID lists fromthe AIoT tasks. If the ID(s) can match, then, the application objects can be successfully inbound or outbound; otherwise, it is considered an illegal operation and an alert is triggered.

-	Inventory: for all the AIoT devices appearance and stay events, the corresponding objects can be seen as the normal inventory information, however, for the AIoT devices disappearance event, an alert or other method such as specific ID inventory should be established.

-	Monitor: for all the AIoT devices appearance, disappearance, moving events, the AIoT device state changed, the AIoT enable layer should record and report.

Step4.	Report the interested application events to the application layer and the AIoT task is completed.

7.x.2	Architecture Impacts

7.x.3	Corresponding APIs

The AIoT service event notification service is introduced.

7.x.4	Solution evaluation

* * * Next Change * * * *