3GPP TSG-SA WG6 Meeting #69	S6-254198

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on support of satellite related information utilizing AI analysis

Spec:	3GPP TR 23.700-02 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution for support of satellite related information utilizing AI analysis.

2. Reason for Change

The VAL server or the UE may act as consumers to subscribe the analysis related to the satellite communication information from ADAES. This solution is proposed to support the analysis related to the satellite communication in the application enablement layer.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-02 v0.1.0.

* * * First Change * * *

5.x	Solution #x: Support of satellite related information analysis

5.x.1	Solution description

This solution addresses the KI#6.

The VAL server or the UE may act as consumers to subscribe the analysis related to the satellite communication information from ADAES. They may ask to analyse the satellite coverage availability information of the target UE to assist the UE's satellite access or to minimize the impact on services (e.g. inform the VAL server in time to pause the downlink data when the target UE is unavailable under the satellite coverage).

5.x.1.1	Procedure of Satellite related information subscription request

Figure 5.x.1.1-1 illustrates the high-level procedure of Satellite related information subscription request.

Pre-condition:

-	The following NFs in the application enablement layer (e.g., ADAES, SEAL) are all deployed on the ground.

Figure 5.x.1.1-1: Procedure of Satellite related information subscription request

1.	Consumers (e.g., the VAL server or UE) initiate a satellite communication analysis subscription request to the ADAES, with the analysis ID (i.e. the satellite coverage availability information), analysis type (statistics or prediction), target UE ID, service ID, target service area, reporting conditions (e.g., periodic reporting or event-triggered reporting), etc.

2.	The ADAES checks whether the VAL server is authorized to invoke such request.

3.	If the request is authorized, the AIoT enabler sends a satellite communication analysis subscription response to the VAL server.

4a. ADAES may obtain the satellite related information for the target UE from the SEAL servers in the following ways:

-	ADAES may obtain the satellite coverage availability information related to the target UE from the configuration management server as specified in clause 21.2.2.1 of 3GPP TS 23.434[4];

-	ADAES may retrieve the location information and access type of the target UE from the location management server as specified in clause 9.3.5 of 3GPP TS 23.434[4].

4b. ADAES may obtain the satellite related information for the target UE from the 3GPP CN in the following ways:

-	ADAES may obtain the location information and UE mobility analysis information for the target UE from the 3GPP CN (i.e. NWDAF) as specified in clause 6.7.2 of 3GPP TS 23.288 [23288]).

5.	The ADAES stores and analyse the received satellite related information based on the satellite communication analysis subscription service request received in Step 1. The ADAES may determine the satellite availability information and the predicted information for the target UE in a specific service area/time point. E.g., the available satellite information for the target UE in a certain area, the predicted time point when the target UE enters/exits the coverage of a certain satellite, or the predicted time periods during which the target UE is available under the coverage of a certain satellite.

NOTE 1:	Whether the ADAES provides the predictive satellite related information depends on the prediction parameter is included in Step 1or not.

6a.	The ADAES will report the satellite communication-related analysis (including the statistics and predict information) to the consumer via the satellite communication analysis notification message. And the consumers may take actions to minimize the service impact if there is. For example, the VAL server may promptly adjust the downlink data polices (e.g., pending the downlink data) based on the time points when the UE is unavailable under the satellite coverage.

6b. The ADAES may generate the corresponding data polices based on the analysed satellite related information and sends these data polices to the consumer directly. These data polices below only apply when the target UE is unavailable under the satellite coverage:

-	Reject forwarding the data for new uplink and downlink data;

-	Pending forwarding the data and cache for new uplink and downlink data;

-	Forwarding the data according to priority for existing uplink and downlink data.

NOTE 2:	The priority for the forwarding polices may be set according to the service QoS/QoE, user levels, operator policies, pre-configurations, etc.

NOTE 3:	It’s up to the ADAES to decide which step (6a or 6b) is performed.

5.x.2	Architecture Impacts

This solution has no impact on the existing architecture.

5.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

5.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for ambient power-enabled IoT, Stage 1".

[3]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things, Stage 2".

[4]	3GPP TR 23.700-30: "Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”.

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[23288]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

* * * End of Change * * * *