3GPP TSG-SA WG6 Meeting #69	S6-254233

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#1 on Functional Architecture

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-15 clause 4.1), the study aspects on application enablement architecture enhancements to support utilization and exposure of sensing results include:

- Whether and how to enhance the application enablement architecture and related functions to support the mapping of sensing service requirements of vertical applications.

- Whether and how the existing exposure mechanisms (e.g., subscription, notification, etc) defined in the SEAL architecture and functions are sufficient to meet vertical application requirements.

- Whether and how to process the collected sensing results (e.g., sensing exposure from the core network), to generate application enabler layer enhanced sensing results, and to expose them to vertical applications.

- Whether and how to resolve conflicts in sensing service requirements among different vertical applications or under resource limitations.

- Whether new application enabler architecture is needed to support the above functions.

There are some common requirements from difference sensing applications, for example, sensing data/result collection, sensing result aggregation, sensing result exposure. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

Thus, this pCR proposes to enhance the application enablement layer architecture by introducing new enabler service to support sensing application relevant services.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1, to enhance the application enablement layer architecture by introducing new enabler service to support sensing application relevant services.

3. Conclusions

This paper proposes solution on functional architecture to support sensing application relevant services.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X | X |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Functional Architecture to Support Sensing Application Relevant Services

### 6.X.1	Solution Description

There are some common requirements from difference sensing application relevant services, for example, sensing data/result collection, sensing result aggregation, sensing result exposure. Enhancements to the application enablement layer architecture are needed to introduce common functionalities to fulfil these comment requirements.

The following clauses specify generic functional model of SAE (Sensing Application Enablement) for support application layer sensing services.

#### 6.X.1.1	Functional Architecture

##### 6.X.1.1.1	General

The functional architecture enhancement for the SAE (Sensing Application Enablement) is based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [xx]. It is organized into functional entities to describe an architecture enhancement which addresses the support for sensing aspects for vertical applications.

##### 6.X.1.1.2	On-Network Functional Architecture

Figure 6.X.1.1.2-1: Generic on-network functional model

Figure 6.X.1.1.2-1 illustrates the generic on-network functional model with SAE as new SEAL service. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The SEAL functional entities with SAE function on the UE and the server are grouped into SAE client(s) and SAE server(s) respectively. The SAE consists of a common set of services (e.g., sensing data/result collection, sensing result aggregation, sensing result exposure) and reference points. The SAE offers its services to the vertical application layer (VAL).

The SAE client(s) communicates with the SAE server(s) over the SAE-UU reference points. SAE-UU supports both unicast and multicast delivery modes. The SAE client(s) provides the service enabler layer support functions to the VAL client(s) over SAE-C reference points. The VAL server(s) communicate with the SAE server(s) over the SAE-S reference points. The SAE server(s) may communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system.

The SAE server may optionally obtains sensing data/result from the 3rd party SAE server via SAE-3P reference point. SAE-3P reference point is out of scope of 3GPP.

##### 6.X.1.1.3	Off-Network Functional Architecture

Figure 6.X.1.1.3-1: Generic off-network functional model

Figure 6.X.1.1.3-1 illustrates the generic off-network functional model with SAE as new SEAL service. In the vertical application layer, the VAL client of UE1 communicates with VAL client of UE2 over VAL-PC5 reference point. An SAE client of UE1 interacts with the corresponding SAE client of UE2 over SAE-PC5 reference points. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

#### 6.X.1.2	Functional Entities Description

##### 6.X.1.2.1	General

The SAE functional entities with sensing application relevant service functions are described in the following subclauses.

##### 6.X.1.2.2	SAE client

The SAE client functional entity acts as the application client supporting sensing data/result collection, and sensing status monitoring. It interacts with the SAE server.

##### 6.X.1.2.3	SAE server

The SAE server functional entity provides for sensing application relevant services supported within the vertical application layer.

##### 6.X.1.2.4	3rd party SAE server

The 3rd party SAE server functional entity provides for sensing application relevant services supported for the vertical application layer.

#### 6.X.1.3	Reference Points Description

##### 6.X.1.3.1	General

The reference points for the functional model for sensing application relevant service are described in the following subclauses.

##### 6.X.1.3.2	SAE-UU

The interactions related to sensing application relevant service functions between the SAE client and SAE server are supported by SAE-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [2] and 3GPP TS 23.501 [3].

SAE-UU reference point is used for VAL service signalling for VAL service data management of the VAL service.

##### 6.X.1.3.3	SAE-S

The interactions related to sensing application relevant services functions between the VAL server(s) and the SAE server are supported by SAE-S reference point.

##### 6.X.1.3.4	SAE-C

The interactions related to sensing application relevant services functions between the VAL client(s) and the SAE client(s) within a VAL UE are supported by SAE-C reference point.

##### 6.X.1.3.5	SAE-PC5

The interactions related to SAE enablement functions between SAE clients in off-network deployments.

##### 6.X.1.3.6	SAE-3P

The reference point SAE-3P is used for sensing data/result collection from a third-party SAE server.

NOTE:      Further definition of SAE-3P is out of scope of this specification.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***