3GPP TSG-SA WG6 Meeting #69	S6-254278

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Lenovo

Title:	Solution #11 update

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This contribution provides an update of solution #11 to complete it.

2. Reason for Change

This contribution provides an update of solution #11 to complete it with corresponding APIs and evaluation.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.11	Solution #11: Solution of FL member registration in multi-operator scenarios

### 7.11.1	Solution description

This solution addresses Key Issue #3.

The solution provides a mechanism for supporting registration for candidate FL member (e.g. FL clients) which are application and/or application enablement layer entities (AIMLE server, VAL server, AIMLE clients, VAL clients), where the candidate FL members aim to operate in a service area which is covered by multiple networks operated by different operators, and the candidate FL members are connected to more than one network (either some members are connected to PLMN1 and some to PLMN2 or both).

In cross-PLMN scenario, for registering the FL members using the multi-operator information, the following enhancements are provided in 3GPP TS 23.482 clause 8.4.2 (for registration) and clause 8.4.3 (for registration update).

8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

NOTE: Such allowed MNO info is applicable to scenarios when the candidate FL member has limitations being grouped with other FL members being connected to other PLMNs. Such limitations are imposed based on the vendor providing the service in cross-PLMN scenarios, e.g. based on an interoperability indication. If the vendor is the same is the same across FL member connected to different PLMNs, then it may be possible that the FL members of different PLMNs collaborate e.g. in cases like VFL.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving or allowed PLMN.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving, or allowed PLMN.  In case of registration update, the mapping to the one or more PLMNs and/or the roles may be updated accordingly.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

### 7.11.2	Architecture Impacts

The architecture impact is the required enhancement of the AIMLE functional model to support multi-operator scenarios (as in clause 6.2).

### 7.11.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #7. This provides enhancements to the following information flows based on TS 23.482 clause 8.4.4.1. The additional information elements as described in the above procedure w.r.t. mulit-operator support will enhance the existing API as in TS 23.482 clause 9.3.1 (MLR_FLMemberRegister API).

### 7.11.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support FL member registration and registration update in cross-PLMN scenarios. This capability mainly introduces enhancements to AIMLE existing procedures as in clause 8.4.2 and 8.4.3 of TS 23.482.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

* * * Next Change * * * *

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |  |
| Sol #3 |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  |  |  |  |
| Sol #6 |  |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  |  |
| Sol #8 |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |
| Sol #11 |  |  | x |  |  |  |  |  |
| Sol #12 |  |  |  |  |  |  |  |  |
| Sol #13 |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  |  |
| Sol #16 |  |  |  |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |
| Sol #20 |  |  |  |  |  |  |  |  |
| Sol #21 |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  |  |  |  |  |  |