3GPP TSG-SA6 Meeting #69	S6-254637

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025                               revision of S6-254159

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0165 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the relationship between SEALDD and NRM in Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 20, SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the SEALDD WI/SI phase, the relationship between the NRM and SEALDD server are not clear as both of them make use of the network resource control to create value-added service to 3rd party. However, there are several aspects make them distingushed from each other: - From the network point of view, both the NRM and the SEALDD are the Af instances. The NRM is not the “gate” to PCF/NEF for network resource control e.g., QoS or MBS - From the SEAL PoV, the NRM and SEALDD are equivalent and independent entity, both of them can invoke the network NB API to create their own services as long as the service are different. - As to the NRM client-NRM server and SEALDD client and SEALDD server interactions, they serve the NRM-S/SEALDD-S services towards the VAL server, the message exchanges are independent with each other even if the IEs in the message are the same or similar. However, it is possible to reuse part of the functionalities, interface, message which is already defined in other SEAL procedures with clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the relationship between SEALDD and NRM in informative Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The differences between SEALDD and NRM in terms of deployment and implementation are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {Annex X, X.1, X.2, X.3} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

Annex X (Informative):
Relationship between SEALDD and NRM

# X.1	General

The clause specifies the relationship between SEALDD and NRM.

# X.2	Deployment and implementation

From the deployment aspect, the NRM function and SEALDD function can be deployed together/co-located in the same place (e.g., the same physical server) as shown in as shown in figure X.2-2. They interact with each other via the standard interface. In some implementations, the NRM function can be implemented as part of SEALDD functions and may interact with each other via the standard interface or proprietary interface.

Figure X.2-1: A co-located deployment of SEALDD and NRM

# X.3	Business consideration

Vendors may have multiple NRM/SEALDD deployement/implementation options per their business requirements as shown in figure X.3-1:

Figure X.3-1: Different deployment/implementation options of SEALDD and NRM

Option 1- start from NRM only.

If in the initial phase, only the NRM service is desired, then a NRM is implemented and deployed. Later, if wish to extend to the SEALDD data delivery service, then a standalone SEALDD function is implemented, then the two can co-located deployed.

Option 2 - start from SEALDD only.

If in the initial phase, only the SEALDD data delivery service is desired, then a SEALDD is implemented and deployed. Later, if wish to extend to the NRM service, then a standalone NRM function is implemented, then the two can co-located deployed.

Option 3 - start from both NRM and SEALDD.

When both NRM service and SEALDD data delivery service are desired, then a SEALDD entity with an internal NRM functions are deployed and may use standard reference point or properitary implementations depending upon the deployment or business needs.

# * * * End of Change * * * *