3GPP TSG-SA WG6 Meeting #69	S6-255525

Dallas, United States 17th – 21st November 2025	(revision of S6-255166)

Source:	Samsung, China Mobile

Title:	Pseudo-CR on Solution#7 evaluation

Spec:	3GPP TR 23.700-51 (v1.1.0)

Agenda item:	9.5

Document for:	Approval

Contact:	Jaehyeon Bae (jerry.bae@samsung.com)

1. Introduction

This pCR proposes solution evaluation for solution#7.

2. Reason for Change

It is required to provide solution evaluation for the agreed solution in the study. This solution is for KI#5 and is related to following open issue:

-	Whether and how the SEALDD server can obtain UE support indication of (S)RTP Multiplexed Media Identification Information and coordinate with the SEALDD client applying specific QoS requirements received from the VAL server for each multiplexed flow.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 (v1.1.0).

* * * First Change * * * *

5.8.3	Solution evaluation

This solution is for KI#5 and is related to following open issue:

-	Whether and how the SEALDD server can obtain UE support indication of (S)RTP Multiplexed Media Identification Information and coordinate with the SEALDD client applying specific QoS requirements received from the VAL server for each multiplexed flow.

This solution considers scenarios to support differentiated QoS handling for multiplexed media for UL when the UE does not support differentiated QoS handling for multiplexed media flows.The solution provides (S)RTP multiplexed media identification information from the SEALDD server to the SEALDD client for supporting traffic identification of UL multiplexed media flows to enable the SEALDD client demultiplexes the corresponding flows and the SEALDD server sends the flows via the SEALDD-S Data transmission connection. There is no architecture impacts identified based on this solution.

Note :	Enhancing the procedure 9.12.2.1.2 of TS 23.433 or creating a new procedure similar to 9.12.2.1.2 of TS 23.433 to support traffic identification and differentiated QoS for UL multiplexed media flows when the UE does not support differentiated QoS handling for multiplexed media flows will be addressed in normative work.

* * * End of Change * * * *