3GPP TSG-SA WG6 Meeting #70	S6-255322

Dallas, United States 17th – 21st November 2025	(revision of S6-255122)

Title:	Follow-up reply LS on N6 measurement exposure

Response to:	Reply LS on N6 measurement exposure (S2-2509811/S6-255010)

Release:	3GPP Rel-20

Work Item:	TEI20, EDGEAPP_Ph3

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG2

Cc:	n/a

Contact person:	Walter Featherstone

w_featherstone@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	n/a

# 1	Overall description

SA6 thanks SA2 for their Reply LS on N6 delay measurement exposure and provides the following response:

Regarding “SA2 requires further details to decide whether and how to support N6 delay measurement exposure to SA6-defined entity (considered as AF from SA2 perspective) in Release 20. SA2 kindly asks SA6 to provide further details, e.g., scenarios, for N6 delay measurement exposure.”

SA6 acknowledges the use of N6 delay measurement by the SMF for Local UPF (re)selection. SA6’s intent is to consume these network-sourced measurements in a complementary manner at the application enablement layer, without duplicating SMF functions.

Specifically, for application enablement layer specific mechanisms (e.g., initial candidate ranking in EAS discovery selection, supporting application context relocation timing/target decisions, EEC notifications conveying the current suitability of the selected EAS) and for requesting AF traffic influence toward the selected EAS. This is without requesting or implying L-UPF (re)selection, which remains under SMF control.

Regarding “SA2 has concern on potential network topology information exposure in case where UPF related information is paired with N6 delay measurement information exposure.”

The DNAI is an opaque logical label for user plane access to a data network in which an EAS can reside; it does not disclose UPF identifiers, IP addresses, or port numbers. Therefore, exposing N6 delay measurements per DNAI constitutes exposure of performance metrics and does not disclose network topology, consistent with topology-hiding requirements. It is also consistent with how EAS profiles already associate EAS instances to DNAIs and optionally provide N6-delay measurement assistance for network monitoring).

Based on the above additional information and use-case clarification, SA6 would like to ask SA2 whether they could support exposure of N6 delay measurement information (per DNAI) to SA6-defined entities (considered as AFs by SA2) in Release 20.

# 2	Actions

To SA2:

ACTION: SA6 kindly requests SA2 to consider support for exposure of N6 measurement information.

# 3	Dates of next TSG SA WG 6 meetings

SA6#71                 9th – 13th February 2026 		Goa, India

SA6#72                 13th – 17th April 2025 		Malta