3GPP TSG-CT WG3 Meeting #143	C3-254092

# Sophia-Antipolis, France, 13 - 17 October 2025								(revision of C3-254abc)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.512 | CR | 1418 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of ATSSS capabilities determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SA2 has specified several new cases of the support for the combination of MPQUIC-IP/MPQUIC-E and ATSSS-LL. Therefore, the corresponding SMF behavior related to the details of ATSSS capabilities determination is required to be captured by the CT3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the indication from SMF to PCF that the MA PDU session may support the following ATSSS capabilities: - MPQUIC-IP/MPQUIC-E, and ATSSS-LL with Active-Standby mode in the uplink; - MPQUIC-IP/MPQUIC-E, and ATSSS-LL with any steering mode except Smallest Delay steering mode in the downlink and Active-Standby mode in the uplink. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing SA2 definition of ATSSS capabilities determination handling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not impact on OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## D.1.1 ATSSS Capabilities determination

The UE can indicate its capable of supporting one or more of:

1.	ATSSS-LL functionality with any steering mode;

2.	MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

3.	MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

4.	MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

5.	MPQUIC-E functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

6.	MPQUIC-IP functionality with any steering mode and without any ATSSS-LL functionality.

The combinations of the 6 considered capabilities lead to 18 possible UE capabilities as indicated in the columns of table D.1.1-1.

The DNN configuration on SMF can be one or more of:

1.	ATSSS-LL functionality with any steering mode;

2.	MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

3.	MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

4.	MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

5.	MPQUIC-E functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

6.	ATSSS-LL functionality with only Active-Standby steering mode.

ATSSS capabilities has been determined as:

1.	ATSSS_LL

2.	MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

3.	MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

4.	MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

5.	MPQUIC-E functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

6.	MPQUIC-IP functionality.

7.	MPTCP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

8.	MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

9.	MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

10.	MPQUIC-E functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

11.	MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

12.	MPTCP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

13.	MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

14.	MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

15.	MPTCP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with any mode;

16.	MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

17.	MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with only the Active- Standby steering mode;

18.	MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode and the ATSSS-LL functionality with any steering mode;

If the DNN configurations supports ATSSS-LL with any steering mode, the network is also configured with RTT measurements support if PMF cannot be used, or RTT measurements supported when PMF can be used.

Considering the current AtsssCapability naming rules, the identified ATSSS capabilities can be defined using the naming rules as defined in clause 5.6.3.26 as:

1.	ATSSS_LL_WITH_ASMODE_DLUL

2.	ATSSS_LL

3.	MPTCP_ATSSS_LL_WITH_ASMODE_UL

4.	MPTCP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

5.	MPTCP_ATSSS_LL_WITH_ASMODE_DLUL

6.	MPTCP_ATSSS_LL

7.	MPQUIC_ATSSS_LL_WITH_ASMODE_UL

8.	MPQUIC_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

9.	MPQUIC_ATSSS_LL_WITH_ASMODE_DLUL

10.	MPQUIC_ATSSS_LL

11.	MPTCP_MPQUIC_ATSSS_LL_WITH_ASMODE_UL

12.	MPTCP_MPQUIC_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

13.	MPTCP_MPQUIC_ATSSS_LL_WITH_ASMODE_DLUL

14.	MPTCP_MPQUIC_ATSSS_LL

15.	MPQUIC_IP

16.	MPQUIC_IP_ATSSS_LL_WITH_ASMODE_UL

17.	MPQUIC_IP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

18.	MPQUIC_IP_ATSSS_LL_WITH_ASMODE_DLUL

19.	MPQUIC_IP_ATSSS_LL

20.	MPQUIC_E

21.	MPQUIC_E_ATSSS_LL_WITH_ASMODE_UL

22.	MPQUIC_E_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

23.	MPQUIC_E_ATSSS_LL_WITH_ASMODE_DLUL

24.	MPQUIC_E_ATSSS_LL

25.	MPTCP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_UL

26.	MPTCP_MPQUIC_IP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

27.	MPTCP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_DLUL

28.	MPTCP_MPQUIC_IP_ATSSS_LL

29.	MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_UL

30.	MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

31.	MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_DLUL

32.	MPQUIC_UDP_MPQUIC_IP_ATSSS_LL

33.	MPTCP_MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_UL

34.	MPTCP_MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_EXSDMODE_DL_ASMODE_UL

35.	MPTCP_MPQUIC_UDP_MPQUIC_IP_ATSSS_LL_WITH_ASMODE_DLUL

36.	MPTCP_MPQUIC_UDP_MPQUIC_IP_ATSSS_LL

# *** End of Changes ***