3GPP TSG-CT WG1 Meeting #157	C1-256317

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.283 | CR | 0004 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN on anyAreaChange. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMCLoc |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolving EN on anyAreaChange. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR propose to remove the attribute anyAreaChange. The usage of this leads to a non deterministic behaviour of the LMC that reports the location information. It is not clear what an any area change means. A MC UE can move 1 mm and consider that as an any area change. It is still possible for the LMC to report the location information when the client enters or leaves a specific area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear usage of the attribute anyAreaChange. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2.10, 7.1.6.2.35, 7.1.6.2.39, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.1.6.2.10	Type: GeographicalAreaChange

Table 7.1.6.2.10-1: Definition of type GeographicalAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| enterSpecificArea | SpecificArea | O | 0..1 | A geographical area which triggers location reporting when entering or exiting the defined geographical area. |  |
| exitSpecificArea | SpecificArea | O | 0..1 | A geographical area which triggers location reporting when entering or exiting the defined geographical area. |  |

* * * Next Change * * * *

##### 7.1.6.2.35	Void

* * * Next Change * * * *

##### 7.1.6.2.39	Void

* * * Forth Change * * * *

# A.2	LMS_Reporting API

openapi: 3.0.0

info:

title: LMS_Reporting

version: 1.0.0-alpha.8

description: |

API for Registration and location reporting to LMS Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 24.283 V2.0.0; Mission Critical Location Management (MCLoc).

url: http://www.3gpp.org/ftp/Specs/archive/24_series/24.283/

servers:

- url: '{apiRoot}/lms-reporting/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 8.2.44D1 of 3GPP TS 24.483

security:

- BearerAuth: []

paths:

/registrations:

post:

description: Creates a new indiviual LMC registration in the LMS.

security:

- BearerAuth: []

operationId: registerLMC

tags:

- LMS Registration (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

responses:

'201':

description: Successful regitration of LMC.

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

locationConfiguration:

'{request.body#/locationConfigUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationConfiguration'

responses:

'204':

description: No Content (The receipt of the Configuration is acknowledged).

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

locationRequest:

'{request.body#/locationRequestUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationRequest'

responses:

'200':

description: Location report.

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

put:

description: >

Updates an existing LMC registration in the LMS

tags:

- Individual LMS registration (Document)

parameters:

- name: registrationId

in: path

description: Identifies an individual LMC registration resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

responses:

'200':

description: >

OK. The individual LMC registration resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

'204':

description: No Content (updated successfully).

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an existing LMC reigstration identified by the registrationId.

operationId: DeleteLMCRegistration

tags:

- Individual LMC registration (Document)

parameters:

- name: registrationId

in: path

description: Identifies an LMC registration resource.

required: true

schema:

type: string

responses:

'204':

description: >

An individual LMC registration resource deleted successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/locationreports:

post:

summary: submit a MC UE location report

security:

- BearerAuth: []

operationId: submitLocationReport

tags:

- MC UE Location Report (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

responses:

'201':

description: Successful submission of location report.

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/configurations:

get:

summary: request to download the location configuration

security:

- BearerAuth: []

operationId: ConfigurationRequest

tags:

- LMC configurations (Collection)

responses:

'200':

description: Successful submission of location report.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationConfiguration'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

BearerAuth:

type: http

scheme: bearer

# API specific definitions

schemas:

LMCRegistration:

description: Resprestents an individual LMS registration resource.

type: object

properties:

expTime:

type: string

format: date-time

mclocClientID:

type: string

description: Identify the LMC of the MC UE

locationConfigUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

locationRequestUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

required:

- mclocClientID

- expTime

- locationConfigUri

- locationRequestUri

LocationConfiguration:

description: Represents an location configuraion objects sent from LMS to an LMC

type: object

properties:

emergencyTriggeringCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

emergencyLocationInformation:

$ref: '#/components/schemas/RequestedLocationInfo'

triggeringCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

nonEmergencyLocationInformation:

$ref: '#/components/schemas/RequestedLocationInfo'

mcServiceId:

type: string

format: Uri

description: Optional attribute that is used to indicate the identity of the

MC user requesting to set the location configuration.

configScope:

$ref: '#/components/schemas/ConfigScope'

historyReporting:

type: boolean

description: The attribute indicates that the LMC shall store location

information if location reporting can not be done, for example if the MC UE is out of

coverage. The stored location information can be reported at a later time.

TriggeringCriteria:

description: Contains the location reporting trigger criteria.

type: object

properties:

adaptiveTrigger:

$ref: '#/components/schemas/AdaptiveTrigger'

cellChange:

$ref: '#/components/schemas/CellChange'

geographicalAreaChange:

$ref: '#/components/schemas/GeographicalAreaChange'

mbsfnAreaChange:

$ref: '#/components/schemas/MbsfnAreaChange'

mbmsSaChange:

$ref: '#/components/schemas/MbmsSaChange'

nr5GMbsfsaAreaChange:

$ref: '#/components/schemas/NR5GMbsfsaAreaChange'

periodicReport:

$ref: '#/components/schemas/PeriodicReport'

plmnChange:

$ref: '#/components/schemas/PlmnChange'

ratTypeChange:

$ref: '#/components/schemas/RatTypeChange'

signallingEvent:

$ref: '#/components/schemas/SignallingEvent'

trackingAreaChange:

$ref: '#/components/schemas/TrackingAreaChange'

travelledDistance:

$ref: '#/components/schemas/TravelledDistance'

AdaptiveTrigger:

description: This attribute specifies parameters controlling adaptive combined time

and distance triggers for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

minPeriod:

type: integer

description: An optional attribute specified as a positive or 0 integer number

of seconds and defaulting at 0, if not present. This attribute specifies the minimum

wait period between consecutive location reports, irrespective of changes in the

distance between the current location and the location of the most recent sending of a

location report.

minDistance:

type: integer

description: An optional attribute specified as a strictly positive integer number of

meters and defaulting at infinity, if not present. This attribute is used in the

decision of sending a location report based on travelled distance, and specifies the

minimum required distance, between the current location and the location of the most

recent sending of a location report.

persistencePeriod:

type: integer

description: An optional attribute specified as a positive or 0 integer number of

seconds and defaulting at 0, if not present. This attribute specifies the time between

the moment when the LMC detected that the distance between the current location and

the location of the most recent sending of a location report exceeded the minDistance

attribute and the moment when the LMC will check again that the updated current

location is still farther away by at least minDistance from the location of the

mentioned sending of the location report. If the check is positive, a location report

based on travelled distance will be sent.

additionalTime:

type: integer

description: An optional attribute, specified as a strictly positive integer number of

seconds and defaulting at 1. If a location report is not sent based on the travelled

distance, the LMC will send a location report after minPeriod + PersistencePeriod

+ AdditionalTime seconds after the previous location report.

rrcInactiveMinPeriod:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactiveMinDistance:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactivePersistencePeriod:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactiveadditionalTime:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

CellChange:

description: The attribute specifies which cell changes trigger location reporting.

type: object

properties:

anyCellChange:

$ref: '#/components/schemas/AnyCellChange'

enterSpecificCell:

$ref: '#/components/schemas/SpecificCellChange'

exitSpecificCell:

$ref: '#/components/schemas/SpecificCellChange'

AnyCellChange:

description: The presence of this attribute specifies that any cell change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificCellChange:

description: A list of cell Ids which triggers location reporting when

entering or exiting the cells.

type: object

properties:

eutranCells:

$ref: '#/components/schemas/EutranCells'

nrCells:

$ref: '#/components/schemas/NrCells'

anyOf:

- required: [eutranCells]

- required: [nrCells]

EutranCells:

description: A list of Eutran cell Ids which triggers location reporting when

entering or exiting the cells.

type: array

items:

$ref: '#/components/schemas/EutranCell'

NrCells:

description: A list of NR cell Ids which triggers location reporting when

entering or exiting the cells.

type: array

items:

$ref: '#/components/schemas/NrCell'

EutranCell:

description: An Eutran cell id that triggers a location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

eutranCell:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

NrCell:

description: A NR cell id that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

nrCell:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

GeographicalAreaChange:

description: The attribute specifies which geographical area changes that

trigger location reporting.

type: object

properties:

enterSpecifcArea:

$ref: '#/components/schemas/SpecificArea'

exitSpecifcArea:

$ref: '#/components/schemas/SpecificArea'

SpecificArea:

description: A geographical area which triggers location reporting when

entering or exiting the defined geographical area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

MbsfnAreaChange:

description: The attribute specifies which MBSFN area changes trigger

location reporting.

type: object

properties:

anyMbfsnAreaChange:

$ref: '#/components/schemas/AnyMbsfnAreaChange'

enterSpecificMbsfnArea:

$ref: '#/components/schemas/SpecificMbsfnArea'

exitSpecificMbsfnArea:

$ref: '#/components/schemas/SpecificMbsfnArea'

AnyMbsfnAreaChange:

description: The precense of this attribute specifies that any MBSFN area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificMbsfnArea:

description: A list of MBSFN area Ids that triggers location reporting when

entering or exiting the MBSFN area Ids.

type: object

properties:

mbsfnAreas:

type: array

items:

$ref: '#/components/schemas/MbsfnAreaId'

MbsfnAreaId:

description: An MBFSN area id that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbsfnArea:

type: integer

minimum: 0

maximum: 255

description: The MBSFN area identity.

MbmsSaChange:

description: The attribute specifies which MBMS SA area changes trigger

location reporting.

type: object

properties:

anyMbmsSaChange:

$ref: '#/components/schemas/AnyMbmsSaChange'

enterSpecificMbmsSa:

$ref: '#/components/schemas/SpecificMbmsSaIdentity'

exitSpecificMbmsSa:

$ref: '#/components/schemas/SpecificMbmsSaIdentity'

AnyMbmsSaChange:

description: The presence of this element specifies that any MBMS SA change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificMbmsSaIdentity:

description: A list of MBMS SAI that triggers location reporting when

entering or exiting the MBMS SAI.

type: object

properties:

mbmsSaIdentities:

type: array

items:

$ref: '#/components/schemas/MbmsSaIdentity'

MbmsSaIdentity:

description: An MBMS SA identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbmsSaIdentity:

type: integer

minimum: 0

maximum: 65535

description: The MBMS SA identity.

NR5GMbsfsaAreaChange:

description: The attribute specifies which 5G Mbsfsa area changes trigger

location reporting.

type: object

properties:

anyNR5GMbsfsaAreaChange:

$ref: '#/components/schemas/AnyNR5GMbsfsaAreaChange'

enterSpecificNR5GMbsfsaArea:

$ref: '#/components/schemas/SpecificNR5GMbsfsaArea'

exitSpecificNR5GMbsfsaArea:

$ref: '#/components/schemas/SpecificNR5GMbsfsaArea'

AnyNR5GMbsfsaAreaChange:

description: The presence of this element specifies that any 5G Mbsfsa area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificNR5GMbsfsaArea:

description: A list of 5G Mbsfsa area that triggers a location reporting when

entering or exiting the 5G Mbsfsa area.

type: object

properties:

nr5GMbsFsaIdentities:

type: array

items:

$ref: '#/components/schemas/NR5GMbsfsaAreaIdentity'

NR5GMbsfsaAreaIdentity:

description: An 5G Mbsfsa area identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbsfnAreaIdentity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

PeriodicReport:

description: The attribute specifies how frequent periodic location reports

shall be sent.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

periodicReport:

type: integer

description: Value in seconds specifies the reporting interval.

PlmnChange:

description: The attribute specifies which PLMN changes trigger

location reporting.

type: object

properties:

anyPlmnChange:

$ref: '#/components/schemas/AnyPlmnChange'

enterSpecificPlmnArea:

$ref: '#/components/schemas/SpecificPlmnChange'

exitSpecificPlmnArea:

$ref: '#/components/schemas/SpecificPlmnChange'

AnyPlmnChange:

description: The presence of this element specifies that any PLMN change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificPlmnChange:

description: A list of PLMNs that triggers a location reporting when

entering or exiting the PLMN.

type: object

properties:

plmnIds:

type: array

items:

$ref: '#/components/schemas/Plmn'

Plmn:

description: A PLMN identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

RatTypeChange:

description: An optional attribute specifying what inter-RAT changes triggers

a location report.

type: object

properties:

anyRatTypeChange:

$ref: '#/components/schemas/AnyRatTypeChange'

AnyRatTypeChange:

description: This attribute specifies that any inter-RAT change triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SignallingEvent:

description: An optional attribute specifying what signalling events triggers

a location report.

type: object

properties:

functionalAliasActivation:

$ref: '#/components/schemas/FunctionalAliasActivation'

functionalAliasDeactivation:

$ref: '#/components/schemas/FunctionalAliasDeactivation'

initialLogOn:

$ref: '#/components/schemas/InitialLogOn'

groupCallNonEmergency:

$ref: '#/components/schemas/GroupCallNonEmergency'

privateCallNonEmergency:

$ref: '#/components/schemas/PrivateCallNonEmergency'

locationConfigurationReceived:

$ref: '#/components/schemas/LocationConfigurationReceived'

FunctionalAliasActivation:

description: This attribute specifies that a functional alias activation

triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

FunctionalAliasDeactivation:

description: This attribute specifies that a functional Alias deactivation

triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

InitialLogOn:

description: This attribute specifies that an initial log on triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

GroupCallNonEmergency:

description: This attribute specifies that a non-emergency group call triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

PrivateCallNonEmergency:

description: This attribute specifies that a non-emergency private call triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

LocationConfigurationReceived:

description: This attribute specifies that a received location configuration triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

TrackingAreaChange:

description: The attribute specifies which tracking area changes trigger

location reporting.

type: object

properties:

anyTrackingAreaChange:

$ref: '#/components/schemas/AnyTrackingAreaChange'

enterSpecificTrackingArea:

$ref: '#/components/schemas/SpecificTrackingAreaChange'

exitSpecificTrackingArea:

$ref: '#/components/schemas/SpecificTrackingAreaChange'

AnyTrackingAreaChange:

description: The presence of this element specifies that any tracking area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificTrackingAreaChange:

description: A list of tracking areas that triggers a location reporting when

entering or exiting the tracking area.

type: object

properties:

trackingAreas:

type: array

items:

$ref: '#/components/schemas/TrackingArea'

TrackingArea:

description: A tracking area identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

trackingArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tac'

## It is FFS if a specific format for Eutran tracking area id is needed

TravelledDistance:

description: The attribute specifies the required travelled distance for a

trigger for location reporting.

shall be sent.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

travelledDistance:

type: integer

description: Value in meter specifies the travelled distance

for location reporting.

RequestedLocationInfo:

description: Contains the details of what the location report shall include.

type: object

properties:

bearingAndSpeed:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

coordinate:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

mbmsSaiId:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

mbsfnArea:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

minimumIntervalLength:

type: integer

description: Value in seconds, the minimum time between location reports.

neigbouringEcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

neigbouringNcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

nr5GmbsfsaArea:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

servingEcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

servingNcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

TriggerId:

type: string

description: The triggerId shall be set to a unique value and is

returned in the location report.

LocationRequest:

description: Represents an location request object used by LMS to

request a location report from an LMC.

type: object

properties:

requestedMcServiceId:

type: string

format: uri

description: The MC service ID of the MC user being requested to send a

location report.

requestingMcServiceId:

type: string

format: uri

description: The MC service ID of the MC user from which location information

is being requested.

requestedFunctionalAliasId:

type: string

format: uri

description: The activated functional alias ID of the MC user being requested

to send a location report.

requestingFunctionalAliasId:

type: string

format: uri

description: The activated functional alias ID of the MC user from which location

information is being requested.

requestedLocationInfo:

$ref: '#/components/schemas/RequestedLocationInfo'

requestLocationHistory:

$ref: '#/components/schemas/RequestedLocationHistory'

requestHistoryStatus:

type: boolean

description: This attributes indicates that a location history status report is requested.

ConfigScope:

description: Indicates if the location configuration shall be updated fully and replace

existing configration, or if the location configuration is added to the exsting.

anyOf:

- type: string

enum:

- FULL

- UPDATE

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

RequestedLocationHistory:

description: Includes details on requested location history information

type: object

properties:

numberOfReports:

type: integer

description: Indicates the number of requested stored reports.

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

minimumIntervalLength:

type: integer

description: Value in seconds, the minimum time between location reports.

MCUELocation:

description: Describes the parameters to perform LMS Location reports.

type: object

properties:

functionalAlias:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

interRatType:

$ref: '#/components/schemas/InterRatType'

locationData:

$ref: '#/components/schemas/LocationInfo'

locTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

mclocClientId:

$ref: '#/components/schemas/MclocClientId'

mcServiceUELabel:

type: string

description: Used to identify the reporting MC UE for the end users, this may be used

when multiple MC UEs are used by the same MC service user.

reportId:

type: string

description: Attribute is used to return the value in the RequestId attribute

in the an Location Request.

reportType:

$ref: '#/components/schemas/ReportType'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

locationHistoryData:

$ref: '#/components/schemas/LocationHistoryData'

locationHistoryStatus:

$ref: '#/components/schemas/LocationHistoryStatus'

MclocClientId:

type: string

format: uuid

description: Used to identify the reporting LMC, this is required when multiple location

information responses for different requested LMC are sent.

LocationInfo:

description: object that includes coordinates typically obtained from GPS.

type: object

properties:

currentCoordinate:

$ref: '#/components/schemas/CurrentCoordinate'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

mbmsSaiId:

type: integer

minimum: 0

maximum: 65535

description: Attribute containing the MBMS Service Area Id the LMC is located in.

mbsfnArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsfnArea'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

neighbouringEcgi:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

neighbouringNcgi:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

nr5GMbsfsaArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

CurrentCoordinate:

description: This object contains one specific location point, speed and bearing (e.g.,

received from a GPS reciever).

type: object

properties:

altitude:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

bearingAndSpeed:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/HorizontalWithVerticalVelocity'

geographicalCoordinate:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicalCoordinates'

horizontalaccuracy:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Accuracy'

verticalaccuracy:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Accuracy'

LocationHistoryData:

description: This object contains one or multiple time stamped location information reports.

type: array

items:

$ref: '#/components/schemas/LocationHistoryEvent'

LocationHistoryEvent:

description: This object respresents a single location information report that has been

stored in the LMC.

type: object

properties:

locationData:

$ref: '#/components/schemas/LocationInfo'

locTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

LocationHistoryStatus:

description: This object respresents Location history summary status of the location reports

that has been stored in the LMC and has not yet been reporeted.

type: object

properties:

storedReports:

type: integer

description: set to the number of stored reports.

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

#

# Simple Types:

#

#

# Enumeration data types

#

InterRatType:

description: Attribute containing the inter-RAT change type.

anyOf:

- type: string

enum:

- 5G_MBS_TO_LTE_MBMS

- 5G_MBS_TO_LTE_UNICAST

- LTE_MBMS_TO_5G_MBS

- LTE_MBMS_TO_5G_UNICAST

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

ReportType:

description: Used to inform whether the LMC is sending the report in an emergency

situation or not.

anyOf:

- type: string

enum:

- EMERGENCY

- NONEMERGENCY

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

* * * End of Changes * * * *