3GPP TSG-CT1 Meeting #157	C1-256799

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0168 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Discovery enhancements to support N6 tunnel and E2E tunnel. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR#0655(see S6-243697) agreed in SA6#62 proposed to 3GPP TS 23.558, have enhancments to support the N6 tunnel and E2E tunnel in EAS Discovery procedure. TS 24.558 needs to be aligned with the above SA6 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes updates the EAS Discovery service operation, information elements and the Eees_EASDiscovery Open API. Backward Compatibility: The changes proposed are backward compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2.2, 6.3.5.1, 6.3.5.2.2, 6.3.7, A.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the API in TS24558_Eees_EASDiscovery.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 5.3.2.2.2	EEC requesting EAS discovery information using Eees_EASDiscovery_EasDiscRequest operation

To request for one-time EAS discovery, the EEC shall send an HTTP POST request to the EES as specified in clause 6.3.2.4.4. The body of the POST message shall include the EasDiscoveryReq data structure as specified in clause 6.3.5.2.2.

Upon reception of the HTTP POST message from the EEC, the EES shall:

a)	process the EAS discovery request information;

b)	the EES verifies and checks if the EEC is authorized to discover the requested EAS(s) from EES;

c)	if EEC is authorized to discover the requested EAS(s) from EES, the EEC is not registered with the EES, and the ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall reject the request by sending an HTTP "403 Forbidden" status code to the EEC including the ProblemDetails data structure with the "cause" attribute containing the "REGISTRATION_REQUIRED" application error;

d)	if the EEC is authorized to discover the requested EAS(s) from EES and the EEC is registered as required by the ECSP policy, then the EES;

1)	may obtain the UE's location as specified in clause 5.3 of 3GPP TS 29.122 [3];

2)	if EAS discovery filters are provided by the EEC without the "appGroupProfile" attribute, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location, and if the enNB1 feature is supported, the "userLocation" attribute may be provided in the "locInf" attribute within the EasDiscoveryReq data type;

i.	if the Metaverse_App feature is supported and if the request includes the "associatedDevices" attribute, the EES shall identify those EAS(s) that serves these associated device types;

3)	if the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES identifies the EAS(s) who supports at least one of the ACR scenarios as indicated by EEC; and

i.	the EES may select one EAS and determine to perform application traffic influence for this EAS in advance based on AC's service KPI or EAS(s) service KPI in desired response time, when the EES does not perform traffic influence in advance; and

ii.	if the EdgeApp_3 feature is supported and if the EES performs the traffic influence for the EAS in advance by triggering the EAS traffic influence procedure as specified in clause 5.15 of 3GPP TS 29.558 [4], then the EES shall provide indication by including the "trafficInfluenceInd" attribute in the DiscoveredEas data structure as part of the EAS discovery response;

4)	if EAS discovery filters are not provided:

i.	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location; and

ii.	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

5)	if the EdgeApp_2 feature is supported and if the "appGroupProfile" attribute is provided within the EasDiscoveryFilter data type, the EES determines the availability of common EAS corresponding to the "appGrpId" attribute. If the common EAS is:

i)	not available, then based on the policy if the EES needs to select the common EAS, the EES shall identify an EAS matching the received "appGrpId" attribute based on the EAS discovery filters or the ECSP policy. The EES shall store the common EAS information and related "appGrpId" attribute by triggering the Eecs_EASInfoManagement API as specified in clause A.16 of 3GPP TS 29.558 [4] and EES shall trigger common EAS announcement procedure as specified in clause 5.14 of 3GPP TS 29.558 [4];

ii)	available at the EES, then the EES provides information of that EAS as part for EAS discovery response; and

NOTE 1:	The EES could have previously determined and stored the common EAS for application group ID, or the EES may have received the common EAS selection information for application group ID during the common EAS announcement procedure as specified in clause 5.14 of 3GPP TS 29.558 [4].

iii)	if the EdgeApp_3 feature is supported and if the "e2eRespTime" attribute is provided within the appGroupProfile data type. The EES determines if the UE is present in:

A)	the overlapping area between the EDNs and identifies the common EAS or those registered EAS with the service KPIs matcing the "service KPI" attribute provided within the EasDiscoveryFilter data type; or

B)	the non-overlapping area, the EES checks with partner ECS for the common EAS as specified in clause 6.4.2.2 of 3GPP TS 29.558 [4] and if there is no common EAS available, then the EES identifies the EAS for the application group ID with the service KPIs matcing the "service KPI" attribute provided within the EasDiscoveryFilter data type and initiates the common EAS announcement procedure as specified in clause 5.14 of 3GPP TS 29.558 [4] towards the partner ECS;

6)	the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs;

7)	if the EdgeApp_2 feature is supported and the EEC indicates the EAS selection support within the "easSelSupInd" attribute, the EES shall select one matching EAS that fulfils the EEC requirements as described in bullets 2), 3) and 4), and the EES shall provide the selected EAS information to the EEC within the "discoveredEas" attribute; and

8)	if the EdgeApp_2_Ext1 feature is supported and the EEC provided the "tunnel_Info" attribute, then the EES consider this tunnel information to identify the EAS(s) else the EES may obtain the N6 tunnel information by invoking NEF API using the 3GPP core network capabilities to identify the EAS(s) close to the EEC for optimal N6 routing;

e)	if the EdgeApp_2 feature is supported, and:

1)	if the EEC indicates EAS Instantiation Triggering using "easIntTrigSup" attribute with the value set to true in the EAS discovery request, the EES may trigger the ECSP to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) and the EES supports such capability. If the "easIntTrigSup" attribute is omitted or set to value false the EES shall not trigger the ECSP to instantiate the EAS and the EES may determine instantiable EAS information using "easInstInfos" attribute, which is provided in the EAS discovery response, for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs; and

2)	if the EEC indicates the predicted expiration time by which the UE reaches location using the "predictExpTime" attribute, the EES may also collect edge load analytics from the ADAES (as specified in clause 8.8.2 of 3GPP TS 23.436 [9]) or performance data from the OAM to find whether the EAS(s) satisfies the expected AC service KPIs or the minimum required AC service KPIs; and

NOTE 2:	How EEC uses the analytics information is implementation specific.

3)	if the EEC indicates the UEs serving MNO information as part of the "servingPLMNInfo" attribute, the EES identifies the EAS(s) that matches the allowed MNO information in their EAS profiles and UE's serving MNO information as specified in clause 6.3.5.2.3; and

f)	if the processing of the request was successful, the EES sends an EAS discovery response to the EEC as specified in clause 6.3.2.4.4.2.2, which includes information about the discovered EASs. The response shall include endpoint information for discovered EASs. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. If the discovered EAS is registered to another EES, then the endpoint information of the EAS shall be included in the "eesEndPt" attribute within the DiscoveredEas data type.

If the successful processing of the request does not result in finding a matching EAS (i.e. there is no client side error), the EES responds with "204 No Content" HTTP status code. Otherwise, the EES shall reject the EAS discovery request and respond with an appropriate failure HTTP status code.

The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat this procedure. If the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC shall send the EAS information provisioning request for EAS selection indicating the selected EAS ID. If the "lifeTime" attribute is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

If the failure response is received for the EAS discovery request, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated "REGISTRATION_REQUIRED" error in the "cause" attribute of the "ProblemDetails" structure, the EEC shall perform an EEC registration as specified in clause 5.2.2.2.2 before resending the EAS discovery request.

# *** Next Change ***

#### 6.3.5.1	General

This clause specifies the application data model supported by the Eees_EASDiscovery API.

Table 6.3.5.1-1 specifies the data types defined specifically for the Eees_EASDiscovery API service.

Table 6.3.5.1-1: Eees_EASDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ACCharacteristics | 6.3.5.2.11 | Represents EAS dynamic information changes filter. |  |
| DiscoveredEas | 6.3.5.2.8 | Represents an EAS discovery information. |  |
| EasCharacteristics | 6.3.5.2.7 | Represents the EAS chararcteristics. |  |
| EASDiscEventIDs | 6.3.5.3.3 | Represents the EAS Discovery Subscription events. |  |
| EasDiscoveryFilter | 6.3.5.2.6 | Represents the EAS characteristics. |  |
| EasDiscoveryNotification | 6.3.5.2.5 | Notification of EAS discovery information. |  |
| EasDiscoveryReq | 6.3.5.2.2 | Contains the EAS discovery request information |  |
| EasDiscoveryResp | 6.3.5.2.3 | Represents the EAS discovery response. |  |
| EasDiscoverySubscription | 6.3.5.2.4 | Represents an Individual EAS Discovery Subscription resource. |  |
| EasDiscoverySubscriptionPatch | 6.3.5.2.12 | Represents an updated Individual EAS Discovery Subscription resource. |  |
| EasDynamicInfoFilter | 6.3.5.2.9 | Represents EAS dynamic information changes filter. |  |
| EasDynamicInfoFilterData | 6.3.5.2.10 | Represents an EAS dynamic information. |  |
| EdgeLoadAnalytic | 6.3.5.2.14 | Contains the statistical analytics data and predictive analytics data. |  |
| PredictiveData | 6.3.5.2.15 | Contains the predictive analytics data for each discovered EAS service change. |  |
| RequestorId | 6.3.5.2.13 | Represents identifier of the requestor. |  |
| StatisticalData | 6.3.5.2.16 | Contains the statistical analytics data. |  |

Table 6.3.5.1-2 specifies data types re-used by the Eees_EASDiscovery API service.

Table 6.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACProfile | Clause 6.2.5.2.3 |  |  |
| ACRScenario | 3GPP TS 29.558 [4] |  |  |
| AppGroupProfile | Clause 8.1.5.2.13 | Represents the application group profile used for Common EAS. | EdgeApp_2 |
| DateTime | 3GPP TS 29.122 [3] |  |  |
| Dnai | 3GPP TS 29.571 [5] |  |  |
| DurationSec | 3GPP TS 29.122 [3] |  |  |
| EASBundleInfo | 3GPP TS 29.558 [4] | Represents EAS bundle information. | EdgeApp_2 |
| EASCategory | 3GPP TS 29.558 [4] | Represents the EAS type. |  |
| EASInstantiationInfo | 3GPP TS 29.558 [4] |  | EdgeApp_2 |
| EASProfile | 3GPP TS 29.558 [4] |  |  |
| EndPoint | 3GPP TS 29.558 [4] |  |  |
| Gpsi | 3GPP TS 29.571 [5] | Used to identify a UE. |  |
| LocationArea5G | 3GPP TS 29.122 [3] |  |  |
| LocationInfo | 3GPP TS 29.122 [3] |  |  |
| PlmnIdNid | 3GPP TS 29.571 [5] | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). | EdgeApp_2 |
| Port | 3GPP TS 29.122 [3] | Identifies a port, unsigned integer with valid values between 0 and 65535. | EdgeApp_3 |
| RouteToLocation | 3GPP TS 29.571 [5] |  |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Represents the scheduled communication time. | EdgeApp_2 |
| SupportedFeatures | 3GPP TS 29.571 [5] |  |  |
| TimeWindow | 3GPP TS 29.122 [3] |  |  |
| TunnelInfo | 3GPP TS 29.558 [4] | Represents the tunnel information. | EdgeApp_2_Ext1 |
| Uinteger | 3GPP TS 29.571 [5] | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. | EdgeApp_2 |
| Uri | 3GPP TS 29.122 [3] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

# *** Next Change ***

##### 6.3.5.2.2	Type: EasDiscoveryReq

Table 6.3.5.2.2-1: Definition of type EasDiscoveryReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | RequestorId | M | 1 | Represents a unique identifier of the requestor (e.g. EEC, EAS, EES). |  |
| ueId | Gpsi | O | 0..1 | Represents the identifier of the UE. |  |
| easDiscoveryFilter | EasDiscoveryFilter | O | 0..1 | Contains EAS characteristics |  |
| eecSvcContinuity | array(ACRScenario) | O | 1..N | Contains service continuity support; indicates EEC supported ACR scenarios. If this attribute is not present, then the EEC does not support service continuity. |  |
| eesSvcContinuity | array(ACRScenario) | O | 1..N | Contains service continuity support; indicates EES supported ACR scenarios. If this attribute is not present, then the EES does not support service continuity. |  |
| easSvcContinuity | array(ACRScenario) | O | 1..N | Contains service continuity support; indicates EAS supported ACR scenarios. If this attribute is not present, then the EAS does not support service continuity. |  |
| locInf | LocationInfo | O | 0..1 | Represents location information of the UE. |  |
| easTDnai | Dnai | O | 0..1 | Contains the target DNAI information which can be associated with potential target-EAS(s) |  |
| easSelSupInd | boolean | O | 0..1 | Indicates if the EEC requires the EAS selection support from the EES (e.g., for constrained device). "true": the EAS selection is required from the EES. "false" (default): the EAS selection is not required from the EES. | EdgeApp_2 |
| suppFeat | SupportedFeatures | C | 0..1 | Represents a list of Supported features used as described in clause 6.3.7. Shall be present in the HTTP POST request/response. |  |
| easIntTrigSup | boolean | O | 0..1 | Indicates to the EES whether the EAS instantiation triggering should be performed for the current request. "false" (default): the EAS instantiation triggering should not be performed. "true": the EAS instantiation triggering should be performed. If the attribute is omitted, then it default value is "false". | EdgeApp_2 |
| predictExpTime | DateTime | O | 0..1 | Represents the predicted expiration time by which the UE reaches location. It is used by the EES as analytics input to get edge load analytics information from the ADAES service as described in clause 8.8.2 of 3GPP TS 23.436 [9]. | EdgeApp_2 |
| servingPLMNInfo | PlmnIdNid | C | 0..1 | Represents the serving PLMN information (e.g. PLMN ID) which is serving the subscriber. (NOTE 1) | EdgeApp_2 |
| svcContinuityPlanInd | boolean | O | 0..1 | Indicates to the EES whether the EAS discovery request is triggered as part of service continuity planning. "true": this request is part of service continuity planning. "false" (default): this request is not of part service continuity planning. (NOTE 2) | EdgeApp_2 |
| appGrpUeIds | array(Gpsi) | C | 1..N | Contains a list of UE identifier(s) in the application group. (NOTE 3) | EdgeApp_3 |
| tunnel_Info | TunnelInfo | O | 0..1 | Contains the target tunnel information. | EdgeApp_2_Ext1 |
| NOTE 1: This IE shall be included if edge node sharing is used. NOTE 2: This attribute is used by EAS when invoking T-EAS discovery procedure. NOTE 3: The "appGrpUeIds" attribute shall be included if the EAS discovery request is triggered by the common S-EAS or S-EES due to overload or maintenance reason. |  |  |  |  |  |

Editor’s Note: Representing the MNO name in "EasDiscoveryReq" type needs to be clarified by stage-2.

# *** Next Change ***

### 6.3.7	Feature negotiation

General feature negotiation procedures are described in clause 7.8 of 3GPP TS 29.558 [4]. Table 6.3.7-1 lists the supported features for Eees_EASDiscovery API.

Table 6.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6 of 3GPP TS 29.558 [4]. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6 of 3GPP TS 29.558 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 3 | enNB1 | This feature indicates the support of the support of enhancements to this northbound API in Rel-18. |
| 4 | EdgeApp_2 | This feature indicates support of the enhancements for the Enabling Edge Applications. Within this feature the following enhancements are covered: - support of constrained devices for Edge (e.g. support of the EEC with Reduced Capabilities); - support of the EAS instantiation triggering; - support of the EAS synchronization; - support of EAS bundle information; - support for predicted/expected UE location or Geographical service area during service continuity; - support for common EAS enhancement; and - obtaining edge load analytics information. |
| 5 | EdgeApp_3 | This feature indicates support of the second set of the enhancements for the Enabling Edge Applications. Within this feature the following enhancements are covered: - support for port number where the EEC shall receive the triggers from EES; - support of common EAS discovery with e2e latency; and - support of common EAS relocation. |
| 6 | EdgeApp_2_Ext1 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature, the following enhancements are covered: - Support of N6 and E2E tunnel in the EAS Discovery service. |

# *** Next Change ***

# A.3	Eees_EASDiscovery API

openapi: 3.0.0

info:

title: Eees_EASDiscovery

description: |

API for EAS Discovery.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.4"

externalDocs:

description: >

3GPP TS 24.558 V19.4.0 Enabling Edge Applications; Protocol specification.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-easdiscovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558.

paths:

/subscriptions:

post:

description: Creates a new individual EAS discovery subscription.

operationId: CreateEASDiscSub

tags:

- EAS Discovery Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

responses:

'201':

description: >

Created. A new Individual EAS Discovery Subscription resource was successfully

created.

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryNotification'

responses:

'204':

description: No Content (The receipt of the Notification is acknowledged).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

description: >

Updates an existing individual EAS discovery subscription identified by the subscriptionId.

operationId: UpdateIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

responses:

'200':

description: >

OK. The individual EAS discovery subscription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

'204':

description: No Content (updated successfully).

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an existing individual EAS discovery subscription identified by the subscriptionId.

operationId: DeleteIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

An individual EAS discovery subscription resource deleted successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Partial update an existing EAS Discovery Subscription resource identified by a

subscriptionId.

operationId: ModifyIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscriptionPatch'

responses:

'200':

description: >

OK (An individual EAS discovery subscription resource updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

'204':

description: No Content (modified successfully).

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/eas-profiles/request-discovery:

post:

description: >

Provides EAS information requested by the service consumer (i.e. EEC, EAS or EES).

operationId: GetEASDiscInfo

tags:

- EAS Profiles (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryReq'

responses:

'200':

description: >

OK (The requested EAS discovery information was returned successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EasDiscoveryReq:

description: EAS discovery request information.

type: object

properties:

requestorId:

$ref: '#/components/schemas/RequestorId'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

eecSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

eesSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EES supports service continuity or not, also indicates which ACR

scenarios are supported by the EES.

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EAS supports service continuity or not, also indicates which ACR

scenarios are supported by the EAS.

locInf:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

easTDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

easSelSupInd:

type: boolean

description: >

Indicates if the EEC requires the EAS selection support from the EES (e.g., for

constrained device). The true value indicate the EAS selection is required from the

EES. The default value false indicates the EAS selection is not required from the EES.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

easIntTrigSup:

type: boolean

description: >

Indicates to the EES whether the EAS instantiation triggering should be performed for

the current request. The default value false indicates the EAS instantiation triggering

should not be performed. The true value indicate the EAS instantiation triggering should

be performed.

predictExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

servingPLMNInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

svcContinuityPlanInd:

type: boolean

description: >

Indicates to the EES whether the EAS discovery request is triggered as part of service

continuity planning. The default value false indicates this request is not part of

service continuity planning. The true value indicate this request is part of

service continuity planning.

appGrpUeIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

description: Contains a list of UE identifier(s) in the application group.

tunnel_Info:

$ref: 'TS29558_Eecs_TargetEESDiscovery.yaml#/components/schemas/TunnelInfo'

required:

- requestorId

EasDiscoveryResp:

description: EAS discovery response.

type: object

properties:

discoveredEas:

type: array

items:

$ref: '#/components/schemas/DiscoveredEas'

description: List of EAS discovery information.

easInstInfos:

type: object

additionalProperties:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EASInstantiationInfo'

minProperties: 1

description: >

Contains the EAS instantiation information for each discovered EAS returned within

the discoveredEas attribute.

edgeLoadAnalytics:

type: object

additionalProperties:

$ref: '#/components/schemas/EdgeLoadAnalytic'

minProperties: 1

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server. The key of the map shall be the EAS ID to which

the provided analytics data within the map value relates.

required:

- discoveredEas

EasDiscoverySubscription:

description: Represents an Individual EAS Discovery Subscription resource.

type: object

properties:

eecId:

type: string

description: Represents a unique identifier of the EEC.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easEventType:

$ref: '#/components/schemas/EASDiscEventIDs'

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

easDynInfoFilter:

$ref: '#/components/schemas/EasDynamicInfoFilter'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the EES to send a test notification. Set to false

or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

easIntTrigSup:

type: boolean

description: >

Indicates to the EES whether the EAS instantiation triggering should be performed for

the current request. The default value false indicates the EAS instantiation triggering

should not be performed. The true value indicate the EAS instantiation triggering should

be performed.

eecTriggerRequest:

type: boolean

description: >

Indicates to the EES whether the application triggering is required by the EEC.

Default value false indicates the application triggering is not required. The true value

indicate the application triggering is required.

eecTriggerPortInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

required:

- eecId

- easEventType

EasDiscoveryNotification:

description: Notification of EAS discovery information.

type: object

properties:

subId:

type: string

description: >

Identifier of the individual service provisioning subscription for which the service

provisioning notification is delivered.

eventType:

$ref: '#/components/schemas/EASDiscEventIDs'

discoveredEas:

type: array

items:

$ref: '#/components/schemas/DiscoveredEas'

minItems: 1

description: List of EAS discovery information.

easInstInfos:

type: object

additionalProperties:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EASInstantiationInfo'

minProperties: 1

description: >

Contains the EAS instantiation information for each discovered EAS returned within

the "discoveredEas" attribute. The key of the map shall be the EAS ID to which the

provided instantiation information within the map value relates.

edgeLoadAnalytics:

type: object

additionalProperties:

$ref: '#/components/schemas/EdgeLoadAnalytic'

minProperties: 1

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server. The key of the map shall be the EAS ID to which

the provided analytics data within the map value relates.

required:

- subId

- eventType

- discoveredEas

EasDiscoveryFilter:

description: Represents the EAS characteristics.

type: object

properties:

acChars:

type: array

items:

$ref: '#/components/schemas/ACCharacteristics'

minItems: 1

description: AC description for which an EAS is needed.

appGroupProfile:

$ref: 'TS24558_Eecs_ServiceProvisioning.yaml#/components/schemas/AppGroupProfile'

easChars:

type: array

items:

$ref: '#/components/schemas/EasCharacteristics'

minItems: 1

description: Required EAS chararcteristics.

EasCharacteristics:

description: Represents the EAS chararcteristics.

type: object

properties:

easId:

type: string

description: EAS application identifier.

appGrpId:

type: string

description: >

Application group identifier, identifying a group of UEs using the same

application service.

easSyncInd:

type: boolean

description: >

Indicates whether the content synchronization between the EASs is required. The

default value false indicates the EAS synchronization is not required. The true

value indicate the EAS synchronization is required.

easProvId:

type: string

description: EAS provider identifier.

stdEasType:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASCategory'

easType:

type: string

description: EAS type with the flexible value set.

easSched:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

svcArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

svcPermLevel:

type: string

description: Service permissions level.

svcFeats:

type: array

items:

type: string

minItems: 1

description: Service features.

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

not:

required: [stdEasType, easType]

DiscoveredEas:

description: Represents an EAS discovery information.

type: object

properties:

eas:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASProfile'

eesEndPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

lifeTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

trafficInfluenceInd:

type: boolean

description: >

Set to true to indicate the traffic influence is performed on the EAS. Set to

false or omitted otherwise.

required:

- eas

EasDynamicInfoFilter:

description: Represents EAS dynamic information changes filter.

type: object

properties:

dynInfoFilter:

type: array

items:

$ref: '#/components/schemas/EasDynamicInfoFilterData'

minItems: 1

description: List of EAS dynamic information required by the EEC per EAS.

required:

- dynInfoFilter

EasDynamicInfoFilterData:

description: Represents an EAS dynamic information.

type: object

properties:

eecId:

type: string

description: The application identifier of the EAS, e.g. FQDN, URI.

easStatus:

type: boolean

description: Notify whether the EAS status is changed or not.

easAcIds:

type: boolean

description: Notify whether the list of AC identifiers changed or not.

easDesc:

type: boolean

description: Notify whether the EAS description is changed or not.

easPt:

type: boolean

description: Notify whether the EAS endpoint is changed or not.

easEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easFeature:

type: boolean

description: Notify whether the EAS feature is changed or not.

easSchedule:

type: boolean

description: Notify whether the EAS schedule is changed or not.

svcArea:

type: boolean

description: Notify whether the EAS service area is changed or not.

svcKpi:

type: boolean

description: Notify whether the EAS KPIs is changed or not.

svcCont:

type: boolean

description: Notify whether the EAS supported ACR changed or not.

required:

- eecId

ACCharacteristics:

description: Represents EAS dynamic information changes filter.

type: object

properties:

acProf:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

required:

- acProf

EASDiscEventIDs:

anyOf:

- type: string

enum:

- EAS_AVAILABILITY_CHANGE

- EAS_DYNAMIC_INFO_CHANGE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the EAS Discovery Subscription events.

Possible values are

- EAS_AVAILABILITY_CHANGE: Represents the EAS availability change event.

- EAS_DYNAMIC_INFO_CHANGE: Represents the EAS dynamic information change event.

EasDiscoverySubscriptionPatch:

description: Represents an Individual EAS Discovery Subscription resource.

type: object

properties:

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

easDynInfoFilter:

$ref: '#/components/schemas/EasDynamicInfoFilter'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

easEventType:

$ref: '#/components/schemas/EASDiscEventIDs'

RequestorId:

description: Represents identifier of the requestor.

type: object

properties:

eesId:

type: string

description: The identifier of the EES (e.g. S-EES).

easId:

type: string

description: The application identifier of the EAS (e.g. S-EAS), e.g. FQDN, URI.

eecId:

type: string

description: The identifier of the EEC.

oneOf:

- required: [eesId]

- required: [easId]

- required: [eecId]

EdgeLoadAnalytic:

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server.

type: object

properties:

easId:

type: string

description: The application identifier of the EAS, e.g. FQDN, URI.

predictData:

$ref: '#/components/schemas/PredictiveData'

statisticData:

$ref: '#/components/schemas/StatisticalData'

required:

- easId

PredictiveData:

description: >

Contains the predictive analytics data for each discovered EAS service status change.

type: object

properties:

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

description: >

Indicates the schedule of the EAS.

minItems: 1

status:

type: string

description: >

Indicates the EAS status (e.g. enabled, disabled, overload warning etc.).

StatisticalData:

description: >

Contains the statistical analytics data (e.g. number of times the client received

expected performance from the EAS).

type: object

properties:

numRecPerf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

# *** End of Changes ***