3GPP TSG-CT1 Meeting #157	C1-256800

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0169 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR enhancements to support N6 tunnel and E2E tunnel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR#0655(see S6-243697) agreed in SA6#62 proposed to 3GPP TS 23.558, have enhancments to support the N6 tunnel and E2E tunnel in EES AppContextRelocation procedure. TS 24.558 needs to be aligned with the above SA6 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes updates the EES AppContextRelocation service operation, information elements and the EES AppContextRelocation Open API. Backward Compatibility: The changes proposed are backward compatible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.1, 6.5.5.2.2, 6.5.7, A.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the API in TS24558_Eees_AppContextRelocation.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 6.5.5.1	General

This clause specifies the application data model supported by the Eees_AppContextRelocation API.

Table 6.5.5.1-1 specifies the data types defined specifically for the Eees_AppContextRelocation API service.

Table 6.5.5.1-1: Eees_AppContextRelocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrDecReq | 6.5.5.2.4 | Represents the parameters to inform about the selected target EAS and provide the associated information. |  |
| AcrDetermReq | 6.5.5.2.2 | Represents the parameters to request ACR with action determination. |  |
| AcrInitReq | 6.5.5.2.3 | Represents the parameters to request ACR with action initiation. |  |
| AcrModificationParams | 6.5.5.2.8 | Represents ACR parameters specific to ACR modification request. | EdgeApp_2 |
| AcrParameters | 6.5.5.2.7 | Represents ACR parameters specific to ACR request initiated for Service continuity planning. | EdgeApp_2 |
| EecCtxtReloc | 6.5.5.2.5 | Represents the EEC Context relocation information. |  |
| ExpectedLocationArea | 6.5.5.2.6 | Represents the predicted/expected location information of the UE or the geographical service area | EdgeApp_2 |

Table 6.5.5.1-2 specifies data types re-used by the Eees_AppContextRelocation API service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_AppContextRelocation.

Table 6.5.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [3] | Represents the predicted expiration time by which the UE reaches location. |  |
| DurationSec | 3GPP TS 29.122 [3] | Unsigned integer identifying a period of time in units of seconds. |  |
| EASBundleInfo | 3GPP TS 29.558 [4] | Represents EAS bundle information. | EdgeApp_2 |
| EndPoint | 3GPP TS 29.558 [4] | Represents the endpoint information of an EAS. |  |
| Gpsi | 3GPP TS 29.571 [5] | Represents a GPSI. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents the service area of the UE. | EdgeApp_2 |
| LocationInfo | 3GPP TS 29.122 [3] | Represents the location information of the UE. | EdgeApp_2 |
| RouteToLocation | 3GPP TS 29.571 [5] | Represent the N6 traffic routing information and/or routing profile ID for a DNAI. |  |
| TunnelInfo | 3GPP TS 29.558 [4] | Represents the tunnel information. | EdgeApp_2_Ext1 |

# *** Next Change ***

##### 6.5.5.2.2	Type: AcrDetermReq

Table 6.5.5.2.2-1: Definition of type AcrDetermReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the EEC or the EAS that is sending the request. |  |
| easId | string | O | 0..1 | Contains the application identifier of the EAS, e.g. FQDN, URI.. |  |
| sEasEndpoint | EndPoint | M | 1 | Contains the endpoint information of the selected S-EAS. |  |
| ueId | Gpsi | M | 1 | Contains the identifier of the concerned UE. |  |
| acId | string | O | 0..1 | Contains the identifier of the AC. |  |
| expectedLocArea | ExpectedLocationArea | O | 0..1 | Represents the predicted/expected location information of the UE or the geographical service area. | EdgeApp_2 |
| tunnel_Info | TunnelInfo | O | 0..1 | Contains the target tunnel information. | EdgeApp_2_Ext1 |

# *** Next Change ***

### 6.5.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8 of 3GPP TS 29.558 [4]. Table 6.5.7-1 lists the supported features for Eees_AppContextRelocation API.

Table 6.5.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_2 | This feature indicates support of the enhancements for the Enabling Edge Applications. Within this feature the following enhacements are covered: - support of prediction expiration time in the ACR request; - support for ACR modification procedure; and - support of ACR for EAS bundle. |
| 2 | EdgeApp_3 | This feature indicates support of the second set of the enhancements for the Enabling Edge Applications. Within this feature the following enhancements are covered: - support of common EAS relocation. |
| 3 | EdgeApp_2_Ext1 | This feature indicates the support of the enhancements to the Edge Applications. Within this feature, the following enhancements are covered: - Support of N6 and E2E tunnel in the AppContextRelocation service. |

# *** Next Change ***

# A.5	Eees_AppContextRelocation API

openapi: 3.0.0

info:

title: Eees Application Context Relocation Service

version: "1.2.0-alpha.4"

description: |

Eees Application Context Relocation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.558 V19.4.0; Enabling Edge Applications; Protocol specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/24.558/'

servers:

- url: '{apiRoot}/eees-appctxtreloc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials:

- eees-appctxtreloc

paths:

/determine:

post:

summary: Request ACR determination.

operationId: Determine

tags:

- Determine ACR

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrDetermReq'

responses:

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/initiate:

post:

summary: Request the initiation of ACR.

operationId: Initiate

tags:

- Initiate ACR

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrInitReq'

responses:

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/declare:

post:

summary: Informs about the selected target EAS and provides the associated information.

operationId: Declare

tags:

- Declare selected target EAS

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AcrDecReq'

responses:

'204':

description: >

No Content. The selected target EAS information is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

eees-appctxtreloc: Access to the Eees_AppContextRelocation API

schemas:

AcrDetermReq:

description: >

Represents the parameters to request ACR with action determination. The ueId attribute

shall be present, although it is not specified as a mandatory due to backward

compatibility reasons.

type: object

properties:

requestorId:

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acId:

type: string

easId:

type: string

sEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

expectedLocArea:

$ref: '#/components/schemas/ExpectedLocationArea'

tunnel_Info:

$ref: 'TS29558_Eecs_TargetEESDiscovery.yaml#/components/schemas/TunnelInfo'

required:

- requestorId

- sEasEndpoint

AcrInitReq:

description: >

Represents the parameters to request ACR with action initiation. The ueId attribute

shall be present, although it is not specified as a mandatory due to backward

compatibility reasons.

type: object

properties:

requestorId:

type: string

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acId:

type: string

easId:

type: string

tEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

sEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

prevTEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

routeReq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteToLocation'

simInactTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

easNotifInd:

type: boolean

default: false

prevEasNotifInd:

type: boolean

default: false

eecCtxtReloc:

$ref: '#/components/schemas/EecCtxtReloc'

expectedLocArea:

$ref: '#/components/schemas/ExpectedLocationArea'

acrParams:

$ref: '#/components/schemas/AcrParameters'

acrModificationParams:

$ref: '#/components/schemas/AcrModificationParams'

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

tEasEndPointBundleList:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

ueIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

expectedLocAreas:

type: array

items:

$ref: '#/components/schemas/ExpectedLocationArea'

minItems: 1

required:

- requestorId

- tEasEndpoint

- easNotifInd

AcrDecReq:

description: >

Represents the parameters to inform about the selected target EAS and provide the

associated information.

type: object

properties:

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acId:

type: string

tEasId:

type: string

tEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

expectedLocArea:

$ref: '#/components/schemas/ExpectedLocationArea'

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

tEasEndPointBundleList:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

requestorId:

description: >

Contains the identifier of the EAS that is sending the request. It shall be

included although it is not specified as a mandatory due to backward

compatibility reasons.

type: string

appGrpUeIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

description: Contains a list of UE identifier(s) in the application group.

required:

- ueId

- tEasId

- tEasEndpoint

EecCtxtReloc:

description: Represents the EEC Context relocation information.

type: object

properties:

eecCtxtId:

type: string

sEesId:

type: string

sEecEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

tEesId:

type: string

tEecEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- eecCtxtId

ExpectedLocationArea:

description: >

Represents the expected location or service area of UE.

type: object

properties:

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

svcArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

AcrParameters:

description: >

Represents ACR parameters specific to ACR request initiated for Service continuity planning.

type: object

properties:

predictExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AcrModificationParams:

description: >

Represents ACR parameters specific to ACR modification request. The sEasEndpoint,

tEasEndpoint and acrParams attributes shall be present although they are not specified

as a mandatory due to backward compatibility reasons.

type: object

properties:

sEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

tEasEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

acrParams:

$ref: '#/components/schemas/AcrParameters'

# *** End of Changes ***