3GPP TSG-CT WG1 Meeting #157	C1-256572

# Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0127 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on LCS session initiated before the LCS-UP connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage 2 requirements for LCS UP in Release 19 have been updated to support multiple LCS UP connections per UE. The following case can be considered: - The UE has multiple LCS sessions, e.g., two LCS sessions with LMF1, while the UE has LCS-UP connection with LMF2. - Once LCS-UP connection with LMF1 is established, the UE still has two available LCS sessions with LMF1, which were initiated previously over the control plane. For this case, whether and how the UE detremines which LCS session to be transferred to the LCS-UP connection with the LMF1 needs to be clarified. Although LCS session transfer from the CP to the UP would ensure efficient utilization of the LCS-UP connection, it requires substantial protocol impact and additional handing at the UE and the LMF to update the association between the LCS session(s) and the LCS-UP connection. Considering this protocol impact, it is proposed to clarify in the spec that - The UE shall use the control plane transport for LCS session(s) unless the LMF indicates explicitly by DL LCS-UP TRANSPORT message that the LCS session should be transferred to UP: It means that any LCS session initiated before the LCS-UP connection setup stays over CP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a NOTE for clarification on handling of LCS session that has been initiated before the LCS-UP connection establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigous specification for any available LCS session(s) that has been intiatied before the LCS-UP connection with the LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.1.1.1	General

The purpose of the network initiated user plane connection establishment procedure is to establish an LCS secured user plane connection between the UE and the LMF as described in clause 6.18.1 of 3GPP TS 23.273 [2]. The USER PLANE CONNECTION ESTABLISHMENT COMMAND message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message. Acknowledgement of the establishment of the LCS secured user plane connection between UE and LMF is indicated by the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message, encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.1.1.1.1 illustrates an example of the signalling transport for network initiated user plane connection establishment procedure messages.

NOTE:	The successful establishment of an LCS secured user plane connection between the UE and the LMF does not affect the LCS session initiated over the control plain prior to the establishment of the LCS secured user plane connection.

Figure 6.2.1.1.1.1: Signalling transport for network initiated user plane connection establishment procedure

* * * End of Change * * * *