3GPP TSG-CT WG1 Meeting #157	C1-256573

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0128 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the LCS Correlation ID for UPP-CM message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in 3GPP TS 23.273 subclause 6.18.1 step 2, LCS User Plane Connection establishment is independent of any Location Session, i.e. the LCS correlation ID is not required for LCS UPP connection procedures. And CT4 removed the LCS correlation ID for LCS UPP CM N2 message transfer in TS 29.518, see details in agreed CR C4-253490. It is proposed to correct the LCS correlation ID from LMF to AMF for network initiated user plane connection establishment procedure in figure 6.2.1.1.1.1, figure 6.2.1.2.1.1 and figure 6.2.2.1.1.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the LCS correlation ID from LMF to AMF in figure 6.2.1.1.1.1, figure 6.2.1.2.1.1, and figure 6.2.2.1.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LCS correlation ID exists in the N1N2MessageTransferReqData which is not aligned with 29.518. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1, 6.2.1.2.1, 6.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.1.1.1	General

The purpose of the network initiated user plane connection establishment procedure is to establish an LCS secured user plane connection between the UE and the LMF as described in clause 6.18.1 of 3GPP TS 23.273 [2]. The USER PLANE CONNECTION ESTABLISHMENT COMMAND message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message. Acknowledgement of the establishment of the LCS secured user plane connection between UE and LMF is indicated by the USER PLANE CONNECTION ESTABLISHMENT COMPLETE message, encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message as defined in 3GPP TS 24.501 [4]. Figure 6.2.1.1.1.1 illustrates an example of the signalling transport for network initiated user plane connection establishment procedure messages.

Figure 6.2.1.1.1.1: Signalling transport for network initiated user plane connection establishment procedure

* * * Next Change * * * *

##### 6.2.1.2.1	General

The purpose of the network initiated user plane connection release procedure is to enable the network to release the LCS secured user plane connection between the UE and the LMF via the control plane. The USER PLANE CONNECTION RELEASE COMMAND message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message, and the USER PLANE CONNECTION RELEASE COMPLETE message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message. Figure 6.2.1.2.1.1 illustrates an example of the signalling transport for the network initiated user plane connection release procedure.

Figure 6.2.1.2.1.1: Signalling transport for network initiated user plane connection release procedure

* * * Next Change * * * *

##### 6.2.2.1.1	General

The purpose of the UE requested user plane connection establishment procedure is for a UE to request to establish an LCS secured user plane connection between the UE and the LMF, as described in clause 6.18.2 of 3GPP TS 23.273 [2].

Unless the network has indicated that it supports the multiple LCS secured user plane connections as specified in clause 5.5.1.2 or clause 5.5.1.3 of 3GPP TS 24.501 [4], the UE shall not request to establish an LCS secured user plane connection if there is a prior LCS secured user plane connection that has been successfully established and has not yet been released.

The UE requests establishment of an LCS secured user plane connection by sending a USER PLANE CONNECTION ESTABLISHMENT REQUEST message to the network. The USER PLANE CONNECTION ESTABLISHMENT REQUEST message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message. If accepted, the LMF initiates the network initiated user plane connection establishment procedure as specified in clause 6.2.1.1. Figure 6.2.2.1.1.1 illustrates an example of the NAS signalling transport for UE requested user plane connection establishment procedure messages.

Figure 6.2.2.1.1.1: NAS signalling transport for UE requested user plane connection establishment procedure

* * * End of Changes * * * *