3GPP TSG-CT WG1 Meeting #157	C1-256508

# Sophia-Antipolis, France 13-17 Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0912 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New AT-command CDISCO2 for discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sateliot, Novamint, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For handling discontinuous coverage, at the moment, there is the + CDISCO AT command that can be used to indicate the discontinuous coverage situation, the status of the AS layer (activated or deactivated) and has some control of the unavailability period. However, in satellite E-UTRAN, there is the SIB32/SIB32-NB message that provides valuable information to be exposed to the application layer to better handle discontinuous coverage. As an example, SIB32/SIB32-NB can include TLE-based ephemeris and beam footprint info that can be used at application layer to predict future satellite passes as well as determine the best time instant to trigger some procedures (e.g. sending a MO when the satellite is at maximum elevation angle). |  |  |  |  |  |  |  |  |
| Summary of change: |  | AT command +CDISCO2 for reporting SIB32/SIB32-NB information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no standardised way to expose SIB32/SIB32-NB information to the application layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9x(new), Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### *********** First change **********

## 8.9x	Discontinuous Coverage +CDISCO2

Table 8.9x-1: +CDISCO2 parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CDISCO2=[<n>] | +CME ERROR: <err> |
| +CDISCO2? | +CDISCO2: <n>, [<number_of_SATID> [<CR><LF>,<SATID1>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius> [<CR><LF><SATID2>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius> … [<CR><LF><SATIDn>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius> ]]]] +CME ERROR: <err> |
| +CDISCO2=? | +CDISCO2: (list of supported <n>s) +CME ERROR: <err> |

Description

Set command enables reporting of SIB32/SIB32-NB contents. The set command controls the presentation of unsolicited result code +CDISCO: <n>, [<number_of_SATID>

[<CR><LF>,<SATID1>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

[<CR><LF><SATID2>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

…

[<CR><LF><SATIDn>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

]]]].

If reporting is enabled, unsolicited result code is reported upon reception of a new SIB32/SIB32-NB message.

Refer clause 9.2 for possible <err> values.

Read command returns the current settings of <n> and the last received SIB32/SIB32-NB content.

Test command returns supported <n>s values as a compound value.

Defined values

<n>: integer type.

- Disable presentation of the unsolicited result code +CDISCO2: [<number_of_SATID>

[<CR><LF>,<SATID1>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

[<CR><LF><SATID2>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

…

[<CR><LF><SATIDn>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

]]]].

1	Enable presentation of the unsolicited result code +CDISCO2: [<number_of_SATID> [<CR><LF>,<SATID1>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

[<CR><LF><SATID2>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

…

[<CR><LF><SATIDn>, <inclination>, <arg_perigee>, <right_ascension>, <mean_anomaly>, <eccentricity>, <mean_motion>, <bStar_decimal>, <bStar_exponent>, <epoch_star>, <t_Service_Start>, <elevation_angle_right>, <elevation_angle_left>, <reference_point_longitude>, <reference_point_latitude>, <radius>

]]]].

<number_of_SATID>: integer type; Indicates the number of satellite IDs present in the satellite assistance information, see 3GPP TS 36.331 [86], section 6.3.1.

<SATID>: integer type; Indicates the satellite ID (e.g., satelliteId-r18) in the S&F monitoring list, see 3GPP TS 36.331 [86], section 6.3.1.

<inclination>: integer type; Indicates the tilt of the satellite orbit, see 3GPP TS 36.331 [86], section 6.3.4.

<arg_perigee>: integer type; Indicates the angle from the satellite's ascending node to its periapsis, measured in the direction of motion, see 3GPP TS 36.331 [86], section 6.3.4.

<right_ascension>: integer type; Indicates the right ascension of ascending node, see 3GPP TS 36.331 [86], section 6.3.4.

<mean_anomaly>: integer type; Indicates the fraction of an elliptical orbit's period that has elapsed since the satellite passed periapsis, expressed as an angle, see 3GPP TS 36.331 [86], section 6.3.4.

<eccentricity>: integer type; Indicates  the amount by which the satellite’s orbit around deviates from a perfect circle, see 3GPP TS 36.331 [86], section 6.3.4.

<mean_motion>: integer type; Indicates the angular speed required for a satellite to complete one orbit, see 3GPP TS 36.331 [86], section 6.3.4.

<bStar_decimal>: integer type; Indicates the decimal part of B*, see 3GPP TS 36.331 [86], section 6.3.4.

<bStar_exponent>: integer type; Indicates the exponential part of B*, see 3GPP TS 36.331 [86], section 6.3.4.

<epoch_star>: integer type; Indicates the Time offset to the beginning of the current week (Monday 00:00:00 UTC) of the Epoch, see 3GPP TS 36.331 [86], section 6.3.4.

<t_service_start>: integer type; Indicates the Time information on when the incoming satellite is going to start serving the area for quasi-earth fixed cell, see 3GPP TS 36.331 [86], section 6.3.1.

<elevation_angle_right>: integer type; Indicates the rightmost (with reference to the satellite direction) elevation angle, see 3GPP TS 36.331 [86], section 6.3.1.

<elevation_angle_left>: integer type; Indicates the leftmost (with reference to the satellite direction) elevation angle, see 3GPP TS 36.331 [86], section 6.3.1.

<reference_point_longitude>: integer type; Indicates the longitude of the reference point, see 3GPP TS 36.331 [86], section 6.3.1.

<reference_point_latitude>: integer type; Indicates the latitude of the reference point, see 3GPP TS 36.331 [86], section 6.3.1.

<radius>: integer type; Indicates the distance between the reference point and the edge of the satellite or beam coverage, see 3GPP TS 36.331 [86], section 6.3.1.

Implementation

Optional.

This command is only applicable to UEs supporting satellite E-UTRAN.

##### *********** Second change **********

Annex B (normative):
Summary of result codes

ITU-T Recommendation V.250 [14] result codes which can be used in the present document and result codes defined in the present document:

Table B.1: Result codes

| Verbose result code (V.250 command V1 set) |  | Numeric (V0 set) |  | Type |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| +C5GNSAOSS |  | as verbose |  | unsolicited |  | refer clause 10.1.96 |  |
| +C5GPDUAUTHU |  | as verbose |  | unsolicited |  | refer clause 10.1.73 |  |
| +C5GREG |  | as verbose |  | unsolicited |  | refer clause 10.1.47 |  |
| +C5GREGN3GPP |  | as verbose |  | unsolicited |  | refer clause 10.1.85 |  |
| +C5GUSMS |  | as verbose |  | unsolicited |  | refer clause 10.1.59 |  |
| +CABTSRI |  | as verbose |  | unsolicited |  | refer clause 10.1.41 |  |
| +CACSP |  | as verbose |  | unsolicited |  | refer clause 11.1.7 |  |
| +CALV |  | as verbose |  | unsolicited |  | refer clause 8.16 |  |
| +CANCHEV |  | as verbose |  | unsolicited |  | refer clause 11.1.8 |  |
| +CAPPLEVMC |  | as verbose |  | unsolicited |  | refer clause 8.78 |  |
| +CAPPLEVMSNR |  | as verbose |  | unsolicited |  | refer clause 8.84a |  |
| +CAPTT |  | as verbose |  | unsolicited |  | refer clause 11.1.4 |  |
| +CAULEV |  | as verbose |  | unsolicited |  | refer clause 11.1.5 |  |
| +CBCAP |  | as verbose |  | unsolicited |  | refer clause 8.59 |  |
| +CBCHG |  | as verbose |  | unsolicited |  | refer clause 8.61 |  |
| +CBCON |  | as verbose |  | unsolicited |  | refer clause 8.60 |  |
| +CC2APT |  | as verbose |  | unsolicited |  | refer clause 18.2.2 |  |
| +CCCM |  | as verbose |  | unsolicited |  | refer clause 7.16 |  |
| +CCKEYREQ |  | as verbose |  | unsolicited |  | refer clause 7.46 |  |
| +CCSFBU |  | as verbose |  | unsolicited |  | refer clause 8.76 |  |
| +CCSTATEREQU |  | as verbose |  | unsolicited |  | refer clause 10.1.72 |  |
| +CCWA |  | as verbose |  | unsolicited |  | refer clause 7.12 |  |
| +CCWV |  | as verbose |  | unsolicited |  | refer clause 8.28 |  |
| +CDEV |  | as verbose |  | unsolicited |  | refer clause 8.10 |  |
| +CDIP |  | as verbose |  | unsolicited |  | refer clause 7.9 |  |
| +CDISCO |  | as verbose |  | unsolicited |  | refer clause 8.87 |  |
| +CDISCO2 |  | as verbose |  | unsolicited |  | refer clause 8.9x |  |
| +CDNSADD |  | as verbose |  | unsolicited |  | refer clause 10.1.80 |  |
| +CDUT |  | as verbose |  | intermediate |  | refer clause 13.2.1 |  |
| +CDUU |  | as verbose |  | unsolicited |  | refer clause 13.2.1 |  |
| +CEASRSIU |  | as verbose |  | unsolicited |  | refer clause 10.1.93 |  |
| +CECN |  | as verbose |  | unsolicited |  | refer clause 6.28 |  |
| +CECSCONFU |  | as verbose |  | unsolicited |  | refer clause 10.1.84 |  |
| +CEDRXSP |  | as verbose |  | unsolicited |  | refer clause 7.40 |  |
| +CEMBMSRI |  | as verbose |  | unsolicited |  | refer clause 14.2.2 |  |
| +CEMBMSSAII |  | as verbose |  | unsolicited |  | refer clause 14.2.6 |  |
| +CEMBMSSRVI |  | as verbose |  | unsolicited |  | refer clause 14.2.3 |  |
| +CEN1 |  | as verbose |  | intermediate unsolicited |  | refer clause 8.67 |  |
| +CEN2 |  | as verbose |  | intermediate unsolicited |  | refer clause 8.67 |  |
| +CEN3 |  | as verbose |  | intermediate unsolicited |  | refer clause 8.67 |  |
| +CEN4 |  | as verbose |  | intermediate unsolicited |  | refer clause 8.67 |  |
| +CEPTT |  | as verbose |  | unsolicited |  | refer clause 11.1.10 |  |
| +CEPSFBS |  | as verbose |  | unsolicited |  | refer clause 8.81 |  |
| +CEREG |  | as verbose |  | unsolicited |  | refer clause 10.1.22 |  |
| +CLOGSIMAPDURD |  | as verbose |  | unsolicited |  | refer clause 20.2.x |  |
| +CLOGSIMAPDUTD |  | as verbose |  | unsolicited |  | refer clause 20.2.x |  |
| +CPBW |  | as verbose |  | intermediate |  | refer clause 8.14 |  |
| +CPNERU |  | as verbose |  | unsolicited |  | refer clause 8.70 |  |
| +CGBRRREP |  | as verbose |  | unsolicited |  | refer clause 10.1.69 |  |
| +CGDEL |  | as verbose |  | intermediate |  | refer clause 10.1.29 |  |
| +CGEV |  | as verbose |  | unsolicited |  | refer clause 10.1.19 |  |
| +CGREG |  | as verbose |  | unsolicited |  | refer clause 10.1.20 |  |
| +CHSR |  | as verbose |  | intermediate |  | refer clause 6.16 |  |
| +CIEV |  | as verbose |  | unsolicited |  | refer clause 8.10 |  |
| +CCIOTOPTI |  | as verbose |  | unsolicited |  | refer clause 7.42 |  |
| +CIREGU |  | as verbose |  | unsolicited |  | refer clause 8.71 |  |
| +CIREPH |  | as verbose |  | unsolicited |  | refer clause 8.64 |  |
| +CIREPI |  | as verbose |  | unsolicited |  | refer clause 8.64 |  |
| +CKEV |  | as verbose |  | unsolicited |  | refer clause 8.10 |  |
| +CLADNU |  | as verbose |  | unsolicited |  | refer clause 10.1.61 |  |
| +CELADNU |  | as verbose |  | unsolicited |  | refer clause 10.1.61a |  |
| +CLAV |  | as verbose |  | unsolicited |  | refer clause 8.31 |  |
| +CLIP |  | as verbose |  | unsolicited |  | refer clause 7.6 |  |
| +CLPWUSSI |  | as verbose |  | unsolicited |  | refer clause 10.1.xx |  |
| +CMCCSI |  | as verbose |  | unsolicited |  | refer clause 8.73 |  |
| +CMCCSS<x> |  | as verbose |  | unsolicited |  | refer clause 8.73 |  |
| +CMCCSSEND |  | as verbose |  | unsolicited |  | refer clause 8.73 |  |
| +CME ERROR |  | as verbose |  | final |  | refer clause 9.2.0 |  |
| +CMICO |  | as verbose |  | unsolicited |  | refer clause 10.1.55 |  |
| +CMOLRE |  | as verbose |  | unsolicited |  | refer clause 9.3.1 |  |
| +CMOLRG |  | as verbose |  | unsolicited |  | refer clause 8.50 |  |
| +CMOLRN |  | as verbose |  | unsolicited |  | refer clause 8.50 |  |
| +CMSSRI |  | as verbose |  | unsolicited |  | refer clause 10.1.89 |  |
| +CMTLR |  | as verbose |  | unsolicited |  | refer clause 8.57 |  |
| +CRTDCP |  | as verbose |  | unsolicited |  | refer clause 10.1.44 |  |
| +CMWN |  | as verbose |  | unsolicited |  | refer clause 7.36 |  |
| +CNAP |  | as verbose |  | intermediate unsolicited |  | refer clause 7.30 |  |
| +CNEC_MM |  | as verbose |  | unsolicited |  | refer clause 9.1b |  |
| +CNEC_GMM |  | as verbose |  | unsolicited |  | refer clause 9.1b |  |
| +CNEC_GSM |  | as verbose |  | unsolicited |  | refer clause 9.1b |  |
| +CNEC_EMM |  | as verbose |  | unsolicited |  | refer clause 9.1b |  |
| +CNEC_ESM |  | as verbose |  | unsolicited |  | refer clause 9.1b |  |
| +CNEMIU |  | as verbose |  | unsolicited |  | refer clause 7.33 |  |
| +CNEMS1 |  | as verbose |  | unsolicited |  | refer clause 7.33 |  |
| +CNEM5G |  | as verbose |  | unsolicited |  | refer clause 7.33 |  |
| +CNRREG |  | as verbose |  | unsolicited |  | refer clause 10.1.47 |  |
| +COEV |  | as verbose |  | unsolicited |  | refer clause 8.10 |  |
| +COLP |  | as verbose |  | intermediate unsolicited |  | refer clause 7.8 |  |
| +CPAGTCC |  | as verbose |  | unsolicited |  | refer clause 10.1.79 |  |
| +CPAGERES |  | as verbose |  | unsolicited |  | refer clause 10.1.78 |  |
| +CPEIPSSI |  | as verbose |  | unsolicited |  | refer clause 10.1.94 |  |
| +CPINR |  | as verbose |  | intermediate |  | refer clause 8.65 |  |
| +CPINRE |  | as verbose |  | intermediate |  | refer clause 8.65 |  |
| +CPOSR |  | as verbose |  | unsolicited |  | refer clause 8.56 |  |
| +CPNERU |  | as verbose |  | unsolicited |  | refer clause 8.70 |  |
| +CPNSTAT |  | as verbose |  | unsolicited |  | refer clause 7.28 |  |
| +CPSB |  | as verbose |  | unsolicited |  | refer clause 7.29 |  |
| +CR |  | as verbose |  | intermediate |  | refer clause 6.9 |  |
| +CREG |  | as verbose |  | unsolicited |  | refer clause 7.2 |  |
| +CREJPAG |  | as verbose |  | unsolicited |  | refer clause 10.1.77 |  |
| +CRING |  | as verbose |  | unsolicited |  | refer clause 6.11 |  |
| +CRLOSPU |  | as verbose |  | unsolicited |  | refer clause 10.1.65 |  |
| +CRTDCP |  | as verbose |  | unsolicited |  | refer clause 10.1.44 |  |
| +CRUEPOLICYU |  | as verbose |  | unsolicited |  | refer clause 10.1.51 |  |
| +CSBTSRI |  | as verbose |  | unsolicited |  | refer clause 10.1.56 |  |
| +CSCON |  | as verbose |  | unsolicited |  | refer clause 10.1.30 |  |
| +CSDBTSRI |  | as verbose |  | unsolicited |  | refer clause 10.1.58 |  |
| +CSECALGI |  |  | as verbose |  | unsolicited |  | refer clause 8.90 |  |
| +CSENSE |  | as verbose |  | unsolicited |  | refer clause 8.89 |  |
| +CSSI |  | as verbose |  | intermediate |  | refer clause 7.17 |  |
| +CSSU |  | as verbose |  | unsolicited |  | refer clause 7.17 |  |
| +CSTFOR |  | as verbose |  | unsolicited |  | refer clause 8.x |  |
| +CTEV |  | as verbose |  | unsolicited |  | refer clause 8.10 |  |
| +CTZE |  | as verbose |  | unsolicited |  | refer clause 8.41 |  |
| +CTZEU |  | as verbose |  | unsolicited |  | refer clause 8.41 |  |
| +CTZV |  | as verbose |  | unsolicited |  | refer clause 8.41 |  |
| +CUNPER |  | as verbose |  | unsolicited |  | refer clause 8.86 |  |
| +CUSATEND |  | as verbose |  | unsolicited |  | refer clause 12.2.4 |  |
| +CUSATP |  | as verbose |  | unsolicited |  | refer clause 12.2.4 |  |
| +CUSATS |  | as verbose |  | unsolicited |  | refer clause 12.2.3 |  |
| +CUSD |  | as verbose |  | unsolicited |  | refer clause 7.15 |  |
| +CUUAAPT |  | as verbose |  | unsolicited |  | refer clause 18.2.1 |  |
| +CUUS1I |  | as verbose |  | intermediate |  | refer clause 7.26 |  |
| +CUUS1U |  | as verbose |  | unsolicited |  | refer clause 7.26 |  |
| +CE5EECSRV |  | as verbose |  | unsolicited |  | refer clause 19.2.3 |  |
| +CWLANOLADI |  | as verbose |  | unsolicited |  | refer clause 10.1.39 |  |
| +CWLANOLCMI |  | as verbose |  | unsolicited |  | refer clause 10.1.40 |  |
| +CWUSSI |  | as verbose |  | unsolicited |  | refer clause 10.1.95 |  |
| +DR |  | as verbose |  | intermediate |  | refer clause 6.26 |  |
| +ILRR |  | as verbose |  | intermediate |  | refer clause 4.3 |  |
| BUSY |  | 7 |  | final |  | busy signal detected |  |
| CONNECT |  | 1 |  | intermediate |  | connection has been established |  |
| CONNECT <text> |  | manufacturer specific |  | intermediate |  | as CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |  |
| ERROR |  | 4 |  | final |  | command not accepted |  |
| NO ANSWER |  | 8 |  | final |  | connection completion timeout |  |
| NO CARRIER |  | 3 |  | final |  | connection terminated |  |
| NO DIALTONE |  | 6 |  | final |  | no dialtone detected |  |
| OK |  | 0 |  | final |  | acknowledges execution of a command line |  |
| RING |  | 2 |  | unsolicited |  | incoming call signal from network |  |
| NOTE: From v6.2.0 onwards, ATV0 numeric result codes 5, 6, 7 for NO DIALTONE, BUSY and NO ANSWER respectively, have been replaced by numeric result codes 6, 7, 8 respectively, to be aligned with the values listed in ITU-T Recommendation V.250 [14] (previously V.25ter). |  |  |  |  |  |  |  |

NOTE:	The table B.1 is as an overview of the result codes, hence the complete syntax of the result codes is not shown.

##### *********** End of changes **********