3GPP TSG-CT WG1 Meeting #157	C1-256577

# Sophia-Antipolis, France, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0908 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New AT Command to identify media flows that carry PDU Sets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PDU Set is one or more PDUs carrying the payload of one unit of information generated at the application layer. For example, a PDU Set can be a video frame or slice that the application can decode. Currently there is no AT command to configure media information in MT for PDU set processing. QOS filters that network installs, does not specify that these are media flows. They are just 5 tuples, and we need additional inputs from AP to tie QoS flow and Media flow information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new AT command +CIDMF is defined to identify media flows upon new media stream creation which carries PDU sets. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no way to identify media flows in media stream that carries PDU sets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 10.1.X	Identify media flows +CIDMF

Table 10.1.X-1: +CIDMF parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CIDMF=<midx>[,<IP_Type>,<local_address_and_subnet_mask>,<remote_address_and_subnet_mask>,<local_port>,<remote_port>] | +CME ERROR: <err> |
| +CIDMF? | [+CIDMF: <midx>,>,<IP_Type>,<local_address_and_subnet_mask>,<remote_address_and_subnet_mask>,<local_port>,<remote_port>] [<CR><LF>+CIDMF: <midx>,>,<IP_Type>,<local_address_and_subnet_mask>,<remote_address_and_subnet_mask>,<local_port>,<remote_port> [...]] |
| +CIDMF=? | +CIDMF: (list of supported <midx>s),(list of supported <IP_Type>s),(list of supported <local_address_and_subnet_mask>s),(list of supported <remote_address_and_subnet_mask>s),(list of supported <local_port>s),(list of supported <remote_port>s |

Description

The set command allows the MT to identify media flows upon new media stream creation that carries PDU Sets. A special form of the set command with media index only will delete stored media flow corresponding to that media index.

The read command returns a list of all stored media flows.

The test command will return maximum number of flows to be stored (supported range is 0 to 15).

Defined values

<midx>: integer type; media index, supported range (0-15).

<IP_Type>: integer type; specifies the IP addressing used.

0	Indicates IPv4 addressing is used.

1	Indicates IPv6 addressing is used.

<local_address_and_subnet_mask>: string type. The string is given as dot-separated numeric (0-255) parameters on the form:
"a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or
"a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8.m9.m10.m11.m12.m13.m14.m15.m16", for IPv6.

<remote_address_and_subnet_mask>: string type; The string is given as dot-separated numeric (0-255) parameters on the form:
"a1.a2.a3.a4.m1.m2.m3.m4" for IPv4 or
"a1.a2.a3.a4.a5.a6.a7.a8.a9.a10.a11.a12.a13.a14.a15.a16.m1.m2.m3.m4.m5.m6.m7.m8.m9.m10.m11.m12.m13.m14.m15.m16", for IPv6.

<local_port>: integer type, with value range from 0 to 65535 which indicates a local UDP port.

<remote_port>: integer type, with value range from 0 to 65535 which indicates a remote UDP port.

Implementation

### Optional. This command is only applicable to UEs supporting 5GS.

End of change