3GPP TSG-CT WG1 Meeting #157	C1-256782

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0836 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting ProSe in SNPN for multi-hop UE-to-network relay discovery procedures over PC5 interface (model A and model B) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ProSe_NPN |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was agreed in C1-255514 and C1-255516 (CT1#156, Goteberg, August 2025) to extend the support of ProSe in SNPN to include also multi-hop relay procedures in alignment with the requirements in stage-2 spec, where in that case all the UE nodes in all the hops need to be in the subscribed SNPN. The corresponding impact on different multi-hop relay procedures need to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the support of ProSe in SNPN for multi-hop UE-to-network relay discovery procedures over PC5 interface (model A and model B). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for ProSe in SNPN for multi-hop relay scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.1.2.2.2, 8b.2.1.2.2.3, 8b.2.1.2.3.2, 8b.2.1.2.3.3, 8b.2.1.2.4.2, 8b.2.1.2.4.3, 8b.2.1.2.5.3, 8b.2.1.3.2.2, 8b.2.1.3.2.3, 8b.2.1.3.3.2, 8b.2.1.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

###### 8b.2.1.2.2.2	ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop UE-to-network relay UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop remote UE.

The UE is authorised to perform the ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation if:

a)	the UE is authorised to act as a multi-hop UE-to-network relay in the PLMN or the subscribed SNPN indicated by the serving cell as specified in clause 5.2.8, and

1)	the UE is served by NG-RAN and the UE is authorised to perform 5G ProSe direct discovery in the PLMN or the subscribed SNPN as specified in clause 5; or

2)	the UE is authorised to perform 5G ProSe direct discovery when not served by NG-RAN as specified in clause 5 and intends to use the provisioned radio resources for multi-hop UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be announced as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe multi-hop layer-3 UE-to-network relay UE,

i)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8;

ii)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

iii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11]; and

2)	the User info ID for the multi-hop UE-to-network relay discovery parameter as specified in clause 5.2.8;

3)	the PDU Session parameters to be used for relayed traffic, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation.

Figure 8b.2.1.2.2.2-1 illustrates the interaction of the UEs in the ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation.

Figure 8b.2.1.2.2.2-1: ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

When the UE is triggered by an upper layer application to announce availability of a connectivity service provided by a multi-hop UE-to-network relay, if the UE is authorised to perform the ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for relay discovery as specified in 3GPP TS 38.331 [13], shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement, the UE:

1)	shall set the announcer info parameter to the User info ID configured for the multi-hop UE-to-network relay discovery, as specified in clause 5.2.8;

2)	shall set the hop count parameter to ‘1’;

3)	may set the hop limit parameter to the value provided by the upper layer, if the value is smaller than the maximum number of allowed hops as specified in clause 5.2.8;

4)	shall set the relay service code parameter to the relay service code configured for the connectivity service to be announced, as specified in clause 5.2.8. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a UE-to-network relay; and

i)	for 5G ProSe layer-2 multi-hop UE-to-network relay UE, the UE received an indication from the lower layers that the cell supports IMS emergency bearer services for UEs in limited service mode as specified in 3GPP TS 38.331 [13]; or

ii)	for 5G ProSe multi-hop UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop remote UE;

4a)	shall include the MIC field computed as described in 3GPP TS 33.503 [34], by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

7)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.15;

8)	if acting as 5G ProSe layer-2 multi-hop UE-to-network relay UE, shall set the NCGI parameter to the NCGI of its serving cell;

9)	if acting as 5G ProSe layer-2 multi-hop UE-to-network relay UE, shall set the RRC container to the RRC container if provided by the lower layers;

10) may set the root relay parameter to the User info ID configured for the multi-hop UE-to-network relay discovery; and

11)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

NOTE 0:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 0A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the destination layer-2 ID to the default destination layer-2 ID as specified in clause 5.2.8 and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery announcement; and

NOTE 1:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model as specified in clause 6.2.14.2.2.2, clause 6.2.15.2.2.2 and clause 8b.2.1.3.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement to the lower layers for transmission over the PC5 interface with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once the PROSE PC5 DISCOVERY message is generated. Each self-assigned source layer-2 ID can be associated with only one relay service code.

The UE shall ensure that it keeps on passing the same PROSE PC5 DISCOVERY message along with the same source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission until the UE is triggered by an upper layer application to stop announcing availability of a connectivity service provided by a multi-hop UE-to-network relay, or until the UE stops being authorised to perform the ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation. How this is achieved is left up to UE implementation.

NOTE 2:	The announcing UE can stop ProSe multi-hop UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

***** Next change *****

###### 8b.2.1.2.2.3	ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop parent UE-to-network relay UE towards a 5G ProSe multi-hop child UE-to-network relay UE; or

b)	a 5G ProSe intermediate UE-to-network relay UE towards a 5G ProSe multi-hop remote UE.

The UE is authorised to perform the 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network relay in the PLMN or the subscribed SNPN indicated by the serving cell as specified in clause 5.2.8; and

1)	the UE is served by NG-RAN and the UE is authorised to perform 5G ProSe direct discovery in the PLMN or the subscribed SNPN as specified in clause 5; or

2)	the UE is authorised to perform 5G ProSe direct discovery when not served by NG-RAN as specified in clause 5 and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be announced as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe intermediate UE-to-network relay UE;

i)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

2)	the User info ID for the multi-hop UE-to-network relay discovery parameter as specified in clause 5.2.8; and

3)	the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic; and

c)	the UE has established PC5 direct link with its 5G ProSe multi-hop parent UE-to-network relay UE identified by the Announcer Info in the 5G ProSe UE-to-Network relay discovery entry;

otherwise, the UE is not authorised to perform the 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation.

Figure 8b.2.1.2.2.3-1 illustrates the interaction of the UEs in the 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation.

Figure 8b.2.1.2.2.3-1: ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

When the UE is triggered by an upper layer application to announce availability of a connectivity service provided by an intermediate UE-to-network relay using the 5G ProSe UE-to-Network relay discovery entry as described in clause 8b.2.1.2.3.2, and if the UE is authorised to perform the 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation, then upon monitoring and processing a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement as described in clause 8b.2.1.2.3.2, the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for relay discovery as specified in 3GPP TS 38.331 [13], shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement, the UE shall use the 5G ProSe UE-to-Network relay discovery entry and:

1)	shall set the announcer info parameter to the User info ID configured for the multi-hop UE-to-network relay discovery, as specified in clause 5.2.8;

2)	shall increment the Hop Count Info stored in the 5G ProSe UE-to-Network relay discovery entry by 1 and set it as the hop count parameter;

3)	shall set the relay service code parameter to the RSC Info stored in the 5G ProSe UE-to-Network relay discovery entry;

4)	may set the root relay info parameter to the Root Relay Info stored in the 5G ProSe UE-to-Network relay discovery entry;

5)	may set the accumulated QoS parameter to the Accumulated QoS for PC5 link Info stored in the 5G ProSe UE-to-Network relay discovery entry;

5a)	shall set the hop limit parameter to the Hop Limit Info stored in the 5G ProSe UE-to-network relay discovery entry, if available;

5b)	shall include the MIC field computed as described in 3GPP TS 33.503 [34], by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

6)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

7)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

8)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.15;

9)	if acting as 5G ProSe layer-2 intermediate UE-to-network relay UE, shall set the NCGI parameter to the NCGI of its serving cell;

10)	if acting as 5G ProSe layer-2 intermediate UE-to-network relay UE, shall set the RRC container to the RRC 	container if provided by the lower layers;

11)	shall include the PLMN ID set to the PLMN ID value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement, and in cleartext;

NOTE 0:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 0A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the destination layer-2 ID to the default destination layer-2 ID as specified in clause 5.2.8 and self-assign a source layer-2 ID for sending the intermediate UE-to-network relay discovery announcement; and

NOTE 1:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model as specified in clause 6.2.14.2.2.2, clause 6.2.15.2.2.2 and clause 8b.2.1.3.1.2.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement to the lower layers for transmission over the PC5 interface with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once the PROSE PC5 DISCOVERY message is generated. Each self-assigned source layer-2 ID can be associated with only one relay service code.

The UE shall ensure that it keeps on passing the same PROSE PC5 DISCOVERY message along with the same source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission until the UE is triggered by an upper layer application to stop announcing availability of a connectivity service provided by an intermediate UE-to-network relay, or until the UE stops being authorised to perform the 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation. How this is achieved is left up to UE implementation.

NOTE 2:	The announcing UE can stop 5G ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

***** Next change *****

###### 8b.2.1.2.3.2	ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring

This procedure is performed by:

a)	a 5G ProSe intermediate UE-to-network relay UE to monitor a 5G ProSe multi-hop UE-to-network relay UE; or

b)	a 5G ProSe multi-hop child UE-to-network relay UE to monitor a 5G ProSe multi-hop parent UE-to-network relay UE.

The UE is authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring if:

a)	the following is true:

1)	if the UE is not served by NG-RAN, the UE is authorised to perform 5G ProSe direct discovery monitoring when the UE is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN; or

2)	if the UE is served by NG-RAN, the UE is authorised to perform 5G ProSe direct discovery monitoring in at least one PLMN or the subscribed SNPN; and

b)	the UE is configured with the relay service code parameter identifying the connectivity service to be monitored, as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE.

otherwise, the UE is not authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.2-1 illustrates the interaction of the UEs in the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.2-1: ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring

When the UE is triggered by the upper layers to monitor proximity of a connectivity service provided by a multi-hop UE-to-network relay or a intermediate UE-to-network relay or when the UE decides to perform 5G ProSe multi-hop UE-to-network relay reselection as specified in clause 8b.2.3, and if the UE is authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages with the default destination layer-2 ID as specified in clause 5.2.8.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement according to clause 10.2.1, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement.

NOTE 1:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

NOTE 2:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the relay service code parameter configured as specified in clause 5 for the connectivity service being monitored;

b)	the target announcer info is not provided by upper layers for the connectivity service being monitored, or the announcer info parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the target announcer info if the target announcer info is provided by upper layers for the connectivity service being monitored;

c)	the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is less than the configured maximum number of allowed hops, as specified in clause 5.2.8; and

d) 	the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is less than the hop limit parameter of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement (if available),

then

a)	the UE may measure the signal strength of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement for relay selection or reselection;

b)	if a 5G ProSe UE-to-Network relay discovery entry corresponding to the relay service code parameter and the root relay info parameter of the received PROSE PC5 DISCOVERY message is not stored at the UE:

1)	the UE shall create and store a 5G ProSe UE-to-Network relay discovery entry corresponding to the received PROSE PC5 DISCOVERY message. The 5G ProSe UE-to-Network relay discovery entry:

i)	shall include RSC Info, set to the relay service code parameter of the received PROSE PC5 DISCOVERY message;

ii)	shall include the Announcer Info, set to the Announcer Info parameter of the received PROSE PC5 DISCOVERY message;

iii)	shall include Hop Count Info, set to the hop count parameter of the received PROSE PC5 DISCOVERY message;

iv)	may include Root Relay Info, set to the root relay info parameter of the received PROSE PC5 DISCOVERY message;

v)	may include the Accumulated QoS for PC5 link Info, based on the Accumulated QoS for PC5 link information parameter of the received PROSE PC5 DISCOVERY message and the QoS of the PC5 link between the UE and the sender of the message;

vi)	may include the NCGI Info, set to the NCGI parameter of the received PROSE PC5 DISCOVERY message;

vii)	may include the RRC Container Info, set to the RRC Container parameter of the received PROSE PC5 DISCOVERY message; and

viii)	shall include the hop limit, set to the hop limit parameter of the received PROSE PC5 DISCOVERY message, if available; and

c)	if a 5G ProSe UE-to-Network relay discovery entry corresponding to the relay service code parameter and the root relay info parameter of the received PROSE PC5 DISCOVERY message is already stored, then the UE shall:

1)	check if the hop count parameter of the received PROSE PC5 DISCOVERY message is less than the Hop Count Info of the stored 5G ProSe UE-to-Network relay discovery entry. If yes, the UE:

i)	shall update the Announcer Info of the stored 5G ProSe UE-to-network relay discovery entry to the Announcer Info parameter of the received PROSE PC5 DISCOVERY message;

ii)	shall update the Hop Count Info of the stored 5G ProSe UE-to-network relay discovery entry to the hop count parameter of the received PROSE PC5 DISCOVERY message; and

iii)	may update the Accumulated QoS for PC5 link Info, based on the Accumulated QoS for PC5 link information parameter of the received PROSE PC5 DISCOVERY message and the QoS of the PC5 link between the UE and the sender of the message; and

2)	check if the hop limit parameter of the received PROSE PC5 DISCOVERY message (if available) is different with the Hop Limit Info of the stored 5G ProSe UE-to-network relay discovery entry. If yes, the UE shall update the Hop Limit Info of the stored 5G ProSe-UE-to-network relay discovery entry to the hop limit parameter of the received PROSE PC5 DISCOVERY message.

If the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the configured maximum number of allowed hops as specified in clause 5.2.8; or is the same as the hop limit parameter of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement (if available), then the UE shall discard the received PROSE PC5 DISCOVERY message.

NOTE 3:	Based on UE implementation, the UE may decide to update the 5G ProSe UE-to-network relay discovery entry based on the Accumulated QoS for PC5 link Info instead of the Hop Count Info, or a combination of the two information elements.

NOTE 4:	Based on UE implementation, the UE sets a locally configured timer for each 5G ProSe UE-to-network relay discovery entry. When the timer expires, if the UE does not have an active PC5 link with the 5G ProSe multi-hop UE-to-network relay UE and does not receive any new PROSE PC5 DISCOVERY message from the multi-hop UE-to-network relay UE, the UE removes 5G ProSe UE-to-network relay discovery entry of the 5G ProSe multi-hop UE-to-network relay UE.

***** Next change *****

###### 8b.2.1.2.3.3	ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring

This procedure is performed by:

a)	a 5G ProSe multi-hop remote UE to monitor a 5G ProSe multi-hop UE-to-network relay UE or a 5G ProSe intermediate UE-to-network relay UE.

The UE is authorised to perform ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring if:

a)	the following is true:

1)	the UE is not served by NG-RAN, is authorised to perform 5G ProSe direct discovery using monitoring when the UE is not served by NG-RAN and is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN;

2)	the UE is served by NG-RAN and is authorised to perform 5G ProSe direct discovery monitoring in at least one PLMN or the subscribed SNPN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14] and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN or the subscribed SNPN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11] ; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discovery using monitoring when the UE is not served by NG-RAN, and:

A)	configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN;

b)	the UE is configured with the relay service code parameter identifying the connectivity service to be monitored, as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE; and

c)	for 5G ProSe multi-hop layer-2 remote UE, the UE is camped on a cell whose TAI is not in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is in the list of "allowed tracking areas",

otherwise, the UE is not authorised to perform the ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.3-1 illustrates the interaction of the UEs in the ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.3-1: ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring

When the UE is triggered by the upper layers to monitor proximity of a connectivity service provided by a multi-hop UE-to-network relay or an intermediate UE-to-network relay or when the UE decides to perform 5G ProSe multi-hop UE-to-network relay reselection as specified in clause 8b.2.3, and if the UE is authorised to perform the ProSe multi-hop remote UE procedure for multi-hop UE-to-network relay discovery monitoring, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages with the default destination layer-2 ID as specified in clause 5.2.8.

If the traffic descriptor is configured as specified in clause 5.2.8, the 5G ProSe multi-hop remote UE shall determine the monitored RSC by mapping the traffic from the upper layer application with the traffic descriptor as follows:

a)	if there are at least one ProSe application traffic descriptor(s) to be used for the relayed traffic as specified in clause 5.2.8 which has not yet been evaluated,

1)	if the ProSe application traffic descriptor matches upper layer application information;

the UE shall select the RSC associated with the matched ProSe application traffic descriptor for monitoring, where:

i)	if more than one RSCs are associated with the upper layer application information and the monitored RSC for the 5G ProSe multi-hop remote UE is specific for emergency, the UE should prioritize selecting an RSC that does not provide an indication of using N3IWF access for the relayed traffic; or

ii) otherwise, it is up to UE implementation which RSC to select in this release of specification;

2)	else, the UE shall select the next ProSe application traffic descriptor which has not yet been evaluated and proceed to step 1);

b)	else, it is up to UE implementation which RSC to select, where if the monitored RSC for the 5G ProSe multi-hop remote UE is specific for emergency, the UE should prioritize selecting an RSC that does not provide an indication of using N3IWF access for the relayed traffic.

NOTE 1:	Selection of relay service code is up to UE implementation if there is no ProSe application traffic descriptor(s) configured in the UE.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement according to clause 10.2.1, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement.

NOTE 1A:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

NOTE 2:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is for 5G ProSe direct discovery based on an indication from the lower layer.

Then if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the relay service code parameter configured as specified in clause 5 for the connectivity service being monitored; and

b)	the target announcer info is not provided by upper layers for the connectivity service being monitored, or the announcer info parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the target announcer info if the target announcer info is provided by upper layers for the connectivity service being monitored,

then the UE shall consider that the connectivity service the UE seeks to monitor has been discovered. In addition, the UE can measure the signal strength of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement for relay selection or reselection.

***** Next change *****

###### 8b.2.1.2.4.2	ProSe multi-hop UE-to-network relay UE procedure

This procedure is performed by:

a)	a 5G ProSe multi-hop UE-to-network relay UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop remote UE.

The 5G ProSe multi-hop UE-to-network relay UE announces the multi-hop UE-to-network relay discovery additional information:

a)	if a 5G ProSe multi-hop remote UE requests the 5G ProSe multi-hop UE-to-network relay UE to announce the NG-RAN Cell Global ID (NCGI) or TAI of the cell serving the 5G ProSe multi-hop UE-to-network relay UE, and optionally the NID of the SNPN in case the serving cell is operated by an SNPN, and as a response the 5G ProSe multi-hop UE-to-network relay UE acknowledges with the ProSe multi-hop additional parameters announcement response message, then the 5G ProSe multi-hop UE-to-network relay UE includes the NCGI or TAI of the serving cell and optionally the NID of the SNPN in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information until the timer T5107 expires (see clause 8b.2.1).

NOTE 1:	5G ProSe multi-hop UE-to-network relay UE announces the multi-hop UE-to-network relay discovery additional information only when it is in NG-RAN coverage.

Figure 8b.2.1.2.4.2-1 illustrates the interaction of the UEs in the ProSe multi-hop UE-to-network relay UE procedure for announcing multi-hop UE-to-network relay discovery additional information.

Figure 8b.2.1.2.4.2-1: ProSe multi-hop UE-to-network relay UE procedure for announcing multi-hop UE-to-network relay discovery additional information

The 5G ProSe multi-hop UE-to-network relay UE may start announcing multi-hop UE-to-network relay discovery additional information if:

a)	the 5G ProSe multi-hop UE-to-network relay UE is currently authorised to perform 5G ProSe direct discovery Model A announcing in the serving PLMN or the subscribed SNPN if the UE is served by NG-RAN, and

1)	additional parameters announcement for the serving cell of the 5G ProSe multi-hop UE-to-network relay UE has been requested and responded to 5G ProSe multi-hop remote UEs, the timer T5107 has not expired (periodic reporting);

2)	additional parameters announcement for the serving cell of the 5G ProSe multi-hop UE-to-network relay UE has been requested and responded to 5G ProSe multi-hop remote UEs, the timer T5107 has not expired and the 5G ProSe multi-hop UE-to-network relay UE detects camping on a new serving cell; or

3)	additional parameters announcement for the serving cell of the 5G ProSe multi-hop UE-to-network relay UE has been requested and responded to 5G ProSe multi-hop remote UEs, the timer T5107 has not expired and the 5G ProSe multi-hop UE-to-network relay UE detects entering a new tracking area.

When the 5G ProSe multi-hop UE-to-network relay UE has some additional information to broadcast (i.e., NCGI, TAI, NID), then the 5G ProSe multi-hop UE-to-network relay UE :

a)	shall request the parameters from the lower layers for ProSe direct discovery announcing (see 3GPP TS 38.331 [13]). If the 5G ProSe multi-hop UE-to-network relay UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], the 5G ProSe multi-hop UE-to-network relay UE shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate PROSE PC5 DISCOVERY message(s) for multi-hop UE-to-network relay discovery additional information according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, the 5G ProSe multi-hop UE-to-network relay UE:

1)	shall include the relay service code used for 5G ProSe direct communication which the 5G ProSe multi-hop remote UE used to request for the multi-hop UE-to-network relay discovery additional information;

2)	shall set the announcer info parameter to the User info ID parameter configured in clause 5.2.8;

3)	shall set the NCGI parameter to the NCGI of the cell serving the UE, if the UE acts as 5G ProSe multi-hop layer-3 UE-to-network relay UE and the NCGI is to be announced;

4)	shall set the Relay TAI parameter to the TAI of the cell serving the UE, if the UE acts as 5G ProSe multi-hop layer-3 UE-to-network relay UE and the TAI is to be announced;

5)	shall set the hop count parameter to ‘1’;

6)	may set the hop limit parameter to the value provided by the upper layer, if the value is smaller than the maximum number of allowed hops as specified in clause 5.2.8;

6a)	shall include the MIC field computed as described in 3GPP TS 33.503 [34], by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

7)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

8)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.16;

9)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

10)	for SNPN, if NCGI, Relay TAI or both is included, shall include the NID of the subscribed SNPN;

NOTE 1A:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1B:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

10)	for SNPN, if NCGI, Relay TAI or both is included, shall include the NID of the subscribed SNPN;

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the default destination layer-2 ID as specified in clause 5.2.5 to the destination layer-2 ID and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery additional information; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model as specified in clause 6.2.14.2.2.2, clause 6.2.15.2.2.2 and clause 8b.2.1.3.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

The 5G ProSe multi-hop UE-to-network relay UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY messages periodically to the lower layers for transmission until the corresponding timer expires. How this is achieved is left up to UE implementation.

NOTE 3:	The announcing UE can stop announcing UE procedure for multi-hop UE-to-network relay discovery additional information for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

NOTE 4:	The periodicity of sending the PROSE PC5 DISCOVERY messages for multi-hop UE-to-network relay discovery additional information by the 5G ProSe multi-hop UE-to-network relay UE is implementation specific and is normally lower than the one related to the additional parameters announcement request refresh timer T5106.

During the announcing operation, if one of the above conditions is no longer met, the 5G ProSe multi-hop UE-to-network relay UE may instruct the lower layers to stop announcing.

***** Next change *****

###### 8b.2.1.2.4.3	ProSe intermediate UE-to-network relay UE procedure

This procedure is performed by:

a)	a 5G ProSe multi-hop parent UE-to-network relay UE towards a 5G ProSe multi-hop child UE-to-network relay UE; or

b)	a 5G ProSe intermediate UE-to-network relay UE towards a 5G ProSe multi-hop remote UE.

The 5G ProSe intermediate UE-to-network relay UE announces the multi-hop UE-to-network relay discovery additional information if it receives a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information from a 5G ProSe multi-hop UE-to-network relay UE or another 5G ProSe intermediate UE-to-network relay UE:

NOTE 1:	5G ProSe intermediate UE-to-network relay UE can announce the multi-hop UE-to-network relay discovery additional information both, when it is in NG-RAN coverage and when it is not in NG-RAN coverage.

Figure 8b.2.1.2.4.3-1 illustrates the interaction of the UEs in the 5G ProSe intermediate UE-to-network relay UE procedure for announcing multi-hop UE-to-network relay discovery additional information.

Figure 8b.2.1.2.4.3-1: ProSe intermediate UE-to-network relay UE procedure for announcing multi-hop UE-to-network relay discovery additional information

The 5G ProSe intermediate UE-to-network relay UE may start announcing multi-hop UE-to-network relay discovery additional information if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network relay in the PLMN or the subscribed SNPN indicated by the serving cell as specified in clause 5.2.8, and

1)	the UE is served by NG-RAN and the UE is authorised to perform 5G ProSe direct discovery in the PLMN or the subscribed SNPN as specified in clause 5; or

2)	the UE is authorised to perform 5G ProSe direct discovery when not served by NG-RAN as specified in clause 5 and intends to use the provisioned radio resources for UE-to-network relay discovery; and

b)	it has received a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information from a 5G ProSe multi-hop UE-to-network relay UE or another 5G ProSe intermediate UE-to-network relay UE as specified clause 8b.2.1.2.5.2.

To announce the multi-hop UE-to-network relay discovery additional information, the 5G ProSe intermediate UE-to-network relay UE:

a)	shall request the parameters from the lower layers for ProSe direct discovery announcing (see 3GPP TS 38.331 [13]). If the 5G ProSe intermediate UE-to-network relay UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], the 5G ProSe intermediate UE-to-network relay UE shall perform a service request procedure or mobility registration procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time as specified in clause 11.2.5;

c)	shall generate PROSE PC5 DISCOVERY message(s) for multi-hop UE-to-network relay discovery additional information according to clause 10.2.1. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, the 5G ProSe intermediate UE-to-network relay UE:

1)	shall include the relay service code set to the value of relay service code parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information;

2)	shall set the announcer info parameter to the User info ID parameter configured in clause 5.2.8;

3)	shall set the NCGI of multi-hop UE-to-network relay parameter to the value of NCGI of multi-hop UE-to-network relay parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information;

4)	shall set the Relay TAI parameter to the value of the Relay TAI parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information;

5)	if the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information is set to 1 and the NCGI of 5G ProSe intermediate UE-to-network relay UE is available, may set the NCGI info set of intermediate UE-to-network relay parameter including the NCGI of the cell serving the UE and the user info ID of the UE;

6)	if the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information is set to a value greater than 1 and the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information contains the NCGI info set of intermediate UE-to-network relay parameter then, shall set the NCGI info set of intermediate UE-to-network relay parameter to the received one;

7)	shall increment the hop count parameter of the received PROSE PC5 DISCOVERY for multi-hop UE-to-network relay discovery additional information message by 1, and set it as the hop count parameter;

8)	shall set the hop limit parameter to the value in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, if available;

8a)	shall include the MIC field computed as described in 3GPP TS 33.503 [34], by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

9)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

10)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.16;

11)	if the root relay info parameter is included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, shall set the root relay info parameter to the received root relay info parameter. If the root relay info parameter is not included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, the 5G ProSe intermediate UE-to-network relay UE shall set the root relay info parameter to the announcer info parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information;

12)	shall include the PLMN ID set to the PLMN ID value included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information, and in cleartext;

NOTE 1A:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1B:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the default destination layer-2 ID as specified in clause 5.2.5 to the destination layer-2 ID and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery additional information; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model as specified in clause 6.2.14.2.2.2, clause 6.2.15.2.2.2 and clause 8b.2.1.3.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

The 5G ProSe intermediate UE-to-network relay UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY messages periodically to the lower layers for transmission until the corresponding timer expires. How this is achieved is left up to UE implementation.

NOTE 3:	The announcing UE can stop announcing UE procedure for multi-hop UE-to-network relay discovery additional information for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

During the announcing operation, if one of the above conditions is no longer met, the 5G ProSe multi-hop UE-to-network relay UE may instruct the lower layers to stop announcing.

If the 5G ProSe intermediate UE-to-network relay UE has the PC5 direct link with the 5G ProSe multi-hop UE-to-network relay UE and the 5G ProSe intermediate UE-to-network relay UE determines an update of the NCGI of the cell serving the UE (e.g., due to change of its serving cell), the 5G ProSe intermediate UE-to-network relay UE may announce the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information with the updated NCGI info set of intermediate UE-to-network relay parameter while keeps NCGI of multi-hop UE-to-network relay parameter and Relay TAI parameter unchanged in the message.

***** Next change *****

###### 8b.2.1.2.5.3	ProSe multi-hop remote UE procedure

This procedure is performed by:

a)	a 5G ProSe multi-hop remote UE to monitor a 5G ProSe multi-hop UE-to-network relay UE or a 5G ProSe intermediate UE-to-network relay UE.

The 5G ProSe multi-hop remote UE monitors multi-hop UE-to-network relay discovery additional information:

a)	until the additional parameters announcement request refresh timer T5106 expires if the 5G ProSe multi-hop remote UE has requested the 5G ProSe multi-hop UE-to-network relay UE to announce the NCGI or TAI of the cell serving the 5G ProSe multi-hop UE-to-network relay UE and received the ProSe additional parameters announcement response message from the 5G ProSe multi-hop UE-to-network relay UE .

The UE may instruct the lower layers to start monitoring if:

a)	a request from upper layers to monitor for multi-hop UE-to-network relay discovery additional information is still in place and either:

1)	the UE is currently authorised to perform 5G ProSe direct discovery Model A monitoring in at least one PLMN or the subscribed SNPN if the UE is served by NG-RAN;

2)	the UE is currently authorised to perform 5G ProSe direct discovery Model A monitoring if the UE is not served by NG-RAN; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14] and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN or the subscribed SNPN as specified in 3GPP TS 36.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed " as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to perform 5G ProSe direct discovery Model A monitoring when the UE is not served by NG-RAN and configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN.

If the UE is in 5GMM-CONNECTED mode, the monitoring UE shall also trigger the corresponding procedure in lower layers as specified in 3GPP TS 38.331 [13].

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information according to clause 10.2.1, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code to be monitored, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop relay discovery additional information.

During the monitoring operation, if one of the above conditions is no longer met, the UE may instruct the lower layers to stop monitoring. When the UE stops monitoring, if the UE is in 5GMM-CONNECTED mode, the UE shall trigger the corresponding procedure in lower layers as specified in 3GPP TS 38.331 [13].

NOTE 1:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

NOTE 2:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service being monitored; and

b)	the target announcer info is not provided by upper layers for the connectivity service being monitored, or the announcer info parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information is the same as the target announcer info if the target announcer info is provided by upper layers,

then the UE shall consider that the multi-hop UE-to-network relay discovery additional information it intends to monitor has been discovered.

***** Next change *****

###### 8b.2.1.3.2.2	Discoverer remote UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop remote UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE.

The UE is authorised to perform the discoverer remote UE procedure for multi-hop UE-to-network relay discovery if:

a)	one of the following is true:

1)	the UE is not served by NG-RAN, is authorised to act as a 5G ProSe multi-hop remote UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, and is configured with the radio parameters to be used for ProSe multi-hop UE-to-network relay discovery when not served by NG-RAN;

2)	the UE is served by NG-RAN, is authorised to act as a 5G ProSe multi-hop remote UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE; or

3)	the UE is:

i)	in 5GMM-IDLE mode, in limited service state as specified in 3GPP TS 23.122 [14] and the reason for the UE being in limited service state is one of the following:

A)	the UE is unable to find a suitable cell in the selected PLMN or the subscribed SNPN as specified in 3GPP TS 38.304 [15];

B)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #11 "PLMN not allowed" as specified in 3GPP TS 24.501 [11]; or

C)	the UE received a REGISTRATION REJECT message or a SERVICE REJECT message with the 5GMM cause #7 "5GS services not allowed" as specified in 3GPP TS 24.501 [11]; and

ii)	authorised to act as a 5G ProSe multi-hop remote UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE when the UE is not served by NG-RAN and configured with the radio parameters to be used for ProSe multi-hop UE-to-network relay discovery use when not served by NG-RAN; and

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be solicited as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE;

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8;

otherwise, the UE is not authorised to perform the discoverer remote UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.2.1 illustrates the interaction of the UEs in the discoverer remote UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.2.1: Discoverer remote UE procedure for multi-hop UE-to-network Relay discovery

For PROSE PC5 DISCOVERY message signal strength measurement, the UE manages a periodic measurement timer T5126, which is used to trigger the periodic PROSE PC5 DISCOVERY message signal strength measurement between the UE and the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established. Timer T5126 is started whenever the UE has established a direct link with a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE and restarted whenever the UE receives the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established.

When the UE is triggered by the upper layers to solicit proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or when the periodic measurement timer T5126 expires and if the UE is authorised to perform the discoverer remote UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for relay discovery as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, the UE:

1)	shall set the discoverer info parameter to the configured User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8;

2)	shall set the relay service code parameter to the relay service code parameter identifying the connectivity service to be solicited, configured in clause 5.2.8. For the 5G ProSe multi-hop layer-3 remote UE, if the traffic descriptor is configured as specified in clause 5.2.8, the UE shall determine the RSC as follows:

i)	if there are at least one ProSe application traffic descriptor(s) to be used for the relayed traffic as specified in clause 5.2.8 which has not yet been evaluated,

A)	if the ProSe application traffic descriptor matches the upper layer application information;

the UE shall select the RSC associated with the matched ProSe application traffic descriptor for solicitation, where:

aa) if more than one RSCs are associated with the upper layer application information and the needed RSC for the 5G ProSe multi-hop layer-3 remote UE is specific for emergency, the UE should prioritize selecting an RSC that does not provide an indication of using N3IWF access for the relayed traffic; or

bb) otherwise, it is up to UE implementation which RSC to select in this release of specification;

B)	else, the UE shall select the next ProSe application traffic descriptor which has not yet been evaluated and proceed to step A);

ii)	else, it is up to UE implementation which RSC to select, where if the needed RSC for the 5G ProSe multi-hop layer-3 remote UE is specific for emergency, the UE should prioritize selecting an RSC that does not provide an indication of using N3IWF access for the relayed traffic.

NOTE 1:	Selection of relay service code is up to UE implementation if there is no ProSe application traffic descriptor(s) configured in the UE.

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

5)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.17;

6)	may include the target discoveree info parameter set to the user info ID of the targeted discoveree user (i.e. the targeted 5G ProSe multi-hop UE-to-network relay UE) if the target discoveree info is provided by the application layer;

7)	may include the Intermediate relays IE containing the list of User info IDs of the 5G ProSe intermediate UE-to-network relay UEs that the 5G ProSe multi-hop remote UE wants to use in the path to the 5G ProSe multi-hop UE-to-network relay UE, if any;

8)	may include the hop count parameter and set it to ‘1’;

9)	may include the hop limit parameter and set it to the value provided by the upper layer if the value is smaller than the maximum number of allowed hops as specified in clause 5.2.8; and

10)	shall include the PLMN ID set to the PLMN ID for which the used security parameters are associated and in cleartext;

NOTE 1A:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1B:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the default destination layer-2 ID as specified in clause 5.2.8 to the destination layer-2 ID and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery solicitation message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to solicit proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, the UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation for transmission until the UE is triggered by an upper layer application to stop soliciting proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or until the UE stops being authorised to perform the discoverer remote UE procedure for multi-hop UE-to-network relay discovery. How this is achieved is left up to UE implementation.

NOTE 3:	The discoverer UE can stop discoverer remote UE procedure for multi-hop UE-to-network relay discovery for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to trigger the PROSE PC5 DISCOVERY message signal strength measurement between the UE and by the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established, the UE shall start the retransmission timer T5125. If retransmission timer T5125 expires, the UE shall retransmit the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and restart timer T5125. If no response is received from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established after reaching the maximum number of allowed retransmissions, the UE shall trigger relay reselection procedure.

NOTE 4:	The maximum number of allowed retransmissions is UE implementation specific.

For the discoverer UE (the 5G ProSe multi-hop remote UE), upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the destination layer-2 ID which the UE is configured to respond for, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for multi-hop 5G ProSe UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response.

NOTE 5:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is for 5G ProSe direct discovery based on an indication from the lower layer.

Then if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation; and

b)	the target discoveree info is not provided by upper layers for the connectivity service being solicited, or the discoveree info parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the target discoveree info if the target discoveree info is provided by upper layers for the connectivity service being solicited;

then the UE shall consider that the connectivity service the UE seeks to discover has been discovered. In addition, the UE can measure the signal strength of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response for relay selection or reselection. If the UE has received the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network Relay UE or the 5G ProSe multi-hop UE-to-network Relay UE with which the UE has a link established, the UE shall stop the retransmission timer T5125 and start the periodic measurement timer T5126.

NOTE 6:	If the 5G ProSe multi-hop remote UE receives multiple matching PROSE PC5 DISCOVERY messages for multi-hop UE-to-network relay discovery response within an implementation dependent time window, the 5G ProSe multi-hop remote UE can decide on the selected path based on multiple factors such as the Multi-hop path info, the PC5 signal strength between the 5G ProSe multi-hop remote UE and its neighbour 5G ProSe intermediate UE-to-network Relay UE, the Accumulated QoS for PC5 link and the number of hops to the 5G ProSe multi-hop UE-to-network Relay UE.

NOTE 7:	If the 5G ProSe multi-hop remote UE does not receive any PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response within an implementation dependent time window, the 5G ProSe multi-hop remote UE can increase the hop limit considering the application requirement and reinitiate the Discoverer remote UE procedure for multi-hop UE-to-network relay discovery. In all cases the hop limit is not allowed to be larger than the maximum number of allowed hops specified in clause 5.2.8.

***** Next change *****

###### 8b.2.1.3.2.3	Discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop child UE-to-network relay UE towards a 5G ProSe multi-hop parent UE-to-network relay UE; or

b)	a 5G ProSe intermediate UE-to-network UE towards a 5G ProSe multi-hop UE-to-network relay UE.

The UE is authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network UE in the PLMN or the subscribed SNPN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be solicited or responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe layer-3 intermediate UE-to-network relay UE:

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.3.1 illustrates the interaction of the UEs in the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.2.3.1: Discoverer intermediate UE procedure for multi-hop UE-to-network Relay discovery

For PROSE PC5 DISCOVERY message signal strength measurement, the UE manages a periodic measurement timer T5126, which is used to trigger the periodic PROSE PC5 DISCOVERY message signal strength measurement between the UE and the next 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established. Timer T5126 is started whenever the UE has established a direct link with a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE and restarted whenever the UE receives the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established.

If the UE is authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery and when the UE is triggered by the upper layers to solicit proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or when the periodic measurement timer T5126 expires, or upon receiving and processing a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation that has a matching RSC and has the User info ID of the receiving UE included in the list of User info IDs in the Intermediate relays IE, if any, then the UE:

a0)	if the hop count parameter received in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is smaller than the hop limit parameter in the message, or is smaller than the maximum number of allowed hops for the RSC as specified in clause 5.2.8;

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages for relay discovery as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11];

b)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

c)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, the UE:

1)	shall set the discoverer info parameter to the User info ID obtained from the discoverer info parameter in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

2)	shall set the relay service code parameter to the relay service code parameter identifying the connectivity service to be solicited, as obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation:

3)	shall include the MIC field computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

4)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

5)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.17;

6)	shall include the Multi-hop path info IE and include in it the list of the User info IDs received in the Multi-hop path info IE of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, if any, and add its own configured User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8, at the end of the list;

Editor’s note [5G_ProSe_Ph3, CR#0680]:	It is FFS whether the Layer-2 IDs are included in the Multi-hop path info IE, together with the list of the User info IDs (or as a separate IE), due to the following statement from clause 6.3.2.5.3 in 3GPP TS 23.304: "A 5G ProSe Intermediate UE-to-Network Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message".

7)	may include the target discoveree info parameter set to the user info ID of the targeted discoveree user (i.e. the targeted 5G ProSe multi-hop UE-to-network relay UE) if it was included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

8)	may include the Intermediate relays IE containing the list of User info IDs of the 5G ProSe intermediate UE-to-network relay UEs if it was included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation;

9)	shall include the hop count parameter after being incremented by 1;

10)		shall include the hop limit parameter and set it to the value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, if available;

11)	may include the Accumulated QoS obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, after updating it with the QoS of the PC5 link between the discoverer UE and its 5G ProSe multi-hop child UE-to-network relay UE or its 5G ProSe multi-hop remote UE; and

12)	shall include the PLMN ID set to the PLMN ID value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

d)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

e)	shall set the default destination layer-2 ID as specified in clause 5.2.8 to the destination layer-2 ID and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery solicitation message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2, is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2 and is different from any other self-assigned source layer-2 ID in use for a simultaneous 5G ProSe direct discovery procedure over PC5 with a different discovery model.

f)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to solicit proximity of a connectivity service provided by another 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, the UE shall ensure that it keeps on passing the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation for transmission until the UE is triggered by an upper layer application to stop soliciting proximity of a connectivity service provided by a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop UE-to-network relay UE, or until the UE stops being authorised to perform the discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery. How this is achieved is left up to UE implementation.

NOTE 3:	The discoverer UE can stop discoverer intermediate UE procedure for multi-hop UE-to-network relay discovery for power saving by implementation specific means e.g. an implementation-specific maximum number of 5G ProSe direct links configured in the UE, or an implementation-specific timer expires.

If the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is used to trigger the PROSE PC5 DISCOVERY message signal strength measurement between the UE and by the next 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established, the UE shall start the retransmission timer T5125. If retransmission timer T5125 expires, the UE shall retransmit the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and restart timer T5125. If no response is received from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established after reaching the maximum number of allowed retransmissions, the UE shall trigger relay reselection procedure.

NOTE 4:	The maximum number of allowed retransmissions is UE implementation specific.

For the discoverer UE (the 5G ProSe intermediate UE-to-network relay UE), upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the destination layer-2 ID which the UE is configured to respond for, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to discover, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for multi-hop 5G ProSe UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response.

NOTE 5:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is for 5G ProSe direct discovery based on an indication from the lower layer.

Then if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation; and

b)	the User info ID of the Discoverer UE receiving the message is included in the list of User info IDs in the Multi-hop path info IE of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

then the UE shall initiate the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery by sending PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response towards its 5G ProSe multi-hop child UE-to-network relay UE or 5G ProSe multi-hop remote UE as specified in clause 8b.2.1.3.3.3.

In addition, the UE can measure the signal strength of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response for relay selection or reselection. If the UE has received the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response from the 5G ProSe intermediate UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE with which the UE has a link established, the UE shall stop the retransmission timer T5125 and start the periodic measurement timer T5126.

***** Next change *****

###### 8b.2.1.3.3.2	Discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop UE-to-network relay UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop remote UE.

The UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe multi-hop UE-to-network relay UE in the PLMN or the subscribed SNPN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe multi-hop layer-3 UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1 illustrates the interaction of the UEs in the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1: Discoveree UE procedure for intermediate UE-to-network Relay discovery

When the UE is triggered by an upper layer application to start responding to solicitation on proximity of a connectivity service provided by the 5G ProSe multi-hop UE-to-network relay UE and if the UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, for the relay service code of the connectivity service which the UE is authorized to respond, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation.

NOTE 1:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service;

b)	the target discoveree info parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the user info ID for the multi-hop UE-to-network relay discovery provided by upper layers or same as the configured user info ID for the multi-hop UE-to-network relay discovery as specified in clause 5.2.8, if the target discoveree info parameter is included in the received PROSE PC5 DISCOVERY message; and

c)	if the hop count parameter in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is not greater than the maximum number of allowed hops as specified in clause 5.2.8;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, the UE:

1)	shall set the Discoveree info parameter to the configured User info ID for the multi-hop UE-to-network Relay discovery parameter, as specified in clause 5.2.8;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a UE-to-network relay, and either of the following:

i)	for 5G ProSe multi-hop layer-3 UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop layer-3 remote UE;

3)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

4)	shall include the MIC filed computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.10;

7)	shall include the Multi-hop path info IE and include in it the ordered list of the User info IDs of the selected path, starting from the User info ID of the 5G ProSe multi-hop remote UE and ending with the User info ID of the last 5G ProSe intermediate UE-to-network relay UE;

Editor’s note [5G_ProSe_Ph3, CR#0680]:	It is FFS whether the Layer-2 IDs are included in the Multi-hop path info IE, together with the list of the User info IDs (or as a separate IE), due to the following statement from clause 6.3.2.5.3 in 3GPP TS 23.304: "the Relay Discovery Response message is forwarded along the path, where each 5G ProSe Intermediate UE-to-Network Relay determines the next hop information (e.g. User Info ID and Layer-2 ID) to forward the Relay Discovery Response message".

8)	may include the hop count parameter to indicate the number of hops between the 5G ProSe multi-hop remote UE and the 5G ProSe multi-hop UE-to-network relay UE on the path selected by the 5G ProSe multi-hop UE-to-network relay UE;

9)	may include the Accumulated QoS IE that includes the QoS supported over all the PC5 links along the path from the 5G ProSe multi-hop remote UE to the 5G ProSe multi-hop UE-to-network relay UE; and

10)	shall include the PLMN ID set to its HPLMN ID and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

c)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

d)	shall set the destination layer-2 ID to the source layer-2 ID received from the discoverer UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

NOTE 3:	It is up to the implementation of the 5G ProSe multi-hop UE-to-network relay UE how it decides when to send the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, e.g. due to considering the need to wait till it gets multiple PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation from different paths. The 5G ProSe multi-hop UE-to-network relay UE can decide on the selected path based on different factors such as the PC5 signal strength, number of hops to the 5G ProSe multi-hop remote UE, the path information in the Multi-hop path info IE, the Accumulated QoS for PC5 link.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

NOTE 4:	If the UE is processing a PROSE DIRECT LINK ESTABLISHMENT REQUEST message from the same source layer-2 ID of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, it depends on UE implementation to avoid the conflict of destination layer-2 ID (e.g. send a PROSE DIRECT LINK ESTABLISHMENT REJECT message containing PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected", or ignore the PROSE DIRECT DISCOVERY message for multi-hop UE-to-network relay discovery solicitation).

***** Next change *****

###### 8b.2.1.3.3.3	Discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe intermediate UE-to-network relay UE towards a 5G ProSe multi-hop remote UE; or

b)	a 5G ProSe multi-hop parent UE-to-network relay UE towards a 5G ProSe multi-hop child UE-to-network relay UE.

The UE is authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network relay UE in the PLMN or the subscribed SNPN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be solicited or responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe layer-3 intermediate UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1 illustrates the interaction of the UEs in the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1: Discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery

When the UE is triggered by an upper layer application to start responding to responses of proximity of a connectivity service provided by the 5G ProSe multi-hop UE-to-network relay UE or another 5G ProSe intermediate UE-to-network relay UE and if the UE is authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, for the relay service code of the connectivity service which the UE is authorized to respond, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to respond the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response.

NOTE 1:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service; and

b)	the User info ID of the receiving UE is included in the list of User info IDs included in the Multi-hop path info IE in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, the UE:

1)	shall set the Discoveree info parameter to the User info ID obtained from the Discoveree info included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a multi-hop UE-to-network relay, and either of the following:

i)	for 5G ProSe multi-hop layer-3 UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop layer-3 remote UE;

3)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

4)	shall include the MIC filed computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.10;

7)	shall include the Multi-hop path info IE and include in it the same list of User info IDs that were included in the Multi-hop path info IE included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

8)	may include the hop count parameter set to the same hop count parameter included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

9)	may include the Accumulated QoS IE set to the Accumulated QoS included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

10)	shall include the PLMN ID set to the PLMN ID value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, and in cleartext;

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

NOTE 1A:	For SNPN, the PLMN ID IE includes the PLMN ID of the subscribed SNPN.

c)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

d)	shall set the destination layer-2 ID to the source layer-2 ID received from the discoverer UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

NOTE 3:	If the UE is processing a PROSE DIRECT LINK ESTABLISHMENT REQUEST message from the same source layer-2 ID of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, it depends on UE implementation to avoid the conflict of destination layer-2 ID (e.g. send a PROSE DIRECT LINK ESTABLISHMENT REJECT message containing PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected", or ignore the PROSE DIRECT DISCOVERY message for multi-hop UE-to-network relay discovery solicitation).

***** End of changes *****