3GPP TSG-CT WG1 Meeting #157	C1-256783

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0837 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting ProSe in SNPN for multi-hop UE-to-network relay, updates for relay selection, radio resources provisioning and public warning notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ProSe_NPN |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was agreed in C1-255514 and C1-255516 (CT1#156, Goteberg, August 2025) to extend the support of ProSe in SNPN to include also multi-hop relay procedures in alignment with the requirements in stage-2 spec, where in that case all the UE nodes in all the hops need to be in the subscribed SNPN. The corresponding impact on different multi-hop relay procedures need to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the support of ProSe in SNPN for multi-hop UE-to-network relay in some miscellaneous procedures (Procedure for UE to use provisioned radio resources, relay selection procedure and public warning notification procedure). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for ProSe in SNPN for multi-hop relay scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.4, 8b.2.5.2, 8b.2.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### 8b.2.4	Procedure for UE to use provisioned radio resources for 5G ProSe multi-hop UE-to-network discovery

When the UE is not served by NG-RAN for 5G ProSe multi-hop UE-to-network relay discovery and is authorized to use 5G ProSe multi-hop UE-to-network relay discovery, the UE shall select the corresponding radio parameters to be used for 5G ProSe multi-hop UE-to-network relay discovery as follows:

a)	if the UE can determine itself located in a geographical area and the UE is provisioned with radio parameters for the geographical area, the UE shall select the radio parameters associated with that geographical area; or

b)	in all other cases, the UE shall not initiate 5G ProSe UE-to-network relay discovery.

If the UE intends to use "non-operator managed" radio parameters as specified in clause 5.2.8, the UE shall initiate 5G ProSe multi-hop UE-to-network relay discovery with the selected radio parameters.

If the UE intends to use "operator managed" radio parameters as specified in clause 5.2.8, before initiating 5G ProSe multi-hop UE-to-network relay discovery, the UE shall check with lower layers whether the selected radio parameters can be used in the current location without causing interference to other cells as specified in 3GPP TS 38.331 [13]; and:

a)	if the lower layers indicate that the usage would not cause any interference, the UE shall initiate 5G ProSe multi-hop UE-to-network relay discovery; or

NOTE:	If the lower layers find that there exists a cell operating the provisioned radio resources (i.e., carrier frequency) and the cell belongs to the registered PLMN or a PLMN equivalent to the registered PLMN, or for SNPN, to the RSNPN and the RSNPN is the subscribed SNPN, and the UE is authorized for 5G ProSe multi-hop UE-to-network relay discovery in this PLMN or this RSNPN, the UE can use the radio parameters indicated by the cell as specified in 3GPP TS 38.331 [13].

b)	else if the lower layers report that one or more PLMNs or SNPNs operate in the provisioned radio resources (i.e., carrier frequency) then:

1)	if the following conditions are met:

i)	either none of the PLMNs reported by the lower layers is the registered PLMN or equivalent to the registered PLMN, or for SNPN none of the SNPNs reported by the lower layers is the RSNPN and the RSNPN is not the subscribed SNPN;

ii)	either at least one of the PLMNs reported by the lower layers is in the list of authorized PLMNs for 5G ProSe multi-hop UE-to-network relay discovery and provides radio resources for 5G ProSe multi-hop UE-to-network relay discovery as specified in 3GPP TS 38.331 [13], or for SNPN one of the SNPNs reported by the lower layer is the subscribed SNPN and the UE is authorized to use 5G ProSe direct communication over PC5 interface in the subscribed SNPN; and

iii)	the UE does not have an emergency PDU session;

then the UE shall:

i)	if in 5GMM-IDLE mode, perform PLMN selection or SNPN selection triggered by 5G ProSe multi-hop UE-to-network relay discovery as specified in 3GPP TS 23.122 [14]; or

ii)	else if in 5GMM-CONNECTED mode, either:

A)	perform a De-registration procedure as specified in 3GPP TS 24.501 [11] and then perform PLMN selection or SNPN selection triggered by 5G ProSe multi-hop UE-to-network relay discovery as specified in 3GPP TS 23.122 [14]; or

B)	not initiate 5G ProSe direct discovery.

Whether the UE performs i) or ii) above is left up to UE implementation; or

2)	else the UE shall not initiate 5G ProSe multi-hop UE-to-network relay discovery.

If the registration to the selected PLMN or the selected SNPN is successful, the UE shall proceed with the procedure to initiate 5G ProSe multi-hop UE-to-network relay discovery as specified in clause 8b.2.1.

If the UE is performing 5G ProSe multi-hop UE-to-network relay discovery using radio parameters associated with a geographical area and moves out of that geographical area, the UE shall stop performing 5G ProSe multi-hop UE-to-network relay discovery and then if the UE is not served by NG-RAN for 5G ProSe multi-hop UE-to-network relay discovery, the UE shall select appropriate radio parameters for the new geographical area as specified above.

***** Next change *****

#### 8b.2.5.2	Multi-hop UE-to-network relay selection procedure initiation

The 5G ProSe multi-hop remote UE or the 5G ProSe intermediate UE-to-network relay UE shall trigger the multi-hop UE-to-network relay selection procedure if the following conditions are met:

a)	the UE is authorised to act as a 5G ProSe multi-hop remote UE or as a 5G ProSe intermediate UE-to-network relay UE as specified in clause 5.2.8 (e.g., relay service code, User info ID, etc.):

1)	if the authorized UE is expected to use 5G ProSe multi-hop layer-3 UE-to-network relay, the indication whether the UE is authorized to use a 5G ProSe multi-hop layer-3 UE-to-network relay UE shall be set;

2)	if the authorized UE is expected to use 5G ProSe multi-hop layer-2 UE-to-network relay, either the subset of the PLMN(s) provided by the 5G ProSe layer-2 UE-to-network relay UE shall be in the list of PLMNs in which the UE is authorized to use a 5G ProSe multi-hop layer-2 UE-to-network relay UE; or the SNPN provided by the 5G ProSe multi-hop layer-2 UE-to-network relay UE shall be the subscribed SNPN in which the UE is authorized to use a 5G ProSe multi-hop layer-2 UE-to-network relay UE and

3)	if the authorized UE is expected to use 5G ProSe multi-hop layer-2 UE-to-network relay, either the authorized UE selected PLMN shall be in the list of PLMNs in which the UE is authorized to use a 5G ProSe multi-hop layer-2 UE-to-network relay UE and in the subset of the PLMN(s) provided by the 5G ProSe multi-hop layer-2 UE-to-network relay UE, or the authorized UE selected SNPN shall be the subscribed SNPN in which the UE is authorized to use a 5G ProSe multi-hop layer-2 UE-to-network relay UE and provided by the 5G ProSe multi-hop layer-2 UE-to-network relay UE;

b)	the UE has obtained a list of 5G ProSe multi-hop UE-to-network relay UE candidate(s) fulfilling ProSe layer criteria with the monitoring procedure for multi-hop UE-to-network relay discovery as specified in clause 8b.2.1.2.3 or the discoverer procedure for UE-to-network relay discovery as specified in clause 8b.2.1.3.1; and

c)	the UE has obtained a list of 5G ProSe multi-hop UE-to-network relay UE candidate(s) fulfilling lower layers criteria as specified in 3GPP TS 38.331 [13].

***** Next change *****

#### 8b.2.9.2	5G ProSe public warning notification relay initiation procedure for 5G ProSe layer-3 multi-hop UE-to-network relay

The 5G ProSe layer-3 multi-hop UE-to-network relay UE broadcasts the 5G ProSe broadcasting message for public warning notification if the 5G ProSe layer-3 multi-hop UE-to-network relay UE receives the public warning notification message from the network as specified in 3GPP TS 23.041 [54].

NOTE:	The 5G ProSe layer-3 multi-hop UE-to-network relay UE can perform the duplication detection function as specified in 3GPP TS 23.041 [54] to suppress received duplicated warning messages over Uu in order to avoid broadcasting the duplicated warning message over PC5.

The 5G ProSe layer-3 multi-hop UE-to-network relay UE shall start broadcasting the received public warning notification message if:

a)	the 5G ProSe layer-3 multi-hop UE-to-network relay UE is served by NG-RAN for 5G ProSe communication;

b)	the 5G ProSe layer-3 multi-hop UE-to-network relay UE is authorised to act as a 5G ProSe layer-3 multi-hop UE-to-network relay UE;

c)	either the registered PLMN is in the list of PLMNs in which the UE is authorized to use 5G ProSe communication over PC5, or the registered SNPN is the subscribed SNPN in which the UE is authorized to use 5G ProSe communication over PC5, when the UE is served by NG-RAN for 5G ProSe communication over PC5 as specified in clause 5.2.4; and

d)	the 5G ProSe layer-3 multi-hop UE-to-network relay UE is configured with the default destination layer-2 ID for broadcasting warning messages and the PC5 QoS parameters for broadcasting warning messages.

The UE shall generate the PROSE PC5 BROADCASTING message for public warning notification which includes:

a)	the warning message received from the network; and

b)	the hop count with the value set to 1.

After generating the PROSE PC5 BROADCASTING message for public warning notification, the UE shall include the PROSE PC5 BROADCASTING message for public warning notification in a protocol data unit with following parameters:

a)	a layer-3 protocol data unit type (see 3GPP TS 38.323 [16]) set to unstructured;

b)	the source layer-2 ID set to the layer-2 ID self-assigned by the UE for 5G ProSe communication over PC5 as specified in clause 7.3.2.1.2;

c)	the destination layer-2 ID set to the destination layer-2 ID configured for 5G ProSe broadcasting message for public warning notification as specified in clause 5.2.8;

d)	the PQFI set to the value corresponding to the configured PC5 QoS parameters for 5G ProSe broadcasting message for public warning notification as specified in clause 5.2.8; and

e)	the NR Tx profile based on the configuration for 5G ProSe broadcasting message for public warning notification as specified in clause 5.2.8.

Then the UE shall request radio resources for 5G ProSe communication over PC5 as specified in 3GPP TS 38.300 [21] and pass the protocol data unit(s) on the PC5 QoS flow identified by the PQFI to lower layers for transmission.

If the UE is camped on a serving cell indicating that 5G ProSe communication over PC5 is supported by the network, but not broadcasting any carrier frequencies and radio resources for 5G ProSe communication over PC5 as specified in 3GPP TS 38.331 [13], the UE shall request radio resources for 5G ProSe communication over PC5 as specified in 3GPP TS 24.501 [11].

***** End of changes *****