3GPP TSG-CT WG4 Meeting #131	C4-254301

# Sophia Antipolis, France; 13th – 17th October 2025							   Revision of C4-254119

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0905 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Offload Identifiers for I-SMF based Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) I-SMF based Local Offloading Management supports preconfiguring I-SMFs with offload identifers and their respective local offloading management information. TS 29.502 contains inconsistent requirements regarding whether the stored offloadIds signaled by an (target) I-SMF to the SMF or (source) I-SMF include or not the list of Offload Ids which are preconfigured at the I-SMF: - In Table 6.1.6.2.81-1 (LocalOffloadingMgtInfoFromIsmf): the text explicitly says it does. - in Table 6.1.6.2.7-1 (SmContextRetrieveData): the text suggests it does not (the text only covers the case of offload Ids learned from previous procedures). 2) Clause 5.2.2.6.1 (Retrieve SM Context service operation) misses to capture requirements about signaling the stored OffloadIds in the request and retrieving accordingly offloadIds and/or local offloading management information in the response, although corresponding attributes have been defined in corresponding messages. 3) It should be clarified that the SMF shall not provide localOffloadingInfo to an I-SMF containing an offload identifier preconfigured at the I-SMF. 4) Various editorials to improve the readability of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) The stored offloadIds signaled by an (target) I-SMF to the SMF or (source) I-SMF include the list of Offload Ids preconfigured at the I-SMF. 2) Clause 5.2.2.6.1 is updated to reflect requirements about signaling the stored OffloadIds in the request and retrieving accordingly offloadIds and/or local offloading management information in the response. 3) It is clarified that the SMF shall not provide a localOffloadingInfo to an I-SMF containing an offload identifier preconfigured at the I-SMF. 4) Various editorials improving the readability of the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing and inconsistent requirements about the use of stored Offload Ids causing interoperability issues and failure to establish PDU session with I-SMF with the expected local offloading management policies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.6.1, 6.1.6.2.7, 6.1.6.2.39, 6.1.6.2.82 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: New NOTE X in Table 6.1.6.2.82-1 |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

#### 5.2.2.6	Retrieve SM Context service operation

##### 5.2.2.6.1	General

The Retrieve SM Context service operation shall be used to retrieve an individual SM context, for a given PDU session, from the (H-)SMF, from the V-SMF during change or removal of V-SMF, or from the I-SMF during change or removal of I-SMF.

It is used in the following procedures:

-	5GS to EPS handover using N26 interface (see clause 4.11.1.2.1 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	5GS to EPS Idle mode mobility using N26 interface (see clause 4.11.1.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	UE Triggered Service Request with I-SMF insertion/change/removal or with V-SMF insertion/change/removal (see clause 4.23.4.3 of of 3GPP TS 23.502 [3]);

-	Xn based inter NG-RAN handover with insertion of intermediate SMF (see clause 4.23.11 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	Inter NG-RAN node N2 based handover, preparation phase, with I-SMF or V-SMF insertion/change (see clause 4.23.7.3.2 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	SMF Context Transfer procedure, LBO or no Roaming, no I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]), for PDU sessions associated with 3GPP access;

-	I-SMF selection per DNAI (see clause 4.23.5.4 of 3GPP TS 23.502 [3]);

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions (see clause 4.3.5.2 of 3GPP TS 23.502 [3]);

-	Network triggered EAS change in HR-SBO context (see clause 6.7.3.2 of 3GPP TS 23.548 [74]);

-	N2 Handover with V-SMF insertion/change/removal in HR-SBO case (see clause 6.7.2.6 of 3GPP TS 23.548 [39]);

-	Inter V-SMF mobility registration update procedure in HR-SBO case (see clause 6.7.2.7 of 3GPP TS 23.548 [39]);

-	Xn Handover with V-SMF change in HR-SBO case (see clause 6.7.2.9 of 3GPP TS 23.548 [39]).

The NF Service Consumer (e.g. AMF or SMF) shall retrieve an SM context by using the HTTP POST method (retrieve custom operation) as shown in Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1: SM context retrieval

1.	The NF Service Consumer shall send a POST request to the resource representing the individual SM context to be retrieved. The POST request may contain a content with the following parameters:

-	target MME capabilities, if available, to allow the SMF to determine whether to include EPS bearer contexts for Ethernet PDN Type, non-IP PDN type, or requiring UP integrity protection or not;

-	SM context type:

-	indicating that this is a request to retrieve the complete SM context (i.e. 5G SM context including EPS context information as defined in clause 6.1.6.2.39), during scenarios with an I-SMF or V-SMF insertion/change/removal or SMF Context Transfer procedure; or

-	indicating that this is a request to retrieve the AF Coordination Information as defined in clause 6.1.6.2.69, during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled.

-	serving core network operator PLMN ID of the new V-SMF, when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change or insertion. Or the serving core network operator PLMN ID of the new I-SMF during the procedure with an I-SMF insertion;

-	notToTransferEbiList IE, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, to request the SMF to not transfer EPS bearer context(s) corresponding to EBIs in the list, during an 5GS to EPS mobility when the target MME does not support 15 EPS bearers;

-	ranUnchangedInd IE, if the NG-RAN Tunnel info is required in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover, I-SMF selection/removal per DNAI or V-SMF selection per DNAI, when the UE is in CM-CONNECTED state as specified in clauses 5.2.2.2.7 and 5.2.2.2.12;

-	the hrsboSupportInd IE set to true, if the procedure is triggered by a target V-SMF supporting the HR-SBO feature;

-	the storedOffloadIds IE, if the target V-SMF has a list of stored offload identifiers from the previous procedures for the HPLMN of the PDU session;

-	the ismfLomSupportInd IE set to true, if the procedure is triggered by a target I-SMF supporting the I-SMF Local Offloading Management feature; and/or

-	the storedOffloadIds IE, if the target I-SMF has a list of stored offload identifiers (for the PLMN of the PDU session), which are preconfigured at the target I-SMF or which were learned from anchor SMFs of the same PLMN during previous procedures.

2a.	On success, "200 OK" shall be returned; the content of the POST response shall contain the mapped EPS bearer contexts if this is a request for the UE EPS PDN connection, or the complete SM context if this is a request for retrieving the complete SM context, or the AF Coordination Information if this is a request for retrieving the AF Coordination Information.

If this is a request for the UE EPS PDN connection and the target MME capabilities were provided in the request parameters:

-	if the target MME supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Unstructured", an EPS bearer context with the "non-IP" PDN type;

-	if the target MME supports the Ethernet PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "Ethernet" PDN type;

-	if the target MME does not support the Ethernet PDN type but supports the non-IP PDN type, the SMF shall return, for a PDU session with PDU session type "Ethernet", an EPS bearer context with the "non-IP" PDN type.

If the notToTransferEbiList IE was included in the request, the SMF shall not provide EPS bearer context(s) corresponding to EBIs in the list.

If this is a request for retrieving the complete SM context and there are downlink data packets buffered at I-UPF, the SMF shall include the "forwardingInd" attribute with value "true" in the response body to indicate downlink data packets are buffered at the I-UPF. The NF Service Consumer receiving the "forwardingInd" attribute with the value "true" shall setup a forwarding tunnel for receiving the buffered downlink data packets.

If this is a request for retrieving the complete SM context for an inter-PLMN V-SMF change, i.e. if the request contains the serving core network operator PLMN ID indicating a different PLMN than the PLMN of the SMF (acting as the old V-SMF), the latter shall not include the chargingInfo IE and the roamingChargingProfile IE in the SM context returned in the response.

During a procedure with an I-SMF or V-SMF insertion, the anchor SMF should use the servingNetwork IE received in the Retrieve SM Context Request to determine whether the inserted entity is an I-SMF or V-SMF, and if so, encode in the SM Context returned in the response the applicable set of attributes (e.g. hsmfUri, hSmfInstanceId, hSmfServiceInstanceId to a V-SMF, or smfUri, smfInstanceId, smfServiceInstanceId to an I-SMF) and the applicable URI in the pduSessionRef if different URIs are used for intra-PLMN and inter-PLMN signaling requests targeting the PDU session context.

NOTE 1:	During an inter-PLMN procedure with an I-SMF or V-SMF change, the old V-SMF or I-SMF returns the attributes of the SM context as were received from the anchor SMF.

If the UE, target MME and AMF support User Plane integrity protection with EPS, the SMF shall include the UP Security Policy IE in the UE EPS PDN connection context if User Plane integrity protection has been enabled by the SMF as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of 3GPP TS 23.502 [3].

NOTE 2:	During a 5GS to EPS handover, if User Plane Integrity Protection is used in 5GS and QoS flows are associated with EPS bearer IDs, the I-SMF/V-SMF considers that the UE supports User Plane Integrity Protection in EPS.

If this is a request for retrieving the complete SM context for an I-SMF or V-SMF insertion, and the smfUri IE or hSmfUri IE is provided by the AMF in the Create SM Context request and is different from the smfUri IE or hSmfUri IE in the SM context returned in the Retrieve SM Context response, the latter (i.e. the IEs received in the Retrieve SM Context response) shall prevail and be used by the I-SMF or V-SMF to trigger the create service operation to the (H-)SMF.

If the target V-SMF supports the HR-SBO feature, for a HR-SBO session, the content of the POST response message may also contain the HR-SBO related information, e.g., hrsboAuthResult, hDnsAddr, hPlmnAddr, vplmnOffloadingInfoList.

The source V-SMF shall determine if the target V-SMF has already received the corresponding VPLMN Offloading Information for an offload identifier based on the storedOffloadIds for the HPLMN of the PDU session provided by the target V-SMF in the Retrieve SM context request message for an intra PLMN V-SMF change. The source V-SMF shall then include:

-	the Offload Identifier(s) only, if the Offload Identifier(s) are known by the target V-SMF, i.e., the Offload Identifier(s) are part of the storedOffloadIds; otherwise

-	a list of vplmnOffloadingInfo attributes where each vplmnOffloadingInfo contains an offload identifier and the corresponding VPLMN Offloading Information.

The source I-SMF shall determine if the target I-SMF knows the Local Offloading Information for an offload identifier based on the storedOffloadIds provided by the target I-SMF in the Retrieve SM context request message; the source I-SMF shall then include:

-	the Offload Identifier(s) only, if the Offload Identifier(s) are known by the target I-SMF, i.e., the Offload Identifier(s) are part of the storedOffloadIds; otherwise

-	a list of LocalOffloadingManagementInfo attributes where each LocalOffloadingManagementInfo contains an offload identifier and the corresponding Local Offloading Management Information, for offload IDs which are not preconfigured at the source I-SMF.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.3.4.4.2-2.

If the EBI value of the QoS Flow associated with the default QoS Rule is included in the notToTransferEbiList IE, the SMF shall set the "cause" attribute in the ProblemDetails structure to "DEFAULT_EBI_NOT_TRANSFERRED".

If a request for the UE EPS PDN connection is rejected due to the target MME not being capable to support the PDU session, e.g. if the PDU session requires UP integrity protection but the target MME does not support User Plane Integrity Protection with EPS, the SMF shall return a 403 Forbidden response with the "cause" attribute in the ProblemDetails structure set to " TARGET_MME_CAPABILITY".

* * * Next Change * * *

##### 6.1.6.2.7	Type: SmContextRetrieveData

Table 6.1.6.2.7-1: Definition of type SmContextRetrieveData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetMmeCap | MmeCapabilities | C | 0..1 | This IE shall be present if it is available. When present, it shall contain the target MME capabilities. |  |
| smContextType | SmContextType | C | 0..1 | This IE shall be present if this is a request to retrieve the complete SM context, during scenarios with an I-SMF or V-SMF insertion/change/removal, or during SMF Context Transfer procedure for LBO or non-roaming PDU session without I-SMF (see clause 4.26.5.3 of 3GPP TS 23.502 [3]). This IE shall also be present if this is a request to retrieve the AF Coordination Information during the change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions, if the runtime coordination between old SMF and AF is enabled (see clause 4.3.5.2 of 3GPP TS 23.502 [3]). | DTSSA, CTXTR EnEDGE |
| servingNetwork | PlmnId | C | 0..1 | This IE shall be present when the procedure is triggered by a new V-SMF, if the new V-SMF supports inter-PLMN V-SMF change or insertion. This IE shall also be present during the procedure with an I-SMF insertion. When present, this IE shall contain the serving core network operator PLMN ID of the NF Service Consumer (i.e. new V-SMF or new I-SMF). | DTSSA |
| notToTransferEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present, if the SM context type IE is absent or indicate a request to retrieve the EPS PDN connection, and the AMF determines that certain EPS bearers shall not to be transferred to EPS during a 5GS to EPS mobility procedure, as specified in clause 4.11.1 of 3GPP TS 23.502 [3]. When present, it shall contain the EBI list not to be transferred. |  |
| ranUnchangedInd | boolean | C | 0..1 | This IE shall be present if AN Tunnel is required, in scenario of I-SMF/V-SMF change/insertion during registration procedure after EPS to 5GS handover or I-SMF selection per DNAI, when UE is in CM-CONNECTED state (see clause 5.2.2.6.1). When present, it shall be set as follows: - true: NG-RAN is not changed and the tunnel information is required; - false (default):NG-RAN is changed and the tunnel information is not required. | DTSSA |
| hrsboSupportInd | boolean | C | 0..1 | This IE shall be present if the procedure is triggered by a new V-SMF which supports the HR-SBO feature. When present, it shall be set as follows: - true: HR-SBO is supported; - false (default): HR-SBO is not supported. |  |
| ismfLomSupportInd | boolean | C | 0..1 | This IE shall be present if the procedure is triggered by a new I-SMF which supports the ISMF-LOM feature. When present, it shall be set as follows: - true: I-SMF Local Offloading Management is supported; - false (default): I-SMF Local Offloading Management is not supported. |  |
| storedOffloadIds | array(OffloadIdentifier) | C | 1...N | The IE shall be present when: - the target V-SMF has a list of the stored offload identifiers for the HPLMN of the PDU session from previous procedures (for the same or different PDU sessions) with the HPLMN; or - the target I-SMF has a list of the stored offload identifiers (for the PLMN of the PDU session) which are preconfigured at the target I-SMF or which were learned from anchor SMFs of the same PLMN during previous procedures (for the same or different PDU sessions). When present, this IE shall contain the list of offload identifiers known by the target V-SMF/I-SMF for the HPLMN (for HR-SBO) or PLMN (for I-SMF-LOM) of the PDU session. (NOTE) | HR-SBO, ISMF-LOM |
| NOTE: The stored Offload identifiers shall be included in any subsequent request message to the source V-SMF/I-SMF for other HR-PDU sessions (for HR-SBO) or other PDU sessions with I-SMF (for I-SMF-LOM). |  |  |  |  |  |

* * * Next Change * * *

##### 6.1.6.2.39	Type: SmContext

Table 6.1.6.2.39-1: Definition of type SmContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSessionId | PduSessionId | M | 1 | This IE shall contain the PDU Session ID. |  |
| dnn | Dnn | M | 1 | This IE shall contain the UE requested DNN of the PDU session. The DNN shall be the full DNN (i.e. with both the Network Identifier and Operator Identifier) for a HR PDU session, and it should be the full DNN in LBO and non-roaming scenarios. If the Operator Identifier is absent, the serving core network operator shall be assumed. |  |
| *** text skipped for clarity *** |  |  |  |  |  |
| offloadIds | array(OffloadIdentifier) | C | 1...N | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO, and if the offloadIds are part of the storedOffloadIds included in the the Retrieve SM Context Request. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain a list of offload identifiers that apply to the PDU session and that are already known by the target V-SMF/I-SMF. (NOTE 5) | HR-SBO, ISMF-LOM |
| easInfoToRefresh | EasInfoToRefresh | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the EAS information to be refreshed for EAS re-discovery. | HR-SBO, ISMF-LOM |
| targetDnai | Dnai | C | 0..1 | This IE shall be present, for a HR PDU session, if it is received from H-SMF in AF triggered EAS re-discovery and edge relocation via interacting with HPLMN (see clause 6.7.3.2 of 3GPP TS 23.548 [74]). When present, this IE shall contain the target DNAI. | HR-SBO |
| trafficInfluInfo | TrafficInfluenceInfo | C | 0..1 | This IE shall be present for a HR PDU session, during a V-SMF change within the same PLMN (i.e. if the Retrieve SM Context Request does not contain the servingNetwork attribute set to a different PLMN ID), if available, HR-SBO was authorized by the H-SMF and the request indicates that the new V-SMF supports HR-SBO. This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the traffic influence information applicable at the VPLMN for the HR-SBO PDU session, or applicable to the PDU session with I-SMF. | HR-SBO, ISMF-LOM |
| pendingUpdateInfoList | array(PendingUpdateInfo) | O | 1..N | This IE should be included by the old V-SMF/I-SMF if received from the (H-)SMF. When present, this IE shall indicate the list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. |  |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if I-SMF based local offloading management was allowed by the SMF and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall Indicate whether I-SMF based local offloading management was allowed by the SMF - true: allowed - false: not allowed | ISMF-LOM |
| dnsAddr | IpAddress | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain the DNS server address to be used for DNS requests related with traffic not to be subject to Local Offloading Management. | ISMF-LOM |
| psaUpfAddr | IpAddress | C | 0..1 | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, this IE shall contain the PSA UPF address information (e.g. PSA UPF IP address on N6) to be used by the I-SMF to configure the EASDF to build EDNS Client Subnet option for target FQDN of DNS queries which are not authorized for Local Offloading Management. | ISMF-LOM |
| localOffloadingInfoList | array(LocalOffloadingManagementInfo) | C | 1..N | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the Local Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the target I-SMF and are not preconfigured at the source I-SMF. (NOTE 5) | ISMF-LOM |
| priorityUserInd | boolean | C | 0..1 | This IE shall be present during intra VPLMN V-SMF change scenario, if previously received from the AMF or from old V-SMF. Presence of this IE with the value false shall be prohibited. (NOTE 6) |  |
| NOTE 1: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 2: See NOTE 7 of Table 6.1.6.2.10-1. NOTE 3: If the anchorSmfOauth2Required IE was received in SmContextCreateData from the AMF, this IE shall be ignored by the new I-SMF or V-SMF. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 5: The same offloadId should not appear in both the offloadIds IE and the vplmnOffloadingInfoList IE (for HR-SBO) or the localOffloadingInfoList IE (for ISMF-LOM). Both the vplmnOffloadingInfoList IE (for HR-SBO) or the localOffloadingInfoList IE (for ISMF-LOM) and the offloadIds IE may be present when multiple offload identifers apply to the PDU session and some of them are already known by the target V-SMF or I-SMF while others are not yet known by the target V-SMF or I-SMF. NOTE 6: For a HR PDU Session, if the VPLMN QoS Constraints is enabled and the VPLMN QoS Constraints for high priority PDU sessions and normal PDU sessions are different, the V-SMF shall determine the VPLMN QoS Constraints applicable for this PDU session based on the priority user indication. |  |  |  |  |  |

* * * Next Change * * *

##### 6.1.6.2.82	Type: LocalOffloadingMgtInfoToIsmf

Table 6.1.6.2.82-1: Definition of type LocalOffloadingMgtInfoToIsmf

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present in the Create or Update Response, if the localOffloadingMgtAllowedInd IE was present and set to true in the request. When present, this IE shall indicate whether the SMF allows the I-SMF to perform Local Offloading Management: - true: allowed - t false: not allowed |
| dnsAddr | IpAddress | O | 0..1 | When present, this IE shall contain the DNS server address to be used for DNS requests related with traffic not to be subject to Local Offloading Management. |
| psaUpfAddr | IpAddress | O | 0..1 | When present, this IE shall contain the PSA UPF address information (e.g. PSA UPF IP address on N6) to be used by the I-SMF to configure the EASDF to build EDNS Client Subnet option for target FQDN of DNS queries which are not authorized for Local Offloading Management. |
| localOffloadingInfoList | array(LocalOffloadingManagementInfo) | O | 0..N | When present, this IE shall include Local Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the I-SMF service instance or whose offload identifier's version has changed. (NOTE 2) (NOTE 3) (NOTE X) |
| offloadIds | array(OffloadIdentifier) | C | 0..N | When present, this IE shall include a list of specific Offload Ids applicable for the PDU session and that are part of the StoredOffloadIds included in the corresponding request message. (NOTE 2) (NOTE 3) (NOTE X) |
| trafficInfluInfo | TrafficInfluenceInfo | O | 0..1 | When present, it shall include AF traffic influence information that applies at the I-SMF to the PDU session. |
| NOTE 1: The SMF sends the EAS rediscovery indication and EAS information to be refreshed for EAS re-discovery received from the I-SMF to the UE in n1SmInfoToUe attribute. NOTE 2: When the SMF determines that the I-SMF service instance already knows the corresponding Local Offloading Info based on the storedOffloadIds provided by the I-SMF service instance in the corresponding request message and the associated localOffloadInfo has not changed, then in the corresponding response message, the SMF shall include the corresponding offload identifier in the offloadIds attribute; otherwise, the SMF shall include it in the localOffloadingInfoList attribute. The SMF shall always provision a complete list of localOffloadingInfo and/or offloadIds. NOTE 3: The I-SMF service instance shall always overwrite any localOffloadingInfoList and the offloadIds stored for the PDU session with the latest localOffloadingInfoList and/or offloadIds received. The SMF may request the I-SMF to remove all localOffloadingInfoList provisioned earlier for a PDU session with an I-SMF by provisioning an empty localOffloadingInfoList array. When the I-SMF receives a LocalOffloadingInfo containing an offloadId which is known by the I-SMF and which does not correspond to an offload identifier preconfigured at the I-SMF, it shall consider that the localOffloadingInfo for the offloadId has changed if the offloadId does not contain a version field or if the offloadId has an incremented version number, and if so, the I-SMF service instance shall enforce the changed localOffloadingInfo for the PDU session with an I-SMF and also for any other PDU sessions with an I-SMF with the same offloadId. NOTE X: If, based on configuration, the PCF is aware that the I-SMF is preconfigured with the Local Offloading Management Policy information, the PCF does only provide the SMF with the offload Identifier, without the Local Offloading Management Policy information, as defined in clause 4.2.2.26 of 3GPP TS 29.512 [30]. Accordingly, the SMF only provides an offload identifier preconfigured at the I-SMF within the offloadIds IE and not within the localOffloadingInfoList IE. |  |  |  |  |

* * * End of Changes * * *

* * * For information * * *

##### 6.1.6.2.81	Type: LocalOffloadingMgtInfoFromIsmf

Table 6.1.6.2.81-1: Definition of type LocalOffloadingMgtInfoFromIsmf

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| localOffloadingMgtAllowedInd | boolean | C | 0..1 | This IE shall be present with the value true in the Create Request (during an I-SMF insertion) and in the Update Request (during an I-SMF change), if the inserted/changed I-SMF is allowed (by the AMF) to apply Local Offloading Management. It shall also be present in an Update Request if the local offloading management allowed indication information needs to be changed, e.g. due to the AMF indicating to the I-SMF that local offloading management is newly allowed or no longer allowed. When present, it shall be set as follows: true: Local Offloading Management is allowed false: Local Offloading Management is not allowed When this IE is received with the value true, the SMF shall not discover, select and configure EASDF as well as DNS message handling for the locally offloaded traffic towards the local part of DN. When this IE is received with the value false in an Update Request (with or without an I-SMF change), or when this IE is not received in an Update Request during an I-SMF change, the SMF shall consider that I-SMF based local offloading management does not apply anymore to the PDU session (if this was applying beforehand) and the SMF shall proceed as specified in clause 6.10.2.4 of 3GPP TS 23.548 [39]). |
| easdfAddr | IpAddress | O | 0..1 | When present, this IE shall contain the IP address of the EASDF or of the new EASDF if the EASDF needs to be changed e.g. as a result of an I-SMF change. |
| easdfSecurityInfo | array(DnsServerSecurityInformation) | O | 1..N | When present, this IE shall contain one or more DNS server security information with length of two octets container(s), set with the EASDF security information. (NOTE 2) |
| easRediscoveryInd | boolean | C | 0..1 | This IE shall be present in the Update Request if the UE indicates the support of EAS rediscovery and the I-SMF needs to trigger an EAS rediscovery. When present, this IE shall be set to true to indicate that EAS rediscovery is requested. |
| easInfoToRefresh | EasInfoToRefresh | O | 0..1 | This IE may be present in the Update Request if the easRediscoveryInd is set to true. When present, it shall contain the EAS information to be refreshed for EAS re-discovery. |
| storedOffloadIds | array(OffloadIdentifier) | C | 1...N | The IE shall be present when the I-SMF service instance has stored a list of offload identifiers which are preconfigured at the I-SMF or which were received from any SMF (of the same PLMN) during previous signalling procedures for this PDU session with an I-SMF and also for other PDU sessions with an I-SMF. When present, this IE shall contain the list of offload identifiers that are known by the I-SMF service instance. (NOTE 1) |
| NOTE 1: The SMF assumes that the I-SMF does not know any Offload Identifiers if this IE is absent. NOTE 2: The I-SMF shall ensure that the EASDF security information that it provides to the SMF is consistent with the DNS security protocol(s) supported by the UE that is received in the PCO IE from the UE. |  |  |  |  |