3GPP TSG CT Meeting #110	CP-253190

Baltimore, United States, 8 - 9 December, 2025	(revision of C4-255423)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.369 | CR | 0008 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New ADM security service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Samsung, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo, Samsung, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 agreed in CR0035 to TS 33.369 (see S3-253801) on new security related services provided by the ADM: Accordingly, this CR specifies the new Nadm_Sec service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose new clauses for Nadm_Sec service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT stage 3 specification not aligned with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5, 5.1, 5.3 and subclauses (new), 6.2 and subclauses (new), A.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29369_AIoT_Data.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1 - Moved resource definition to clause 6.2.4 - Changes to dashed boxes - Use verb in the beginning of custom operations - Remove T-ID n+1 - Reuse definition of Tid and TidHandlingInformation from Nadm_DM API - Reuse definiton of Rand and Xres - Corrected boolean type - Added cosigners Revision2 - added refences - fixed alpha.1 - correct company name Revision3 - Update OpenAPI version and externalDocs |  |  |  |  |  |  |  |  |

# *** First Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 9113: "HTTP/2".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 9457: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[15]	IETF RFC 9110: "http Semantics".

[16]	IETF RFC 9111: "HTTP Caching".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[BB]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[xx]	3GPP TS 33.369: "Security aspects of Ambient IoT service for isolated private networks".

# *** Next Change ***

5	Services offered by the ADM

5.1	Introduction

The ADM offers the following services via the Nadm interface:

-	Nadm_DM Service

-	Nadm_Sec

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nadm_DM | 6.1 | ADM Data Management | TS29369_Nadm_DM.yaml | nadm-dm | A.2 |
| Nadm_Sec | 6.2 | ADM Security Service | TS29369_Nadm_Sec.yaml | nadm-sec | A.3 |

# *** Next Change ***

5.3	Nadm_Sec Service

5.3.1	Service Description

See 3GPP TS 33.369 [xx].

5.3.2	Service Operations

5.3.2.1	Introduction

For the Nadm_Sec service the following service operations are defined:

-	RAND_Get

-	Authentication_Get

-	SessionKey_Get

-	TID_Get

The Nadm_Sec Service is used by the AIOTF to request the ADM to provide a random number (RAND), authentication data for a single AIoT device or a group of AIoT devices, the session key for an AIoT device, and the T-ID for an AIoT device.

5.3.2.2	RAND_Get

5.3.2.2.1	General

The following procedures using the RAND_Get service operation are supported:

-	RAND Retrieval

5.3.2.2.2	RAND Retrieval

Figure 5.3.2.2.2-1 shows a scenario where the NF service consumer (e.g. AIOTF) sends a request to the ADM to receive a random number (see 3GPP TS 33.369 [xx]).

Figure 5.3.2.2.2-1: RAND Retrieval

1.	The NF service consumer sends a POST request (custom method: get-rand).

2.	The ADM responds with "200 OK" with the message body containing the generated random number.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

5.3.2.3	Authentication_Get

5.3.2.3.1	General

The following procedures using the Authentication_Get service operation are supported:

-	Authentication Data Retrieval

5.3.2.3.2	Authentication Data Retrieval

Figure 5.3.2.3.2-1 shows a scenario where the NF service consumer (e.g. AIOTF) sends a request to the ADM to receive the authentication data for a single AIoT device or a group of AIoT devices (see 3GPP TS 33.369 [xx]).

Figure 5.3.2.3.2-1: Authentication Data Retrieval

1.	The NF service consumer sends a POST request (custom method: get-authentication) to the ADM, including authentication input parameters for a single AIoT Device, identified by an AIoT device permanent identifier, or a group of AIoT Devices, identified by Filter Information.

2a.	The ADM responds with "200 OK" with the message body containing the derived authentication data.

2b.	If the AIoT device permanent identifier or Filtering Information is not recognized, HTTP status code "404 Not Found" may be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the group of devices derived from the Filtering Information is too large to compute the expected authentication data (XRESAIOT values) in a reasonable amount of time, HTTP status code "413 Payload Too Large" may be returned. The ADM shall include the application error "FILTER_INFORMATION_TOO_BROAD" in the ProblemDetails element to indicate that the filtering criteria resulted in an unmanageable device set.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

5.3.2.4	SessionKey_Get

5.3.2.4.1	General

The following procedures using the SessionKey_Get service operation are supported:

-	Session Key Retrieval

Figure 5.3.2.4.2-1: Session Key Retrieval

1.	The NF service consumer sends a POST request (custom method: get-session-key) to the ADM, including input parameters for an AIoT Device, identified by an AIoT device permanent identifier.

2a.	The ADM responds with "200 OK", with the message body containing the derived Session Key.

2b.	If the AIoT device permanent identifier is not recognized, HTTP status code "404 Not Found" may be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

5.3.2.5	TID_Get

5.3.2.5.1	General

The following procedures using the TID_Get service operation are supported:

-	TID Retrieval

5.3.2.5.2	TID Retrieval

Figure 5.3.2.5.2-1 shows a scenario where the NF service consumer (e.g. AIOTF) sends a request to the ADM to receive the TID (see 3GPP TS 33.369 [xx]).

Figure 5.3.2.5.2-1: TID Retrieval

1.	The NF service consumer sends a POST request (custom method: get-tid) to the ADM, including input parameters such as the AIoT device permanent identifier and, optionally, a resynchronization indicator.

2a. The ADM responds with "200 OK", with the message body containing the allocated Temporary ID (T-ID) and the associated T-ID handling information.

2b.	If the AIoT device permanent identifier is not recognized, HTTP status code "404 Not Found" may be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

# *** Next Change ***

6.2	Nadm_Sec Service API

6.2.1	Introduction

The Nadm_Sec shall use the Nadm_Sec API.

The API URI of the Nadm_Sec API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nadm-sec".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

6.2.2	Usage of HTTP

6.2.2.1	General

HTTP/2, IETF RFC 9113 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nadm_Sec API is contained in Annex A.

6.2.2.2	HTTP standard headers

6.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

6.2.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [13].

6.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

6.2.3	Resources

6.2.3.1	Overview

There are no resources defined for this API in this release of the specification.

6.2.4	Custom Operations without associated resources

6.2.4.1	Overview

This clause describes the structure for custom operations without associated resources.

Figure 6.2.4.1-1 depicts the custom operation URIs structure for the Nadm_Sec API.

Figure 6.2.4.1-1: Custom operation URI structure of the Nadm_Sec API

Table 6.2.4.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 6.2.4.1-1: Custom operations without associated resources

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| getRand | /get-rand | POST | Retrieve a random number from ADM |
| getAuthentication | /get-authentication | POST | Retrieve authentication data from ADM |
| getSessionKey | /get-session-key | POST | Retrieve a session key from ADM |
| getTid | /get-tid | POST | Retrieve a T-ID from ADM |

6.2.4.2	Operation: getRand

6.2.4.2.1	Description

The custom operation enables to generate and retrieve a random number. See 3GPP TS 33.369 [xx].

6.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 6.2.4.2.2-1 and the response data structure and response codes specified in table 6.2.4.2.2-2.

Table 6.2.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RandGetResponse | M | 1 | 200 OK | Upon success, a response body containing the generated random value RANDAIOT_n shall be returned |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

6.2.4.3	Operation: getAuthentication

6.2.4.3.1	Description

This custom operation enables to calculate and retrieve the authentication data. See 3GPP TS 33.369 [xx].

6.2.4.3.2	Operation Definition

This operation shall support the request data structures specified in table 6.2.4.3.2-1 and the response data structure and response codes specified in table 6.2.4.3.2-2.

Table 6.2.4.3.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GetAuthenticationRequest | M | 1 | Contains input parameters required to retrieve authentication data from ADM. |

Table 6.2.4.3.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GetAuthenticationResponse | M | 1 | 200 OK | Upon success, a response body containing the authentication data shall be returned |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

6.2.4.4	Operation: getSessionKey

6.2.4.4.1	Description

This custom operation enables to calculate and retrieve the Session Key. See 3GPP TS 33.369 [xx].

6.2.4.4.2	Operation Definition

This operation shall support the request data structures specified in table 6.2.4.4.2-1 and the response data structure and response codes specified in table 6.2.4.4.2-2.

Table 6.2.4.4.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| GetSessionKeyRequest | M | 1 | Contains input parameters required to retrieve the Session Key from ADM. |

Table 6.2.4.4.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GetSessionKeyResponse | M | 1 | 200 OK | Upon success, a response body containing the Session Key shall be returned |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

6.2.4.5	Operation: getTid

6.2.4.5.1	Description

This custom operation enables to derive a T-ID. See 3GPP TS 33.369 [xx].

6.2.4.5.2	Operation Definition

This operation shall support the request data structures specified in table 6.2.4.5.2-1 and the response data structure and response codes specified in table 6.2.4.5.2-2.

Table 6.2.4.5.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AiotDevPermId | M | 1 | Contains the AIoT device permanent identifier. |

Table 6.2.4.5.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GetTidResponse | M | 1 | 200 OK | Upon success, a response body containing the T-ID information shall be returned |
| NOTE: In addition common data structures as listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] are supported. |  |  |  |  |

6.2.5	Notifications

6.2.6	Data Model

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nadm_Sec service based interface protocol.

Table 6.2.6.1-1: Nadm_Sec specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| GetAuthenticationRequest | 6.2.6.2.2 | Contains input parameters for computing authentication data |  |
| AuthData | 6.2.6.2.3 |  |  |
| AuthDataSet | 6.2.6.2.4 |  |  |
| GetAuthenticationResponse | 6.2.6.2.5 | Contains authentication data |  |
| GetSessionKeyRequest | 6.2.6.2.6 | Contains input parameters for computing a session key |  |
| GetSessionKeyResponse | 6.2.6.2.7 | Contains a session key information |  |
| GetTidRequest | 6.2.6.2.8 | Contains input parameters for retrieving the T-ID |  |
| GetTidResponse | 6.2.6.2.9 | Contains T-ID information |  |
| Kaiotf | 6.2.6.3.2 | AIoT session key KAIOTF |  |

Table 6.2.6.1-2 specifies data types re-used by the Nadm_Sec service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nadm_Sec service based interface.

Table 6.2.6.1-2: Nadm_Sec re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [17] | see 3GPP TS 29.500 [4] clause 6.6 | SupportedFeatures |
| AiotDevPermId | 3GPP TS 29.571 [17] | AIoT device permanent identifier |  |
| AiotFilteringInformation | 3GPP TS 29.571 [17] | AIoT Filtering Information |  |
| ProblemDetails | 3GPP TS 29.571 [17] | Used in error responses to provide more detailed information about an error. |  |
| Tid | 6.1.6.3.2 | Temporary ID (T-ID) of the AIoT device |  |
| TidHandlingInformation | 6.1.6.2.x | T-ID Handling Information |  |
| Rand | 3GPP TS 29.503 [BB] | RAND |  |
| Xres | 3GPP TS 29.503 [BB] | XRES |  |

6.2.6.2	Structured data types

6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.6.2.2	Type: GetAuthenticationRequest

Table 6.2.6.2.2-1: Definition of type GetAuthenticationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | O | 0..1 | Indicates the AIoT device permanent identifier. (NOTE) |  |
| filteringInfo | AiotFilteringInformation | O | 0..1 | Indicates the AIoT Filtering Information. (NOTE) |  |
| randN | Rand | M | 1 | Indicates the RANDAIOT_n |  |
| randDlist | array(Rand) | M | 1..N | Indicates the RANDAIOT_d(s) |  |
| NOTE: At least, one of aiotDevPermId or filteringInfo shall be included. |  |  |  |  |  |

6.2.6.2.3	Type: AuthData

Table 6.2.6.2.3-1: Definition of type AuthData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| xres | Xres | M | 1 | Contains the calculated XRES. |  |
| aiotDevPermId | AiotDevPermId | O | 0..1 | Indicates the AIoT device permanent identifier used to calculate the XRES. |  |

6.2.6.2.4	Type: AuthDataSet

Table 6.2.6.2.4-1: Definition of type AuthDataSet

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| randD | Rand | M | 1 | Indicates the RANDAIOT_d used to derive the authentication data |  |
| authDataSet | array(AuthData) | M | 1..N | Contains the list of calculated XRES. |  |

6.2.6.2.5	Type: GetAuthenticationResponse

Table 6.2.6.2.5-1: Definition of type GetAuthenticationResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| authDataSets | array(AuthDataSet) | M | 1..N |  |  |

6.2.6.2.6	Type: GetSessionKeyRequest

Table 6.2.6.2.6-1: Definition of type GetSessionKeyRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | M | 1 | Indicates an AIoT device permanent identifier. (NOTE) |  |
| randN | Rand | M | 1 |  |  |
| randD | Rand | M | 1 |  |  |

6.2.6.2.7	Type: GetSessionKeyResponse

Table 6.2.6.2.7-1: Definition of type GetSessionKeyResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| kAiotf | Kaiotf | M | 1 | Contains the Session Key |  |

6.2.6.2.8	Type: GetTidRequest

Table 6.2.6.2.8-1: Definition of type GetTidRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aiotDevPermId | AiotDevPermId | M | 1 | Indicates an AIoT device permanent identifier. (NOTE) |  |
| resyncInd | boolean | O | 0..1 | Indicates if re-synchronization is required. true: re-synchronization is required; false (or absent): re-synchronization is not required; |  |

6.2.6.2.9	Type: GetTidResponse

Table 6.2.6.2.9-1: Definition of type GetTidResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tidHandInfo | TidHandlingInformation | M | 1 | Contains T-ID Handling Information |  |
| tid | Tid | M | 1 | Contains T-ID_n |  |
| tidPrevious | Tid | O | 0..1 | Contains T-ID_n-1 |  |

6.2.6.3	Simple data types and enumerations

6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| Kaiotf | string | pattern: "^[A-Fa-f0-9]{64}$" |  |

6.2.7	Error Handling

6.2.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

6.2.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

6.2.7.3	Application Errors

The common application errors defined in Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may be used for the Nadm_Sec. The application errors defined for the Nadm_Sec service are listed in Table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| DATA_NOT_FOUND | 404 Not Found | The AIoT device permanent identifier or Filtering Information is not recognized. |
| FILTER_INFORMATION_TOO_BROAD | 413 Payload Too Large | The group of devices derived from the Filtering Information is too large to compute the expected authentication data in a reasonable amount of time. |

6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the Nadm_Sec API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

6.2.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nadm_Sec API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nadm_Sec API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nadm_Sec service.

The Nadm_Sec API defines the following scopes for OAuth2 authorization.

Table 6.2.9-1: OAuth2 scopes defined in Nadm_Sec API

| Scope | Description |
| --- | --- |
| "nadm-sec" | Access to the Nadm_Sec API |
| "nadm-sec:get-rand" | Access to retrieve a RAND |
| "nadm-sec:get-authentication" | Access to retrieve Authentication Data |
| "nadm-sec:get-session-key" | Access to retrieve a Session Key |
| "nadm-sec:get-tid" | Access to retrieve a T-ID |

6.2.10	HTTP redirection

An HTTP request may be redirected to a different ADM service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different ADM producer instance will return the NF Instance ID of the new ADM producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an ADM redirects a service request to a different ADM using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new ADM towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

# *** Next Change ***

A.3	Nadm_Sec API

openapi: 3.0.0

info:

version: 1.0.0

title: Nadm_Sec API

description: |

Nadm_Sec Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.369 Ambient IoT Data Management Services, version 19.1.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.369/'

servers:

- url: '{apiRoot}/nadm-sec/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- oauth2ClientCredentials:

- nadm-sec

- {}

paths:

/get-rand:

post:

summary: Retrieve a random value (RANDAIOT_n)

operationId: getRand

responses:

'200':

description: Successful retrieval of RANDAIOT_n

content:

application/json:

schema:

$ref: '#/components/schemas/GetRandResponse'

security:

- oauth2ClientCredentials:

- nadm-sec:get-rand

/get-authentication:

post:

summary: Retrieve authentication data (XRESAIOT)

operationId: getAuthentication

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/GetAuthenticationRequest'

responses:

'200':

description: Successful retrieval of authentication data

content:

application/json:

schema:

$ref: '#/components/schemas/GetAuthenticationResponse'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

description: Payload Too Large

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

security:

- oauth2ClientCredentials:

- nadm-sec:get-authentication

/get-session-key:

post:

summary: Retrieve session key (KAIOTF)

operationId: getSessionKey

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/GetSessionKeyRequest'

responses:

'200':

description: Successful retrieval of session key

content:

application/json:

schema:

$ref: '#/components/schemas/GetSessionKeyResponse'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

security:

- oauth2ClientCredentials:

- nadm-sec:get-session-key

/get-tid:

post:

summary: Retrieve Temporary ID (T-ID)

operationId: getTid

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/GetTidRequest'

responses:

'200':

description: Successful retrieval of T-ID

content:

application/json:

schema:

$ref: '#/components/schemas/GetTidResponse'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

security:

- oauth2ClientCredentials:

- nadm-sec:get-tid

components:

securitySchemes:

oauth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: https://example.com/oauth2/token

scopes:

nadm-sec: Access to Nadm_Sec API

nadm-sec:get-rand: Access to retrieve a RAND

nadm-sec:get-authentication: Access to retrieve Authentication Data

nadm-sec:get-session-key: Access to retrieve a Session Key

nadm-sec:get-tid: Access to retrieve a T-ID

schemas:

#

# STRUCTURED TYPES

#

GetRandResponse:

type: object

properties:

randN:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

required:

- randN

GetAuthenticationRequest:

type: object

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

randN:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

randDlist:

type: array

items:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

minItems: 1

required:

- randN

- randDlist

AuthData:

type: object

description: Contains the calculated XRES and optionally the AIoT device permanent

identifier used to calculate it.

properties:

xres:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Xres'

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

required:

- xres

AuthDataSet:

type: object

description: Contains a RANDAIOT_d value and a list of associated AuthData entries.

properties:

randD:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

authDataSet:

type: array

items:

$ref: '#/components/schemas/AuthData'

minItems: 1

required:

- randD

- authDataSet

GetAuthenticationResponse:

type: object

description: Response type containing one or more AuthDataSet entries.

properties:

authDataSets:

type: array

items:

$ref: '#/components/schemas/AuthDataSet'

minItems: 1

required:

- authDataSets

GetSessionKeyRequest:

type: object

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

randN:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

randD:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/Rand'

required:

- aiotDevPermId

- randN

- randD

GetSessionKeyResponse:

type: object

properties:

kAiotf:

$ref: '#/components/schemas/Kaiotf'

required:

- kAiotf

GetTidRequest:

type: object

properties:

aiotDevPermId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

resyncInd:

type: boolean

required:

- aiotDevPermId

GetTidResponse:

type: object

properties:

tidHandInfo:

$ref: 'TS29369_Nadm_DM.yaml#/components/schemas/TidHandlingInformation'

tid:

$ref: 'TS29369_Nadm_DM.yaml#/components/schemas/Tid'

tidPrevious:

$ref: 'TS29369_Nadm_DM.yaml#/components/schemas/Tid'

required:

- tid

- tidHandInfo

#

# SIMPLE TYPES

#

Kaiotf:

type: string

pattern: '^[A-Fa-f0-9]{64}$'

#

# ENUMS

#

# *** End of Changes ***