3GPP TSG-CT Meeting #110	CP-253195

# Baltimore, United States, 8th Dec 2025 - 9th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.574 | CR | 0139 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF data indication EN removal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  | 2025-11-28 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | Since SA2 has not yet concluded whether the DCCF can be used to collect data using the Nlmf_DataExposure service, this CR proposes to remove the related Editor’s Note and the related provisions for the LMF data subscription via the DCCF, while maintaining the ability to apply processing instructions and/or provide additional information to LMF data at the NWDAF. |  |  |  |  |  |  |
| Summary of change: |  |  | Remove the Editor’s Note for the LMF data subscription indication, indicating that it is not applicable for the DCCF. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Open issue in the specification. |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.4, 5.1.6.2.3, 5.1.6.2.5, 5.1.6.2.13, 5.1.8 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact any OpenAPI file. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 4.2.2.2.4	Subscription for data notifications

Figure 4.2.2.2.4-1 shows a scenario where the NF service consumer sends a request to the DCCF to subscribe for data notifications.

Figure 4.2.2.2.4-1: NF service consumer subscribes to data notifications

The NF service consumer (i.e. NWDAF) shall invoke the Ndccf_DataManagement_Subscribe service operation to subscribe to data notification(s). The NF service consumer shall send an HTTP POST request with "{apiRoot}/ndccf-datamanagement/<apiVersion>/data-subscriptions" as Resource URI, as shown in figure 4.2.2.2.4-1, step 1, to create a subscription for an "Individual DCCF Data Subscription" resource according to the information in message body. The NdccfDataSubscription data structure provided in the request body shall include:

-	a notification target address within the "dataNotifUri" attribute;

-	a notification correlation identifier within the "dataNotifCorrId" attribute; and

-	a data subscription within the "dataSub" attribute, which contains one of the following:

-	AMF event exposure subscription within the "amfDataSub" attribute;

-	SMF event exposure subscription within the "smfDataSub" attribute;

-	UDM event exposure subscription within the "udmDataSub" attribute;

-	NEF event exposure subscription within the "nefDataSub" attribute;

-	AF event exposure subscription within the "afDataSub" attribute;

-	NRF event exposure subscription within the "nrfDataSub" attribute;

-	NSACF event exposure subscription within the "nsacfDataSub" attribute;

-	GMLC event exposure subscription within the "gmlcDataSub" attribute;

-	UPF event exposure subscription within the "upfDataSub" attribute, if the "UpEvents" feature is supported;

-	PCF event exposure subscription within the "pcfDataSub" attribute, if the "PcfEvents" feature is supported;

and may include:

-	the notification endpoints within the "notifEndpoints" attribute, if the "DataAnaCollect" feature is supported;

-	formatting instructions within the "formatInstruct" attribute;

-	processing instructions within the "procInstructs" attribute;

-	the indication for data storage within the "storeInd" attribute, if the "DataAnaCollect" feature is supported;

-	a target NF identifier within the "targetNfId" attribute" or a target NF set identifier within the "targetNfSetId" attribute";

-	an ADRF identifier within the "adrfId" attribute or an ADRF set identifier within the "adrfSetId" attribute; and/or

-	time window of the occurrence of the requested data collection within the "timePeriod" attribute.

NOTE 1:	The DCCF can use the provided time window e.g. to determine when to (un)subscribe to the data source NF and/or what subscription duration to indicate to it.

-	the purpose of data collection within the "dataCollectPurposes" attribute.

-	the indication that the NF service consumer has already checked the user consent within the "checkedConsentInd" attribute.

-	storage handling information within the "storeHandl" attribute, if the "EnhDataMgmt" feature is supported.

Upon the reception of an HTTP POST request with: "{apiRoot}/ndccf-datamanagement/<apiVersion>/subscriptions" as Resource URI and NdccfDataSubscription data structure as request body, the DCCF shall use the contents (e.g. "smfDataSub" attribute in NdccfDataSubscription data structure) of the request to determine whether the subscription can already be served or interactions with data sources (e.g. creation or modification of event exposure subscription for Nsmf_EventExposure service) are required. If the DCCF cannot use the contents of the request to determine this, the DCCF shall send an HTTP "400 Bad Request" error response including the "cause" attribute set to "SUBSCRIPTION_CANNOT_BE_SERVED".

NOTE 2:	The "SUBSCRIPTION_CANNOT_BE_SERVED" error can occur, for example, when the request is syntactically valid and there is no DCCF internal error, but the DCCF can neither find an existing subscription to a data source nor construct one based on the received subscription contents.

If the user consent has not been checked by the NF service consumer and is required for the requested data collection depending on local policy and regulations, then the DCCF shall check user consent for the targeted UE(s) based on the user consent subscription data that is retrieved via the Nudm_SDM service API of the UDM as described in clause 5.2.2.24 and clause 6.1.3.32 of 3GPP TS 29.503 [20]. If the DCCF receive the response from the UDM that it is not granted for the impacted user(s), then the DCCF shall send an HTTP "403 Forbidden" error response including the "cause" attribute set to "USER_CONSENT_NOT_GRANTED".

NOTE 3:	When the target of reporting is a SUPI or a GPSI then the subscription can be rejected, e.g. because user consent is not granted, and the error is sent to the consumer. When the target of reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, and the user consent is not granted for a subset of the impacted users, then no error is sent, but a subset of the SUPIs/GPSIs is skipped if user consent is not granted.

Otherwise, if the user consent subscription data retrieved from the UDM indicate that the user consent is granted for the impacted user(s), the DCCF shall subscribe to notification of changes of the user consent (unless it is already subscribed) by invoking the Nudm_SDM_Subscribe service operation by sending an HTTP POST request targeting the resource "SdmSubscriptions" to the UDM as described in clause 5.2.2.3 of 3GPP TS 29.503 [20].

If the DCCF determines that the subscription can already be served (without requiring further interactions with the data sources) or a successful response from the data source(s) is received for the creation or modification of subscription(s) to serve this subscription, the DCCF shall:

-	create a new subscription;

-	assign a subscriptionId;

-	store the subscription.

If the DCCF created an "Individual DCCF Data Subscription" resource, the DCCF shall respond with "201 Created" with the message body containing a representation of the created subscription, as shown in figure 4.2.2.2.4-1, step 2. The DCCF shall include a Location HTTP header field. The Location header field shall contain the URI of the created subscription i.e. "{apiRoot}/ndccf-datamanagement/<apiVersion>/data-subscriptions/{subscriptionId}". If an immediate reporting indication is provided in the subscription, the DCCF shall include the reports of the events subscribed, if available, in the HTTP POST response within the "dataSub" attribute, or, potentially within the "immReport" attribute, if the DataAnaCollect feature is supported.

When the notification flag of the "dataSub" attribute (e.g. the "notifFlag" attribute within the "eventsRepInfo" attribute in the case of AF events) is included and set to "DEACTIVATE" in the request, the DCCF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the notification flag to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer). When a muting exception occurs, if the EnhDataMgmt feature is supported, the DCCF may consider the contents of the muting instructions of the "dataSub" attribute (if provided; e.g. the "notifFlagInstruct" attribute within the "eventsRepInfo" attribute in the case of AF events) and/or local configuration to determine its actions.

If the EnhDataMgmt feature is supported and the DCCF accepts the provided notification flag and muting instructions, it may indicate the applied muting notification settings in the response (e.g. within the "mutingSetting" attribute in the case of AF events). If the DCCF does not accept the provided notification flag and muting instructions, it shall send an HTTP "403 Forbidden" error response including the "cause" attribute set to "MUTING_INSTR_NOT_ACCEPTED".

If the DCCF receives storage handling information in the request but determines (e.g. based on local policy) that a different storage approach shall be followed, it indicates the determined storage approach to the consumer by setting accordingly the "storeHandl" attribute (e.g. providing a different lifetime, or setting the indication about deletion alerts to "false") in the message body of the response. When more than one consumer has requested storage lifetime for the same data, the storage approach should be based on the longest requested storage lifetime.

NOTE 4:	The default operator policy for how long data is to be stored can be longer or shorter than the lifetime requested by the consumer. A default operator policy can for example accept only consumer requested lifetimes that are shorter or longer than the default policy.

If an error occurs when processing the HTTP POST request, the DCCF shall send an HTTP error response as specified in clause 5.1.7.

# *** Next Change ***

##### 5.1.6.2.3	Type NdccfDataSubscription

Table 5.1.6.2.3-1: Definition of type NdccfDataSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSub | DataSubscription | M | 1 | Represents the requested events subscription. (NOTE 1) |  |
| dataNotifUri | Uri | M | 1 | Notification target address. |  |
| dataNotifCorrId | string | M | 1 | Notification correlation identifier. |  |
| notifEndpoints | array(NotifyEndpoint) | O | 1..N | The notification target address(es) and correlation identifier(s) of additional endpoints that need to be notified, if any. | DataAnaCollect |
| formatInstruct | FormattingInstruction | O | 0..1 | Formatting instructions to be used for sending event notifications. If provided, they take the precedence over any potentially conflicting event reporting requirements provided within the "dataSub" attribute. |  |
| procInstructs | array(ProcessingInstruction) | O | 1..N | Processing instructions to be used for sending event notifications. |  |
| targetNfId | NfInstanceId | O | 0..1 | Data Producer NF instance identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| targetNfSetId | NfSetId | O | 0..1 | Data Producer NF set identifier to which the DCCF shall create the requested subscription. (NOTE 3) |  |
| adrfId | NfInstanceId | O | 0..1 | Identifier of the ADRF to be used by the DCCF. If the subscription is for runtime data (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF. If the subscription is for historical data (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF. (NOTE 3) |  |
| adrfSetId | NfSetId | O | 0..1 | Identifier of the ADRF Set to be used by the DCCF. If the subscription is for runtime data (i.e. the "timePeriod" attribute is either absent or contains a time window in the future) then the DCCF shall store the notifications in this ADRF Set. If the subscription is for historical data (i.e. the "timePeriod" attribute contains a time window in the past) then the DCCF shall retrieve the data from this ADRF Set. (NOTE 3) |  |
| storeInd | boolean | C | 0..1 | The indication for data storage. This attribute shall be provided and set to "true" if the consumer requests to store the data in an ADRF but both the "adrfId" and "adrfSetId" attributes are not provided. The default value is "false". | DataAnaCollect |
| storeHandl | StorageHandlingInformation | O | 0..1 | Contains storage handling information for the data that will be collected and stored in an ADRF based on the requested subscription. | EnhDataMgmt |
| immReport | NdccfDataSubscriptionNotification | O | 0..1 | Immediate report including available DCCF notification. May only be present in the DCCF response to a subscription request and only if immediate reporting together with formatting/processing instructions was requested in the subscription request. | DataAnaCollect |
| timePeriod | TimeWindow | O | 0..1 | Represents a start time and a stop time during which data was collected or is requested to be collected. If this attribute is included, then the internal attributes of the data subscription that indicate a subscription duration (e.g. the "targetPeriod" attribute of an "eventSubs" attribute of an "smfDataSub" attribute, or the "monDur" attribute of the ReportingInformation data type) shall not be provided. (NOTE 2) |  |
| dataCollectPurposes | array(DataCollectionPurpose) | O | 1..N | The purposes of data collection. This attribute may only be provided if user consent is required depending on local policy and regulations and the consumer has not checked user consent. |  |
| checkedConsentInd | boolean | O | 0..1 | If set to "true", it indicates that the NF service consumer has already checked the user consent, otherwise, if set to "false", it indicates that the consumer has not yet checked the user consent. The default value is "false". | UserConsent |
| suppFeat | SupportedFeatures | C | 0..1 | This attribute represents a list of Supported features as described in clause 5.1.8. It shall be present if feature negotiation needs to take place. In the POST request for UE data subscription context transfer, it represents the features supported by the data consumer. |  |
| NOTE 1: The notification target address contained in the subscription attribute that is provided within the "dataSub" attribute (i.e. "eventNotifyUri" of "amfDataSub", "notifUri" of "smfDataSub", "callbackReference" of "udmDataSub", "notifUri" of "nefDataSub", "notifUri" of "afDataSub", "nfStatusNotificationUri" of "nrfDataSub", "eventNotifyUri" of "nsacfDataSub", "eventNotifyUri" of "upfDataSub" if the UpEvents feature is supported, "eventNotificationUri" of "gmlcDataSub" if the LocEvents feature is supported, or "notifUri" of "pcfDataSub" if the PcfEvents feature is supported) and the notification correlation ID contained in the subscription attribute that is provided within the "dataSub" attribute (i.e. the "notifyCorrelationId" of "amfDataSub", "notifId" of "smfDataSub", "notifyCorrelationId" of "udmDataSub", "notifId" of "nefDataSub", "notifId" of "afDataSub", "notifyCorrelationId" of "nsacfDataSub", "notifyCorrelationId" of "upfDataSub" if the UpEvents feature is supported, "ldrReference" of "gmlcDataSub" if the LocEvents feature is supported, or "notifId" of "pcfDataSub" if the PcfEvents feature is supported) shall be ignored by the DCCF. The DCCF will provide the notification target address and the notification correlation ID of the DCCF itself in the data type during the event subscription request to each NF. The event reporting information (e.g. "eventsRepInfo" attribute in the case of AF events) may include muting instructions (e.g. within the "notifFlagInstruct" attribute in the case of AF events) and/or muting notifications settings (e.g. within the "mutingSetting" attribute in the case of AF events) only if the EnhDataMgmt feature is supported. NOTE 2: It includes the time period either in the past or in the future (i.e., start time as past time and stop time as future time is not allowed). NOTE 3: "targetNfId" and "targetNfSetId" are mutually exclusive. "adrfId" and "adrfSetId" are also mutually exclusive. |  |  |  |  |  |

# *** Next Change ***

##### 5.1.6.2.5	Type NdccfDataSubscriptionNotification

Table 5.1.6.2.5-1: Definition of type NdccfDataSubscriptionNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataNotif | DataNotification | C | 0..1 | Data subscription notification. (NOTE 1, NOTE 3, NOTE 4) |  |
| dataReports | array(NotifSummaryReport) | C | 1..N | List of reports with summarized data from multiple notifications received from data producer. (NOTE 1) |  |
| fetchInstruct | FetchInstruction | C | 0..1 | Instructions for the NF service consumer to fetch the notifications itself. (NOTE 1, NOTE 2) |  |
| delAlert | DeletionAlert | C | 0..1 | Information about data that is about to be deleted. (NOTE 1, NOTE 2) | EnhDataMgmt |
| newSubscriptionUri | Uri | C | 0..1 | The URI of the subscription resource located in the target DCCF. This parameter shall be present if the notification is for informing the successful transfer of a subscription from the source DCCF to the target DCCF. (NOTE 1, NOTE 2) | SubscriptionTransfer |
| dataNotifCorrId | string | M | 1 | Notification correlation identifier. |  |
| suppFeat | SupportedFeatures | C | 0..1 | This attribute represents the features supported by both the data consumer and the target DCCF as described in clause 5.1.8. It shall be present if the "newSubscriptionUri" attribute is present. | SubscriptionTransfer |
| pendDataNotifCause | PendingNotificationCause | O | 0..1 | Represents the Pending Data Notification Cause for the stored unsent data. | EnhDataMgmt |
| reUserConsentPurs | array(DataCollectionPurpose) | O | 1..N | The purposes of data collection for which the user consent is revoked. This attribute may only be provided if subscription is requested to be terminated due to user consent revocation, i.e. the value of "termCause" is "USER_CONSENT_REVOKED". | TerminationCause |
| terminationReq | boolean | O | 0..1 | If provided and set to "true", it indicates that the subscription is requested to be terminated, i.e. no further notifications related to this subscription will be provided. The default value is "false". |  |
| termCause | TermCause | O | 0..1 | A cause for which the DCCF will collect no further data for this subscription. Its presence indicates that the DCCF requests the termination of the subscription. This attribute may only be provided if the "terminationReq" attribute is set to "true". | TerminationCause |
| timeStamp | DateTime | M | 1 | It represents the time when DCCF completed preparation of the requested analytics. (NOTE 3) |  |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: This attribute shall not be included in the response body of a Fetch operation. NOTE 3: If the DCCF has received the notifications from another source without a timestamp, then the DCCF adds itself a timestamp based on the time it received the notification in the "timeStamp" attribute within the DataNotification data type. NOTE 4: The "upfEventNotifs" attribute within the "dataNotification" attribute is applicable only if the "UpEvents" feature is supported. The "gmlcEventNotifs" attribute within the "dataNotification" attribute is applicable only if the "LocEvents" feature is supported. The "pcfEventNotifs" attribute within the "dataNotification" attribute is applicable only if the "PcfEvents" feature is supported. |  |  |  |  |  |

# *** Next Change ***

##### 5.1.6.2.13	Type DccfEvent

Table 5.1.6.2.13-1: Definition of type DccfEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nwdafEvent | NwdafEvent | C | 0..1 | Identifies an NWDAF event type. (NOTE 1) |  |
| smfEvent | SmfEvent | C | 0..1 | Identifies an SMF event type. (NOTE 1) |  |
| amfEvent | AmfEventType | C | 0..1 | Identifies an AMF event type. (NOTE 1) |  |
| nefEvent | NefEvent | C | 0..1 | Identifies an NEF event type. (NOTE 1) |  |
| udmEvent | EventType | C | 0..1 | Identifies a UDM event ID that the processing instructions apply to. (NOTE 1) |  |
| afEvent | AfEvent | C | 0..1 | Identifies an AF event type. (NOTE 1) |  |
| sacEvent | SACEvent | C | 0..1 | Identifies an NSACF event type. (NOTE 1) |  |
| nrfEvent | NotificationEventType | C | 0..1 | Identifies an NRF event type. (NOTE 1) |  |
| gmlcEvent | EventNotifyDataType | C | 0..1 | Identifies a GMLC event type. (NOTE 1) | LocEvents |
| upfEvent | EventType | C | 0..1 | Identifies a UPF event. (NOTE 1) | UpEvents |
| lmfEventInd | boolean | C | 0..1 | If provided it identifies an LMF Data Exposure event. It may only have the value "true". (NOTE 1) (NOTE 2) | LmfEvents |
| pcfEvent | PcEvent | C | 0..1 | Identifies a PCF event. (NOTE 1) | PcfEvents |
| NOTE 1: Exactly one of these attributes shall be provided. NOTE 2: The "lmfEventInd" attribute is not applicable for the Ndccf_DataManagement and Nnwdaf_DataManagement APIs in this release of the specification, and it is included here only for usage by other APIs. |  |  |  |  |  |

# *** Next Change ***

### 5.1.8	Feature negotiation

The optional features in table 5.1.8-1 are defined for the Ndccf_DataManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | UserConsent | This feature indicates the support of detailed handling of user consent, e.g. indications that user consent has been checked and error responses related to the user consent is not granted. |
| 2 | DataAnaCollect | This feature indicates support for the enhancement of data and analytics collection. |
| 3 | EnhDataMgmt | This feature indicates the support of enhanced data management mechanisms, including support of pending notification, muting and storage handling. |
| 4 | TerminationCause | This feature indicates support for data collection subscription termination requests with termination cause and the purposes of data collection if the user consent is revoked sent by the DCCF to the NF service consumer. |
| 5 | LocEvents | This feature indicates the support of location events from the GMLC. |
| 6 | UpEvents | This feature indicates the support of UPF events. |
| 7 | SubscriptionTransfer | This feature indicates support for the data subscription transfer. |
| 8 | Void |  |
| 9 | PcfEvents | This feature indicates the support of PCF event exposure events. |

# *** End of Changes ***