3GPP TSG-CT WG1 Meeting #156	C1-254569

Goteberg, Sweden 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0803 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completing the handling for 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-07-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA3 has specified in TS 33.503 the following: ##### 6.3.8.2.2 Security procedure over Control Plane with multi-hop UE-to-Network Relay discovery with Model B (…) Each of the Intermediate UE-to-Network Relay needs to establish secured PC5 link with the upstream node (Intermediate UE-to-Network Relay or the UE-to-Network Relay) before it can serve the Remote UE, or the Intermediate UE-to-Network Relay acting the role of the Remote UE. The Intermediate UE-to-Network Relay’s UDM checks whether the Intermediate UE-to-Network Relay is authorised to offer multi-hop UE-to-Network relay service based on the RSC. And also: ##### 6.3.8.2.4 Security procedure over User Plane after multi-hop UE-to-Network Relay discovery with Model B (…) 3. If the Intermediate UE-to-Network Relay does not have an existing PC5 link with the selected 5G ProSe UE-to-Network Relay or an Intermediate UE-to-Network relay on the path to the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay shall establish a PC5 link with the 5G ProSe UE-to-Network Relay or the intermediate UE-to-Network relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2. 4. The Intermediate UE-to-Network Relay, then, performs the Key Request/Response procedure with the 5G PKMF/DDNMF of Intermediate UE-to-Network Relay using the parameters received in step 2. The above requirements clarify that an intermediate relay UE initiates the security procedure with a Remote UE/Child relay UE only after establishing a secure link with its parent relay UE. There corresponding handling in stage-3 for the above is missing, where there are some Ens added in this area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completing the missing handling 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery and resolved the related ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery remains incomplete and related ENs are not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

##### 8b.2.2.2.3	5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery

The 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery shall be initiated by the 5G ProSe multi-hop remote UE towards the first 5G ProSe intermediate UE-to-network relay UE, after the 5G ProSe multi-hop remote UE completes the multi-hop UE-to-network relay discovery over PC5 interface with model B.

The 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay based on model B discovery reuses the procedure for 5G ProSe direct link establishment procedure for UE-to-network relay communication described in clause 7.2.2 but with the following modifications:

a)	for the first hop, i.e. the hop between the 5G ProSe multi-hop remote UE and the first 5G ProSe intermediate UE-to-network relay UE, the PROSE DIRECT LINK ESTABLISHMENT REQUEST message created by the 5G ProSe multi-hop remote UE:

1)	shall additionally include the Multi-hop path info IE set to the list of the User Info IDs of the 5G ProSe intermediate UE-to-network relay UE(s), if any, and the 5G ProSe multi-hop UE-to-network relay UE selected by the 5G ProSe multi-hop remote UE based on the path information provided to the 5G ProSe multi-hop remote UE during the multi-hop UE-to-network relay discovery over PC5 interface with model B. The User Info IDs in the list shall be ordered starting from the first 5G ProSe intermediate UE-to-network relay UE till the 5G ProSe multi-hop UE-to-network relay UE; and

2)	may additionally include the Multi-hop end-to-end QoS Info IE to indicate the end to end QoS Information;

Editor's note (CR0616, 5G_ProSe_Ph3):	The details of QoS information are FFS.

b)	Further, any 5G ProSe intermediate UE-to-network relay UE, after receiving and accepting the PROSE DIRECT LINK ESTABLISHMENT REQUEST message from its 5G ProSe child UE-to-network relay UE or the 5G ProSe multi-hop remote UE, shall use the list of the User Info IDs in the Multi-hop path info IE to determine its 5G ProSe parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE. The 5G ProSe intermediate UE-to-network relay UE shall then initiate the 5G ProSe direct link establishment procedure towards its 5G ProSe parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE before proceeding with establishing the secured PC5 link with its 5G ProSe child UE-to-network relay UE or the 5G ProSe multi-hop remote UE. The PROSE DIRECT LINK ESTABLISHMENT REQUEST message created by the 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE or the 5G ProSe multi-hop UE-to-network relay UE:

1)	shall additionally include the Multi-hop path info IE set to the same list of the User Info IDs received from the 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop remote UE,

2)	may additionally include the Multi-hop end-to-end QoS Info IE to indicate the end to end QoS Information; and

3)	shall additionally include the Multi-hop remaining QoS Info IE to indicate the remaining QoS Information of the hops; and

Editor's note (CR0616, 5G_ProSe_Ph3):	The details of all QoS information are FFS.

c)	any 5G ProSe intermediate UE-to-network relay UE, after receiving and accepting the PROSE DIRECT LINK ESTABLISHMENT ACCEPT message from its 5G ProSe multi-hop parent UE-to-network relay UE or its 5G ProSe multi-hop UE-to-network relay UE, shall proceed with establishing the secured PC5 link with its 5G ProSe multi-hop child UE-to-network relay UE or the 5G ProSe multi-hop remote UE.

***** End of changes *****