3GPP TSG-CT WG1 Meeting #156	C1-254700

# Goteburg, SE, 25– 29 August 2025

Source:	Lenovo

Title:	HFL training service

Spec:	3GPP TS 24.560V1.0.0

Agenda item:	19.41

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Stage 2 text exists for this feature in TS 23.482 and therefore stage 3 is needed.

The HFL training is a very iterative process and is performed over many training rounds and many AIMLE clients. Therefore, the AIMLE server is required to have not only the capability to subscribe to the HFL training event for the training schedules of the selected AIMLE clients, but also the capabilities to modify, update, and terminate the configuration of the training schedules of the selected AIMLE clients.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.560V1.0.0.

* * * First Change * * * *

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API descriptions

| Service Name | Clause | Description | OpenAPI specification File | API name | Annex |
| --- | --- | --- | --- | --- | --- |
| Member participation configurations provisioning and management | 5.2 | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |
| Aimlec_HFLTraining | 5.3 | HFL training service | TS24560_Aimlec_HFLTraining.yaml | aimless-hfl-trng |  |
| Aimles_AIMLEClientRegistration | 5.4 | AIMLE client registration service | TS24560_Aimles_AIMLEClientRegistration.yaml | aimles-client-reg |  |
| Lifecycle management service | 5.5 |  |  |  |  |
| Operational splitting and provisioning management | 5.6 |  |  |  |  |
| Aimlec_FLGroupIndication | 5.7 | FL group indication service | TS24560_Aimlec_FLGroupIndication.yaml | aimlec-flgi |  |
| Data management | 5.8 |  |  |  |  |
| Edge | 5.9 |  |  |  |  |
| Model distribution | 5.10 |  |  |  |  |
| Aimlec_AIMLEClientServiceOperations | 5.11 | AIMLE client service operations service | TS24560_Aimlec_AIMLEClientServiceOperations.yaml | aimlec-serv-ops |  |
| Aimlec_AimlTaskTransfer | 5.12 | AIMLE client AIML task transfer service | TS24560_Aimlec_AimlTaskTransfer.yaml | aimlec-task-transfer |  |
| Aimles_AimlTaskTransfer | 5.13 | AIMLE server AIML task transfer service | TS24560_Aimles_AimlTaskTransfer.yaml | aimles-task-transfer |  |
| Aimlec_MLModelTrainingCapabilityEva | 5.15 | ML model training capability evaluation service | TS24560_Aimlec_MLModTngCapEva.yaml | aimlec-ml-mtce |  |

NOTE:	When 3GPP TS 29.122 [5] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the AIMLE service producer (AIMLE server or AIMLE client) takes the role of the SCEF and the AIMLE service consumer (AIMLE client or AIMLE server) takes the role of the SCS/AS.

* * * Next Change * * * *

## 5.3	HFL training service

### 5.3.1	Service description

The Horizontal Federated Learning (HFL) training service enables the AIMLE server to configure training schedule for one or more AIMLE clients and subscribe to that as defined in 3GPP TS 23.482 [4].

### 5.3.2	Service operations

#### 5.3.2.1	Introduction

The service operation defined for Aimlec_HFLTraining API is shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the Aimlec_HFLTraining API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimlec_HFLTraining_Subscribe | This service operation allows a service consumer to create an HFL training subscription. | e.g., AIMLE server |
| Aimlec_HFLTraining_Retrieve | This service operation allows a service consumer to retrieve an existing individual HFL training subscription. | e.g., AIMLE server |
| Aimlec_HFLTraining_Update | This service operation allows a service consumer to update an existing individual HFL training subscription. | e.g., AIMLE server |
| Aimlec_HFLTraining_Unsubscribe | This service operation allows a service consumer to delete an existing individual HFL training subscription. | e.g., AIMLE server |
| Aimlec_HFLTraining_Notify | This service operation allows a service consumer to receive HFL training notifications. | AIMLE client |

#### 5.3.2.2	Aimlec_HFLTraining_Subscribe

##### 5.3.2.2.1	General

This service operation is used by a service consumer e.g., the AIMLE server to subscribe with the AIMLE client for HFL training event.

##### 5.3.2.2.2	AIML operation for HFL training subscription

To subscribe to the HFL training event with the AIMLE client, the AIMLE server shall send to the AIMLE the HTTP POST request containing the HFLTrngSub structure data type as specified in clause 6.2.6.2.2.

Upon receipt of the HTTP POST request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to subscribe to the HFL training event, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

b)	is authorized to subscribe to the HFL training event:

1)	the AIMLE client may create the HFL training event subscription and store the subscription information. If the AIMLE client:

i)	has created the subscription, in the response, the AIMLE client shall include an indication of success, an HTTP "Location" header containing the URI of the newly created resource, and the HFLTrngSub structure data type as specified in clause 6.2.6.2.2; or

ii)	has not created the subscription, in the response, the AIMLE client shall include an indication of failure that the subscription is not created and shall include an appropriate error response as specified in clause 6.2.7.

#### 5.3.2.3	Aimlec_HFLTraining_Retrieve

##### 5.3.2.3.1	General

This service operation is used by a service consumer e.g., the AIMLE server to retrieve an existing individual HFL training event subscription with the AIMLE client.

##### 5.3.2.3.2	AIML operation for HFL training subscription retrieval

To retrieve an individual HFL training event subscription with the AIMLE client, the AIMLE server shall send the HTTP GET request to the AIMLE client.

Upon receipt of the HTTP GET request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to retrieve the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

b)	is authorized to retrieve the existing individual HFL training event subscription and if the receival request is:

1)	handled locally, the AIMLE client may retrieve the existing individual HFL training subscription. If the AIML client:

i)	has retrieved the subscription, in the response, the AIMLE client shall include an indication of success and may include the HFLTrngSub structure data type as specified in clause 6.2.6.2.2; or

ii)	has not retrieved the subscription, in the response, the AIMLE client shall include an indication of failure and may include an appropriate error response as specified in clause 6.2.7; or

2)	redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.3.2.4	Aimlec_HFLTraining_Update

##### 5.3.2.4.1	General

This service operation is used by a service consumer e.g., the AIMLE server to update an existing individual HFL training event subscription with the AIMLE client.

##### 5.3.2.4.2	AIML operation for HFL training subscription update

To update an existing individual HFL training event subscription with the AIMLE client, the AIMLE server shall send to the AIMLE client:

-	the HTTP PATCH request (for partial update) containing the HFLTrngSubPatch structure data type as specified in clause 6.2.6.2.3; or

-	the HTTP PUT request (for full replacement) containing the HFLTrngSub structure data type as specified in clause 6.2.6.2.2.

Upon receipt of the HTTP PATCH request or HTTP PUT request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to update the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

b)	is authorized to update the existing individual HFL training event subscription and if the received request is:

1)	handled locally, the AIMLE client may update the existing individual HFL training event subscription with the updated subscription information. If the AIML client:

i)	has updated the subscription, in the response, the AIMLE client shall include an indication of success and may include the HFLTrngSub structure data type as specified in clause 6.2.6.2.2; or

ii)	has not updated the subscription, in the response, the AIMLE client shall include an indication of failure and may include an appropriate error response as specified in clause 6.2.7; or

2)	redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.3.2.5	Aimlec_HFLTraining_Unsubscribe

##### 5.3.2.5.1	General

This service operation is used by a service consumer e.g., the AIMLE server to unsubscribe an existing individual HFL training event subscription with the AIMLE client.

##### 5.3.2.5.2	AIML operation for HFL training unsubscription

To unsubscribe an existing individual HFL training event subscription with the AIMLE client, the AIMLE server shall send an HTTP DELETE request to the AIMLE client.

Upon receipt of the HTTP DELETE request from AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to unsubscribe for the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

b)	is authorized to update the existing individual HFL training event subscription and if the received request is:

1)	handled locally, the AIMLE client may unsubscribe the existing individual HFL training event subscription. If the AIMLE client:

i)	has unsubscribed the subscription, in the response, the AIMLE server shall include an indication of success; or

ii)	has not unsubscribed the subscription, in the response, the AIMLE client shall include an indication of failure and may include an appropriate error response as specified in clause 6.2.7; or

2)	redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

#### 5.3.2.6	Aimlec_HFLTraining_Notify

##### 5.3.2.6.1	General

This service operation is used by the AIMLE client to notify a service consumer e.g., the AIMLE server, information about the HFL training event.

##### 5.3.2.6.2	AIML operation for HFL training notification

To notify about the HFL training event to the AIMLE server, the AIMLE client shall to the AIMLE server an HTTP POST request containing the HFLTrngNotify data structure as specified in clause 6.2.6.2.4.

Upon receipt of the HTTP POST request from the AIMLE client, the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request. If the AIMLE client:

a)	is not authorized, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

b)	is authorized to notify for the HFL training subscription and if the received request is:

1)	handled locally, the AIMLE server shall respond to the AIMLE client with:

i)	if the HTTP POST request is handled successfully, an HTTP "204 No Content" status code; or

ii)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.2.7; or

2)	redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

* * * End of Changes * * * *